3GPP TSG RAN WG4 Meeting #115	R4-2506465

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.21.2

Source:	vivo

Title:	Discussion on general aspects for AI/ML for NR air interface

Document for:	Discussion

- Introduction

On the general aspects for AI/ML, in the last meeting, Due to time constraints, RAN4 did not discuss the details for post-deployment handling. According to the WF, this issue will be addressed in RAN4#115. In this contribution, we provide our views for the issue on post deployment handling. The latest way forward was captured in [1].

- Discussion

  - Test post-deployment

Way forward: The following table presents the description of options 1 and 2 and sub-options as already agreed. A new name for “option 1” and “option 2” is proposed to reduce ambiguity when discussing in the future. The descriptions of the options are the same as the already agreed description. The “further potential clarifications” box captures issues that have been raised that could further clarify the options. It does not represent any agreement on the issues, or that all are relevant, but is intended to stimulate input and discussion to future meetings. The “significant issues” line is intended to capture issues that have been raised that should be answered in order to make a decision. Option 1; Option 2. New name; Pre-activation functionality/model update testing; Post deployment management based on LCM. Description; Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs • Validation takes into account the UE hardware in which the model is to be deployed/activated.; Using performance monitoring and LCM procedures • Performance monitoring will be designed in other groups • RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.. Sub-options; Possible collection of input data to a model during conformance testing for use later on.. Further potential clarifications (not resolved); The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. o Whether models that have been updated post conformance testing should be kept in a device. o Whether parallel operating of one model and (in-device) testing of another can be assumed. o Whether signalling from a UE of changes/updates are needed. - Whether there is any relation to GCF procedures. - Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. - Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy.; The following proposals have been made for option 1 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only.. Significant issues; Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates.; Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs.. Agreement in RAN4#114bis: - Rename “option 1” as “Post-deployment pre-activation functionality/configuration update testing” - Rename “option 2” as “Post-deployment post-activation functionality testing based on performance monitoring”

For Post deployment testing, the main purpose is to guarantee the performance for updated AI/ML-enabled functionalities deployed in the field. Without consideration of conformance testing, there are two directions in the ongoing discussion as listed as above. One is to conduct the specific validation procedure before its deployment. And another is to use performance monitor mechanism to monitor its performance. Generally, both of the two options aim to verify that model performance is still good, mainly by enabling a comparison between expected results and prediction results.

After the discussion in RAN4#114 meeting, the issues that need to be solved before either/both of solutions can be adopted are summarized in the table above. In the following, we will present our views on thi issues.

Analysis on Option 1

- Issue 1: Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes.

  - Definition of Fine-tuning and Model update

    - Model update can have a performance impact.

    - Fine-tuning might have little performance impact.

The first and most critical step in the validation process for model updates is to define which types of updates actually require performance validation. Under the RAN1/2 framework, the model itself is invisible to NW, and current discussions are conducted at the functionality level. Against this backdrop, we must clarify how major and minor updates are distinguished in terms of functionality—and whether this distinction should be: Standardized (i.e., explicitly defined in specifications), or left to the discretion of the UE, if functionality is updated by the UE.

- If standardization is chosen, defining specific criteria for major/minor updates could require extensive deliberation, given the vast diversity in AI model/functionality updates and their varying degrees of impact.

- Conversely, if UE autonomously decides whether validation is needed before conducting a functionality update, this approach may significantly weaken the enforceability of the validation procedure.

Another critical aspect to clarify is whether the completion of such updates needs to be conveyed to the network or TE. Since the model/functionality update occurs on the UE side, the network remains inherently unaware of these changes. If explicit visibility is required, the UE would need to inform the network this update. This raises further questions: Should such reporting be mandatory or optional? What forma should be used for this information(e.g., signaling message, capability indication)? And How would this impact UE complexity and network overhead?

- Issue 2: Whether the testing of a new model is on a device or in a lab.

Closely tied to the first issue is the critical question of where this model validation process should occur. Each approach presents significant implementation challenges:

- Laboratory-Based Validation:

On the one hand, requiring lab testing for every update (depending on validation triggers as discussed previously) creates substantial operational burdens. Besides, the end-to-end cycle - from model update completion → lab validation → deployment approval - may introduce prohibitive latency.

On the other hand, compared with LCM, laboratory testing would demand exponentially greater effort.

- Certification Process Implications (e.g., GCF):

Under the method, each update after initial certification may trigger new validation requirements, creating unsustainable costs. Whether the current certification frameworks can fit the continuous AI model iteration is also a question.

- On device testing:

Testing with alternative UEs cannot fully capture performance characteristics across all devices due to hardware implementation differences

- Issue 3: Whether models that have been updated post conformance testing should be kept in a device

Regarding the testing scheme conducted on a device, based on the issues outlined below, we believe further clarification is needed—particularly on how to interpret "whether models should be kept in a device."

More importantly, if this scheme assumes field testing on a single UE, where the UE autonomously validates the updated model (e.g., using pre-deployment verified data to check whether the updated model functions properly), this essentially means the UE independently determines validity (i.e., validation is entirely performed offline by the UE itself) without network interaction (or merely notifying the network that the updated model has passed performance validation), this appears more like a UE-specific implementation. We think there needs to be more clarification about such a validation, how to verify that UE has conducted such a process, and what the potential specification impact implications are.

- Issue 4: Whether parallel operating of one model and (in-device) testing of another can be assumed.

- Issue 5: Whether signalling from a UE of changes/updates are needed.

Regarding the two issues mentioned above, these are essentially details or optimizations that should only be considered after the validation approach has been finalized. We propose that these discussions be deferred until after RAN4 concludes on a definitive testing scheme.

Based on the above discussion, we summarize the key issues and propose as follows:

Proposal 1: For Pre-activation functionality/model update testing, at least the following issues need to be taken into account

Table 1 Summary of Issues in Model Update Validation

| Issue | Key aspects | Standardization impact | Challenges or problems |
| --- | --- | --- | --- |
| 1. Major vs. Minor Model Changes | No clear definition of "major" (high-impact) vs. "minor" updates. | For Alt 1: Define universal criteria for diverse AI updates. For Alt 2: If the major/minor update is judged by UE itself, whether to mandate UE reporting this update. | Over-/under-validation risk |
|  | Solutions: - Alt 1: Defining standardized criteria - Alt 2: UE's sole discretion |  |  |
| 2. Lab vs. On-Device Validation | - Lab testing: - High cost/latency; - Whether GCF certification can fit the iterative AI is a question - On-Device testing: - Hardware variability biases results; - Offline validation lacks oversight. | - Lab testing: Lab testing scalability. - On-Device testing: How to Standardize on-device test procedures (this appears more like a UE-specific implementation) | - Lab testing: the end-to-end cycle - from model update completion → lab validation → deployment approval - may introduce prohibitive latency laboratory testing would demand exponentially greater effort - On-Device testing: Testing with alternative UEs cannot fully capture performance characteristics across all devices due to hardware implementation differences |

Analysis on Option 2

Next, we will focus on the analysis for Option 2 on conducting performance monitoring. For Option 2, the concern raised mainly include:

- Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs.

  - Whether the metric used for performance testing and the metric used for monitoring can be aligned.

- Whether monitoring used for post-deployment testing should be NW sided only.

- If there is performance degradation occurs in the updated model, how to find out the causes for degradation, e.g., due to model update/data drift or other factors that possibly affect functionality performance.

Response to Question 1

Taking the beam management use case as an example, regarding performance monitoring, the design of the performance monitoring process and corresponding metrics are still under discussion in other WGs. Apart from the already defined Top-1 or Top-K beam prediction accuracy, other metrics—such as L1-RSRP difference—remain under consideration.

From RAN4 requirement perspective, the definition of performance monitoring requirements will be further discussed, including whether to directly reuse the performance metrics designed in RAN1. Currently, RAN4’s definition of KPIs is still based on simulation analysis results. The question of whether the metric used for performance testing can align with the one used for monitoring can be revisited once RAN1 and RAN4 finalize their respective discussions on monitoring metrics and performance testing metrics.

From a technical perspective, the metrics currently under discussion in RAN4 for performance testing include beam prediction accuracy and L1-RSRP difference. However, for monitoring metrics, there is a possibility that L1-RSRP may not be included (depending on RAN1’s decision). Regarding whether this misalignment could affect the accuracy of updated model validation, our thinking is :

On the one hand, based on existing simulation results, beam prediction accuracy and L1-RSRP difference accuracy are highly correlated, meaning that beam prediction accuracy alone is already sufficient to reflect the model’s predictive performance.

On the other hand, considering there is still the case that the prediction results support reporting only the beam ID (without the corresponding RSRP), performance testing in such cases would only involve validation of beam prediction accuracy. Under this case, the metric used for performance testing and the metric used for monitoring is aligned

Based on these two points, we think the impact on the possible disalignment between the metric used for performance testing and the metric used for monitoring would be minimal.

Response to Question 2

According to RAN2’s design for UE-side model, in addition to "network decision, network-initiated" and "network decision, UE-initiated", it also includes "UE autonomous decision, reported to the network". Therefore, for post-deployment monitoring and testing, the scope is not limited to NW-side monitoring—UE-side monitoring is also included.

Observation 1: For one sided model, taking BM as the example, regarding the impact on the possible disalignment between the metric used for performance testing and the metric used for monitoring, it would be minimal by considering the following reasons:

- Based on existing simulation results, beam prediction accuracy and L1-RSRP difference accuracy are highly correlated, meaning that beam prediction accuracy alone is already sufficient to reflect the model’s predictive performance.

- Considering there is still the case that the prediction results support reporting only the beam ID (without the corresponding RSRP), Under this case, the metric used for performance testing and the metric used for monitoring can be aligned

Response to Question 3

For question 3, considering the issue may be more common for two-sided model, here we will take CSI compression as an example for clarifying whether the cause can be identified by conducting performance monitoring.

For two-sided model in CSI compression use case, there have been some studies in terms of this issue in RAN1 discussion. Based on the RAN1 study, performance monitoring is capable of identifying the cause of performance degradation. Here we will provide the main processes of cause identification respectively for both NW-side monitoring and UE-side monitoring.

- Cause identification with NW-side monitoring

- Illustration of cause identification with NW-side monitoring

The procedure of NW monitoring is the same for option 1/3/4/5 (at least for inter-mediate KPI based monitoring), i.e., UE reporting ground-truth CSI to help NW compute the inter-mediate KPI. Assuming an obvious performance degradation for ongoing model has been observed by NW, NW side can:

a) check whether NW side CSI reconstruction model is correctly deployed by comparing the expected output and real output given the same input. Actually, checking CSI reconstruction model does not rely on real-time measurement, which can be done at any time.

b) compare the performance between NW-side original model (i.e., green part in the above Fig. 1) with reported ground-truth CSI from UE (NW-side original model refers to the model firstly trained by NW, and the possible model transfer/dataset delivery in option 3/4/5 are all based on the original model) and ongoing model (i.e., UE-side generation and NW-side reconstruction). If the performance of NW-side original model is satisfying while that of ongoing model is not good, we can identify that at least UE side causes the degradation. If the performance of NW-side original model also degrades, we can identify that at least data drifts happen for the ongoing model.

c) compare reconstructed target CSI based on the expected CSI feedback (i.e., output of NW-side original model) or reconstructed target CSI’ based on reported CSI feedback. If the reconstructed target CSI and reconstructed target CSI’ is not aligned (e.g., the difference is larger than the threshold), we can identify it’s UE side cause for the degradation. Otherwise, the UE side cause can be excluded. Or, we can compare the expected CSI feedback (i.e., output of NW-side original model) or reported CSI feedback. If the expected CSI feedback and reported CSI feedback is not aligned (e.g., the difference is larger than the threshold), we can identify UE side cause for the degradation. Otherwise, the UE side cause can be excluded.

Based on the above analysis, we can observe that identifying the cause of performance degradation is not a challenge for NW-side monitoring.

- Cause identification with UE-side monitoring

- Illustration of cause identification for UE-side monitoring

Identifying the cause of performance degradation for UE-side monitoring will be more complicated. Basically, it is challenging for NW side to identify whether it is data drift and/or UE side cause, because ground-truth CSI is not available at NW side for UE side monitoring. The only thing that NW can do is to check whether NW side CSI reconstruction model is correctly deployed by comparing the expected output and real output given the same input. Meanwhile, it is also not easy for UE to identify data drifts, since UE can hardly acquire the original CSI generation and reconstruction model simultaneously. For UE-side cause identification, we may further discuss according to the training collaboration considered (also illustrated in Fig. 2):

a) If the original CSI generation model is available at UE side (e.g., for option 3a-1/5a-1 and option 3b), UE could compare the expected output of transferred CSI generation model and real output of deployed CSI generation model to identify whether it is the UE side cause for the performance degradation. We believe that the comparison of expected output and real output is effective enough for cause identification, because model optimization at UE side usually targets mimicking the original CSI generation model with another model structure in most cases where it is available at UE side.

b) If the original CSI generation model is not available at UE side (e.g., for option 3a/5a-2 and option 4), it is challenging for UE to identify whether the deployed UE side model is correctly developed. In such cases, assistance information such as testing datasets from NW can help to identify UE side cause.

If there is performance degradation but no gNB-side or UE-side causes are identified, the cause of the performance degradation would then should be determined as data drift.

Based on the above analysis, even though identifying data drifts is challenging at both NW and UE when considering UE-side monitoring, there are still some approaches to identify UE side cause (at UE side) especially for option 3a/5a-1 and 3b.

To sum up, for two-sided model, the causes for performance degradation (e.g., UE sided cause, NW sided cause, or data drift, etc.) can be identified by conducting performance monitoring. For the scenario where the model is updated on the UE side, if it is proved through the performance monitoring process that the performance degradation is the cause of the NW side, the impact of the model update can be excluded to a certain extent. If it turns out that the performance degradation is the cause on the UE side, UE needs to take responsibility for the result and go to the next step, such as checking whether it was caused by a model update or other aspects. In general, it can be observed that the performance monitoring is sufficient for verification in post-deployment stage and no additional validation procedures are needed.

Furthermore, regarding the bullet that monitoring can be used for managing fall-back, changes in functionality, model update/model switching/model transfer, the subsequent model management decisions after the malfunction is identified by monitoring may be related to many factors. For example, it may depend on UE decision on how to implement performance recovery. And also, it may be up to the design of the performance monitoring process with involvement from other WGs.

Observation 2: For two-sided model, the causes for performance degradation (e.g., UE sided cause, NW sided cause, or data drift, etc.) can be identified by conducting performance monitoring

However, based on RAN1 progress, only one general agreement (shown as follows) for studying post-deployment performance monitoring was reached. RAN1 is still working on the details of this mechanism. From RAN4 perspective, it is suggested to wait for more progress before going into the detailed discussion of testing design.

RAN1 Agreement in RAN1#117 For option 1/3/4/5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field.

Based on our analysis of the potential issues associated with Option 1 (Pre-activation functionality/model update testing) and Option 2 (Post-deployment management based on LCM), we think that performance monitoring-based validation of the updated model's performance is the superior approach when considering factors:

- Lower Complexity – Performance monitoring leverages existing mechanisms, reducing additional implementation overhead.

- Higher Feasibility – No test environment/test duration restrictions, e.g. laboratory testing /GCF certification.

- Cost Efficiency – Avoids extra testing while ensuring reliable post-deployment model validation.

Option 2 ensures a balanced trade-off between validation costing and practical deployment efficiency.

Obsveration 3: Compared with Option 1, with consideration of the following factors, Option 2 ensures a balanced trade-off between validation costing and practical deployment efficiency

- Lower Complexity – Performance monitoring leverages existing mechanisms, reducing additional implementation overhead.

- Higher Feasibility – No test environment/test duration restrictions, e.g. laboratory testing /GCF certification.

- Cost Efficiency – Avoids extra testing while ensuring reliable post-deployment model validation.

Proposal 2: By leveraging the follow factors listed in Obsveration 3, RAN4 to consider verifying AI performance after device deployment by designing the test for performance monitoring in RAN4

- Summary

In this contribution, we provided our views on general aspects for AI/ML. Based on above analysis, following proposals are present.

Proposal 1: For Pre-activation functionality/model update testing, at least the following issues need to be taken into account

Table 1 Summary of Issues in Model Update Validation

| Issue | Key aspects | Standardization impact | Challenges or problems |
| --- | --- | --- | --- |
| 1. Major vs. Minor Model Changes | No clear definition of "major" (high-impact) vs. "minor" updates. | For Alt 1: Define universal criteria for diverse AI updates. For Alt 2: If the major/minor update is judged by UE itself, whether to mandate UE reporting this update. | Over-/under-validation risk |
|  | Solutions: - Alt 1: Defining standardized criteria - Alt 2: UE's sole discretion |  |  |
| 2. Lab vs. On-Device Validation | - Lab testing: - High cost/latency; - Whether GCF certification can fit the iterative AI is a question - On-Device testing: - Hardware variability biases results; - Offline validation lacks oversight. | - Lab testing: Lab testing scalability. - On-Device testing: How to Standardize on-device test procedures (this appears more like a UE-specific implementation) | - Lab testing: the end-to-end cycle - from model update completion → lab validation → deployment approval - may introduce prohibitive latency laboratory testing would demand exponentially greater effort - On-Device testing: Testing with alternative UEs cannot fully capture performance characteristics across all devices due to hardware implementation differences |

Observation 1: For one sided model, taking BM as the example, regarding the impact on the possible disalignment between the metric used for performance testing and the metric used for monitoring, it would be minimal by considering the following reasons:

- Based on existing simulation results, beam prediction accuracy and L1-RSRP difference accuracy are highly correlated, meaning that beam prediction accuracy alone is already sufficient to reflect the model’s predictive performance.

- Considering there is still the case that the prediction results support reporting only the beam ID (without the corresponding RSRP), Under this case, the metric used for performance testing and the metric used for monitoring can be aligned

Observation 2: For two-sided model, the causes for performance degradation (e.g., UE sided cause, NW sided cause, or data drift, etc.) can be identified by conducting performance monitoring

Obsveration 3: Compared with Option 1, with consideration of the following factors, Option 2 ensures a balanced trade-off between validation costing and practical deployment efficiency

- Lower Complexity – Performance monitoring leverages existing mechanisms, reducing additional implementation overhead.

- Higher Feasibility – No test environment/test duration restrictions, e.g. laboratory testing /GCF certification.

- Cost Efficiency – Avoids extra testing while ensuring reliable post-deployment model validation.

Proposal 2: By leveraging the follow factors listed in Obsveration 3, RAN4 to consider verifying AI performance after device deployment by designing the test for performance monitoring in RAN4

- References

- R4-2505107, WF on AI/ML air interface study, Ericsson