3GPP TSG-RAN WG4 Meeting # 115	                                      R4-2506501

Malta, MT, May 19 – 23, 2025

Agenda Item: 7.29.3.2

Source: MediaTek Inc.

Title: Discussion on CSI-RS L1 measurement for LTM

Document for: Discussion

- Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of CSI-RS L1 measurements.

- Discussion

2.1 Measurement procedure of CSI-RS based L1 measurement

In the past meetings, RAN4 had extensive discussions on the measurement procedure of CSI-RS based L1 measurement. In RAN4#114bis, the following agreement was reached.

Issue 4-2: Measurement procedure Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not. Issue 4-2-2: How to interpretate the UE capability (for FR2-1) Agreement: - For a UE not supporting the capability, Step 2 is necessary. - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2. - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped: - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1. - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

There are two open issues related to the measurement procedure. One is whether step 2 can be skipped for serving cell. We have strong concern on extending the new measurement procedure to the case when the candidate cell is a serving cell, e.g., PSCell or SCell. If so, then there will be two totally different measurement procedures for the same serving cell. From UE side, it is not acceptable to have two totally different CSI-RS measurement procedures for the same cell. UE will get confused on how to perform the measurement.

Another open issue is how to define the beam sweeping factor for CSI-RS L1 measurement when step 2 is skipped. In legacy procedure as shown in Fig.1, UE will perform beam sweeping at P1 and the beam sweeping factor is 8. At P2, UE uses the fixed Rx beam. At P3, UE will further refine the Rx beam and the beam sweeping factor is reported as maxNumberRxBeam. In our understanding, the goal of the new procedure is to get the best Rx beam for each CSI-RS resource. If the goal is to get the beam as fine as legacy P1/P2, then the beam sweeping factor can be the same as legacy P1, i.e., 8. If the goal is to get the beam as fine as legacy P3, then the beam sweeping factor can be the same as 8* maxNumberRxBeam. We are open to define the beam sweeping factor as 8 or 8* maxNumberRxBeam.

Proposal 1: For CSI-RS based L1-RSRP measurement in FR2-1, step 2 can not be skipped when the candidate cell is a serving cell, e.g., PSCell or SCell.

Proposal 2: For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, when step 2 is skipped, the beam sweeping factor can be 8 or 8* maxNumberRxBeam.

2.2 Definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

In the past meetings, RAN4 discussed whether and how to define intra-frequency for CSI-RS L1 measurement and reached the following progress.

Issue 4-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

As the definition is only for the convenience of capturing the requirements, we can be a bit open. We understand the intention of option 1 is to align with legacy intra-frequency definition of L3 CSI-RS measurement and SSB based measurement. But if we look back at the legacy intra-f and inter-f measurement in LTE, intra-f is always measured without gap and inter-f is measured with gap. We understand that there are intra-f without gap, intra-f with gap, inter-f with gap and inter-f without gap in NR due to configuration flexibility. But these concepts usually confuse others who are not familiar with RRM spec. Comparing the two options, we still prefer Option 2.

Proposal 3: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

2.3 RRM requirements for intra-f CSI-RS based L1-RSRP measurement when RTD<CP

For intra-f CSI-RS based L1-RSRP measurement (with the definition as proposal 3) when RTD<CP, UE can do the measurement on different cells simultaneously in FR1. Measurement delay of serving cell can be reused, i.e., max(TReport, ceil(M*P)*TCSI-RS). Measurement restriction and scheduling restriction requirements can also be reused.

Proposal 4: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell (when RTD<CP) in FR1, legacy measurement period, measurement restriction and scheduling restriction requirements are all applicable.

In FR2, the requirements will be different depending on SSB based L1 measurement is skipped or not.

- When SSB based L1 measurement is not skipped

Due to Rx beam for CSI-RS on different cells may be different, UE may not be able to perform CSI-RS L1-RSRP measurement on different cells simultaneously. In legacy requirements, when two CSI-RS resources overlap in time domain and require different Rx beam, measurement restriction apply, but not define a measurement delay with sharing factor among different CSI-RS resources. The reason behind is that the configuration of CSI-RS resources is quite flexible, and NW can avoid the colliding. So when two CSI-RS resources from difference cells overlap in time domain, we prefer to follow legacy, i.e., define measurement restriction but not a measurement delay with sharing factor among different CSI-RS resources. Then legacy measurement period for CSI-RS L1 measurement with repetition off is applicable.

In addition, scheduling restriction should be defined.

- When SSB based L1 measurement is skipped

In this case, we think the logic is similar, so when two CSI-RS resources from difference cells overlap in time domain, we prefer to follow legacy, i.e., define measurement restriction but not a measurement delay with sharing factor among different CSI-RS resources. And scheduling restriction should be defined too.

For measurement period, the structure in Table 9.5.4.2-2 is also applicable except the value of beam sweeping factor shall be updated.

Proposal 5: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when two CSI-RS resources from difference cells overlap in time domain.

Proposal 6: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define scheduling restriction.

Proposal 7: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, legacy measurement period for CSI-RS L1 measurement with repetition off is applicable when SSB based L1 measurement is not skipped.

Proposal 8: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, the structure of legacy measurement period for CSI-RS L1 measurement is applicable and the value of beam sweeping factor shall be updated when SSB based L1 measurement is skipped.

2.4 Others

For CSI acquisition, RAN1 had reached the following agreement.

- Baseline: - CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE - The report is sent directly to target cell - Optional with a UE capability: - CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

For the baseline (CSI acquisition after LTM CSC MAC-CE reception), UE will acquire CSI after UE is ready to receive data on target cell. We do not see any RRM requirement to be defined for this case. There was proposal to discuss what is the point in time during the cell switch delay UE starts CSI-acquisition. In our understanding, if T/F tracking can be skipped during cell switch, UE is able to start CSI acquisition after TLTM-processing. Otherwise, UE is able to start CSI acquisition after T/F tracking. UE will acquire CSI as early as possible. Before CSI information reported, NW can schedule the UE in a relative conservative way. If to add CSI acquisition delay in cell switch delay and NW starts scheduling after CSI report, the whole cell switch delay and interruption will be longer, which is not preferred.

Proposal 9: For the baseline (CSI acquisition after LTM CSC MAC-CE reception), there is no RRM impact.

For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), it is far more complex than CSI-RS L1-RSRP measurement. Similar issues with CSI-RS based L1-RSRP measurement from the candidate cell should be discussed, such as the pre-requisite condition, maximum RTD value, gap-less vs. gap-based, etc. But not all the agreements for CSI-RS L1-RSRP measurement are applicable here. For example, UE can perform L1-RSRP measurement on truncated CSI-RS resources (48 RBs), but this may be not workable for CSI acquisition. At least from our point, we prefer not to define any RRM requirements for the optional feature.

Proposal 10: For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), not to define any RRM requirements in R19.

- Summary

In this paper, we provide some views on CSI-RS measurement on neighbor cell. We have the following proposals:

Proposal 1: For CSI-RS based L1-RSRP measurement in FR2-1, step 2 can not be skipped when the candidate cell is a serving cell, e.g., PSCell or SCell.

Proposal 2: For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, when step 2 is skipped, the beam sweeping factor can be 8 or 8* maxNumberRxBeam.

Proposal 3: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

    - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Proposal 4: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell (when RTD<CP) in FR1, legacy measurement period, measurement restriction and scheduling restriction requirements are all applicable.

Proposal 5: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when two CSI-RS resources from difference cells overlap in time domain.

Proposal 6: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define scheduling restriction.

Proposal 7: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, legacy measurement period for CSI-RS L1 measurement with repetition off is applicable when SSB based L1 measurement is not skipped.

Proposal 8: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, the structure of legacy measurement period for CSI-RS L1 measurement is applicable and the value of beam sweeping factor shall be updated when SSB based L1 measurement is skipped.

Proposal 9: For the baseline (CSI acquisition after LTM CSC MAC-CE reception), there is no RRM impact.

Proposal 10: For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), not to define any RRM requirements in R19.

4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#114bis.