3GPP TSG-RAN WG4 Meeting # 115	                                      R4-2506504

Malta, MT, May 19 – 23, 2025

Agenda Item: 7.29.4

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for R19 mobility

Document for: Discussion

- Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

In this contribution, we would like to discuss the RRM performance requirements to be defined.

- Discussion

In RAN4#112bis, the following agreement was reached for CSI-RS L1-RSRP measurement on neighbor cell (R4-2416862). Last meeting, RAN4 agreed not to define RAN4 requirements for L1 measurement with gap or RTD>CP case for CSI-RS based L1 measurement on neighbor cell.

Issue 2-11: CSI-RS resource density and BW - Agreements: - For CSI-RS based L1-RSRP measurement on neighbor cell, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs.

Based on the above agreement, legacy CSI-RS L1-RSRP measurement requirements for serving cell are also applicable to neighbor cell.

Proposal 1: For CSI-RS L1-RSRP measurement on neighbor cell, legacy CSI-RS L1-RSRP accuracy requirements for serving cell are also applicable.

- Summary

In this paper, we provide some view on performance requirements of R19 mobility. We have the following proposal:

Proposal 1: For CSI-RS L1-RSRP measurement on neighbor cell, legacy CSI-RS L1-RSRP accuracy requirements for serving cell are also applicable.

4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.