3GPP TSG-RAN WG4 Meeting #115	R4-2506518

Malta 19th – 23rd May 2025

Source: 	Nokia

Title: 					n26 7 MHz A-MPR simulations and proposals

Agenda Item: 			7.8.3

Document for:	Approval

# 1	Introduction

We simulate the A-MPR for 7 MHz CBW on operating band n26. We consider the additional spurious emission masks signaled via NS_12, NS_13, NS_14 and NS_15, and we make proposals for the A-MPR.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Operating band n26 (UL 814 MHz – 849 MHz)

- Carrier positions according to the WF of the last meeting [1]:

  - NS_12 considering 7 MHz deployed within 814-824 MHz

  - NS_13 considering 7 MHz deployed in 817-824 MHz (Fc = 820.5 MHz).

  - NS_14 considering 7 MHz deployed within 824-849 MHz

  - NS_15 considering 7 MHz deployed within 814-845 MHz or within 841.5-849 MHz.

- 7 MHz CBW with NRB = 35 as defined in [2]

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Only additional spurious emission masks from NS_12, NS_13, NS_14 and NS_15 are simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

## 2.2	NS_12 mask

Table 6.5.3.3.17-1: Additional requirements "NS_12"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

## 2.3	NS_13 mask

Table 6.5.3.3.18-1: Additional requirements "NS_13"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 817 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

## 2.4	NS_14 mask

Table 6.5.3.3.19-1: Additional requirements "NS_14"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz, 15 MHz, 20MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 824 MHz NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

## 2.5	NS_15 mask

Table 6.5.3.3.20-1: Additional requirements "NS_15"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

# 3	Simulation results

We provide complete sets of the A-MPR triangles from the simulations at the most relevant center frequencies in Appendix. In this section we present some highlights of the results.

## 3.1	NS_12

A-MPR is needed everywhere in the deployed frequency range 814-824 MHz. In Fig. 1 we show examples of the A-MPR triangles at the lower edge of the deployed range.

Figure 1: Examples of the simulated A-MPR to satisfy the NS_12 additional spurious emission limit for 7 MHz channel at Fc = 817.5 MHz.

## 3.2	NS_13

In Fig. 2 we present examples of the A-MPR triangles at Fc = 820.5 MHz.

Figure 2: Examples of the simulated A-MPR to satisfy the NS_13 additional spurious emission limit for 7 MHz channel at Fc = 820.5 MHz.

## 3.3	NS_14

No A-MPR is needed for 7 MHz CBW.

## 3.4	NS_15

A-MPR is needed when Fc is above 838.1 MHz. In Fig. 3 we present examples of the A-MPR triangles at Fc = 845.5 MHz and Fc = 841.5 MHz. The rather high A-MPR values for pi/2-BPSK are noteworthy. Sharps spikes in the spectrum are characteristic for pi/2-BPSK and they can be significant with small MBWs. For pi/2-BPSK the largest A-MPR values come from the IMD between the allocation and the carrier leak.

Fc = 841.5 MHz

Fc = 845.5 MHz

Figure 3: Examples of the simulated A-MPR to satisfy the NS_15 additional spurious emission limit for 7 MHz channel at Fc = 845.5 MHz (top row) and Fc = 841.5 MHz (bottom row).

# 4	A-MPR proposals

We make the following A-MPR proposals based on our simulation results. We have highlighted the new entries in the tables with yellow color.

## 4.1	NS_12

Proposal 1: RAN4 shall consider defining A-MPR for n26 PC3 NS_12 as shown in tables 6.2.3.21-1 and 6.2.3.21-2 below.

Table 6.2.3.21-1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.16 | >0 | A3 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 6.2.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | 4 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | 4 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | 4 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |  |

## 4.2	NS_13

Proposal 2: RAN4 shall consider defining A-MPR for n26 PC3 NS_13 as shown in tables 6.2.3.22-1 and 6.2.3.22-2 below.

Table 6.2.3.22-1: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
| 7MHz | Fc = 820.5 | ≤1.62 | <1.44 | A5 |
|  |  | ≤1.98 | ≥1.44 | A6 |

Table 6.2.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 | 2 | 2.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 | 2 | 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 | 2.5 | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |  | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |  |  |

## 4.3	NS_14

Proposal 3: RAN4 shall consider defining no A-MPR for 7 MHz CBW with n26 PC3 NS_14.

## 4.4	NS_15

Proposal 4: RAN4 shall consider defining A-MPR for n26 PC3 NS_15 as shown in tables 6.2.3.24-1 and 6.2.3.24-2 below.

Table 6.2.3.24-1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 7MHz | Fc = 845.5 | ≥ 0 | ≥ -1.7*12*SCS*RBend + 8.2 | A7 |
|  |  | ≤ 1.26 | ≥ 12*SCS*RBend – 0.15 | A8 |
|  | 838.1 < Fc ≤ 841.5 | ≥ 4.68 | ≥ -0.7*12*SCS*RBend + 7.8 | A9 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |

Table 6.2.3.24-2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Edge/Inner | Edge/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | 10.5 | 1 | 1 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | 9 |  | 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | 9 |  | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | 9 |  | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | 9 |  |  |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | 9.5 |  | 4.5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | 9.5 |  | 4.5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | 9.5 |  | 4.5 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | 9.5 |  |  |

We present examples of the A-MPR triangles in Fig. 4. Complete sets of the A-MPR triangles are provided in Appendix.

Fc = 841.5 MHz

Fc = 845.5 MHz

Figure 4: Examples of the proposed A-MPR for NS_15 with 7 MHz channel at Fc = 845.5 MHz (upper row) and Fc = 841.5 MHz (bottom row).

# 5	Conclusions

We have performed A-MPR simulations for n26 and 7 MHz CBW using the emission requirements signaled via NS_12, NS_13, NS_14 and NS_15. We have made the following proposals based on the simulations.

Proposal 1: RAN4 shall consider defining A-MPR for n26 PC3 NS_12 as shown in tables 6.2.3.21-1 and 6.2.3.21-2 below.

Table 6.2.3.21-1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7MHz |  | ≤2.16 | >0 | A3 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 6.2.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | 4 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | 4 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | 4 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | 5 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |  |

Proposal 2: RAN4 shall consider defining A-MPR for n26 PC3 NS_13 as shown in tables 6.2.3.22-1 and 6.2.3.22-2 below.

Table 6.2.3.22-1: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
| 7MHz | Fc = 820.5 | ≤1.62 | <1.44 | A5 |
|  |  | ≤1.98 | ≥1.44 | A6 |

Table 6.2.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 | 2 | 2.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 | 2 | 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 | 2.5 | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |  | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | 3.5 | 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |  |  |

Proposal 3: RAN4 shall consider defining no A-MPR for 7 MHz CBW with n26 PC3 NS_14.

Proposal 4: RAN4 shall consider defining A-MPR for n26 PC3 NS_15 as shown in tables 6.2.3.24-1 and 6.2.3.24-2 below.

Table 6.2.3.24-1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 7MHz | Fc = 845.5 | ≥ 0 | ≥ -1.7*12*SCS*RBend + 8.2 | A7 |
|  |  | ≤ 1.26 | ≥ 12*SCS*RBend – 0.15 | A8 |
|  | 838.1 < Fc ≤ 841.5 | ≥ 4.68 | ≥ -0.7*12*SCS*RBend + 7.8 | A9 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |

Table 6.2.3.24-2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Edge/Inner | Edge/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | 10.5 | 1 | 1 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | 9 |  | 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | 9 |  | 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | 9 |  | 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | 9 |  |  |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | 9.5 |  | 4.5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | 9.5 |  | 4.5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | 9.5 |  | 4.5 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | 9.5 |  |  |

# 6	References

[1] R4-2505098, WF on requirements for 7MHz channel bandwidth, Ericsson

[2] R4-2502870, WF on requirements for 7MHz channel bandwidth, Ericsson

# 7	Appendix: A-MPR triangles

## 7.1	NS_12 simulations

## 7.2	NS_13 simulations

## 7.3.1	NS_15 simulations

## 7.3.2	NS_15 proposal