3GPP TSG-RAN WG4 Meeting #115                                 R4-2506520

St. Julians, Malta, 19th – 23rd May, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on power domain enhancement for NR single carrier

Agenda Item:	7.1.2.2.1

Document for:	Approval

- Introduction

In RAN#114bis, a WF was agreed upon in [1]. This document aims to progress the further open issues.

- 2. Discussion

Width/RB numbers of extended CBW at each side of UE CBW

Agreement at RAN4#114bis:

- Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature

  - In addition, consider a limited number of additional fixed values with capability

Regarding whether to apply this for just the maximum CBW or smaller CBWs, we are open minded. There may be cases where the relaxed emission may apply in guardband, where the UE cannot be configured with a wider bandwidth, so it may make sense here to allow for smaller UE CBWs.

On applying smaller multiples than 0.5*, we would suggest limiting to no more than 2 points, due to diminishing returns with more granularity, not to mention testing complexity for devices.

Proposal 1: Shifted OOBE can be defined for any existing UE CBW up to the maximum CBW supported by UE.

Proposal 2: If smaller values other than 0.5* UE CBW are specified, they should be limited to a maximum of 2 discrete points.

On whether to apply 0.5*UE CBW or 0.5*NRB, we have simulated the Symmetrical and Asymmetrical shifted OOBE for 0.5 * CBW and ceil (0.5 * NRB) shift (with original UE CBW guardband added). The 20MHz results below are for 15kHz SCS which is a worst case than 30kHz SCS in terms of NRB and spectrum utilization. Therefore, the equivalent ratios can apply for 30kHz SCS too. The Tables below show that in both cases, there should be no problem to apply either 0.5*UE CBW or ceil(0.5 * NRB).

Proposal 3: Either 0.5 * CBW and ceil(0.5 * NRB) shift are acceptable to be adopted, with the following requirements applicable for modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms for PC3 (with applicability for PC2 FFS):

- Maximum inner RB allocation of LCRB = NRB for symmetrical shift of 0.5* on each side, and LCRB = 2*NRB/3 for an asymmetrical shift of 0.5* on one side and zero on the other side. Maximum inner RB allocation of LCRB for any potential additional multiplier values smaller than 0.5*NRB are FFS.

- If the 0.5*NRB approach is selected, a guardband of the “original UE CBW” guardband should be added between the last RB of 0.5*NRB and the start of the shifted OOBE region.

Table 1a: Evaluation results for Symmetric shifted OOBE (20MHz CBW with 0.5* shift both sides

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {0, 100}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {0, 106}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {0, 100}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {0, 106}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |

Table 1b: Evaluation results for Asymmetrical shifted OOBE (20MHz CBW with 0.5* shift right side)

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {35, 70}, {105, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {35, 70}, {105, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, QPSK, 23dBm | {91, 182}, {272, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |

Table 1c: Evaluation results for Asymmetrical shifted OOBE (100MHz CBW with 0.5* shift right side)

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {91, 182}, {272, 1} | ALL PASS | ALL PASS |

Why to specify the extension as “shifted OOBE” rather than “extended CBW”

When defining the SEM/ACLR requirements as integrated according to UE CBW, but where the starting point is shifted from the channel edge, this is not the same as the approach taken for scaling SEM/ACLR integration over a wider CBW.

It cannot be assumed that the existing MPR rules can be applied for any combination of UE CBW and wider overlaid CBW in the same way as for a partial RB allocation operating in such a wider CBW. This would need all of the cases (including all UE CBW locations within each wider CBW) to be simulated for every combination of CBW and wider CBW, which appears to be a lot of effort for very limited gain.

Proposal 4: Define the term in the specifications as “shifted OOBE” rather than “extended CBW”

InBand Emissions (IBE) between UE CBW edge and extended edge

WF from RAN4#114bis

- Case 1 and case 3 could be further down selected

- GB of UE CBW may or may not be scheduled to other UEs

- Extended IBE can be applied in the GB of UE CBW

  - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW

The 2 cases left for down-selection are shown below:

Figure 1: Case 1 for IBE

Figure 2: Case 3 for IBE

At RAN4#114bis, results were shown that suggested that an MPR relaxation is needed with a full RB allocation for case 3, where IBE applies directly from the edge allocated RBs. We have performed analysis of the General IBE mask, and we have still not found any reason to suggest that the MPR would be any worse than the inner RB allocation MPR if IBE is applied from the edge allocated RBs instead of after the guardband or needing the IBE relaxation for case 1 vs case 3. We also analysed the different formulas of the general IBE mask, and found that when there is a full RB allocation, the “condition 1” only ever applies for QPSK and 16QAM modulation for RBs in a range far beyond 0.5*LCRB from the allocated RBs, and therefore for a full RB allocation with 0.5*NRB shift or 0.5 * UE CBW shift, this would be in the new (shifted) OOB region outside of the so-called “extended BW”. And for “condition 2” NRB does not apply so a change of NRB would make no difference.

While we acknowledge that relaxing IBE could allow the UE to transmit higher power, the question is whether the “inner RB allocation MPR” requirements can be achieved or not, and we have found no evidence to suggest that would not be the case. Also, intuitively, if IBE is not an issue for inner RB allocation MPR where there is no ACLR/SEM issue, why would it now be more impacting than inner RB allocation MPR if ACLR/SEM is no longer a bottleneck. Therefore, we see have not found any justification for relaxing the IBE requirements and have seen no justification for a more relaxed MPR than inner RB allocation MPR to be required in this case.

Proposal 5: Adopt Case 3 for IBE with IBE starting from the last allocated RB, unless there is clear evidence and explanation as to why inner RB allocation MPR cannot be achieved. If it is proven that inner RB allocation MPR cannot be achieved, then consider a relaxation as necessary.

Potential MPR rules

The MPR rules need to be discussed once we see the full picture of how many shift multipliers would be defined, and corresponding simulation results proving that LCRB can scale with the shifted OOBE for values other than 0.5*. However, we provide the following possible options below to cover both Symmetrical and asymmetrical OOBE shifts,

Option 1: Applying for a 0.5* NRB or 0.5* UE CBW shift

RBStart,Low = min(max(1, floor(LCRB/2)), OOBExt,Low),

where OOBExt,Low:

= 0, if OOBE shift from lower UE CBW edge applied,

= ∞ otherwise

RBStart,High = NRB – min(max(1, floor(LCRB/2)), OOBExt,High) – LCRB,

where OOBExt,High:

= 0, if OOBE shift from upper UE CBW edge applied,

= ∞ otherwise

The RB allocation is an inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High

LCRB ≤ ceil(NRB/2 * LCRB_ Ext) , where LCRB_ Ext:

= 2, if OOBE shifted by 0.5 * UE CBW on both upper and lower edge

= 4/3, if OOBE shifted by 0.5 * UE CBW on only one of either upper or lower edge

= 1 otherwise

Option 2: Applying only for any shifts – pending simulations proving that such current BW scaling can apply to all shifts.

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

- NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1

- RBStart,Low = ceil [ LCRB * (NRB - (NRB + RBExt,Low)*2/3) / ((NRB + RBExt,Low)*2/3) ]

where RBExt,Low = number of RBs extension of start of OOBE mask from lowest RB

where ceil(x) is the smallest integer greater than or equal to x.

- RBStart,High = NRB – ceil [ LCRB * (NRB - (NRB + RBExt,High)*2/3) / ((NRB + RBExt,High)*2/3) ] – LCRB,

where RBExt,High = number of RBs extension of start of OOBE mask from highest RB

where ceil(x) is the smallest integer greater than or equal to x.

- The RB allocation is an inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High

and

LCRB ≤ floor ( NRB / [( ceil [ (NRB - ((NRB + RBExt,Low)*2/3)) / ((NRB + RBExt,Low)*2/3) ]) + ( ceil [(NRB – ((NRB + RBExt,High)*2/3)) / ((NRB + RBExt,High)*2/3) ] ) + 1 ] )

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

Proposal 6: Consider the example spec changes in this section as a starting point for modification to the MPR section of TS 38.101-1.

Network-UE signalling aspects

It is important to quickly identify what sort of network-UE interactions are needed and inform RAN2 before the end of this quarter, given that RAN2 will functionally freeze RRC in September this year.

There are some different potential motivations for network to UE signalling, and in past meetings they have been somewhat bundled together in discussions. We believe it would be best to separate them.

Potential Motivations (1,2,3) are:

- To indicate to the UE that the MNO permits the application of relaxed OOBE in its spectrum block. This may be needed due to any geographically specific regulatory conditions not allowing the relaxed OOBE, or it may just be because the operator has some concern about the UE applying IBE requirement levels instead of OOBE outside of its CBW but inside the operator’s spectrum block.

Our view: This makes sense and de-risks deployment/adoption of the feature.

- There may also be regulatory or MNO operational reasons as to why a symmetrical shift may not be permitted, even if the device is not operating right at the edge of the BS CBW, e.g. some specific RAN sharing scenario with common Base Station bandwidth applying to both operators, but with separate spectrum blocks.

Our view: We think it would be more appropriate if the UE could rely on the network to indicate to UE what sort of shift would be permitted, as the network operator is more aware of the operating environment. The UE could also additionally verify that it is adhering to regulations.

- To indicate exactly what is the level of extension permitted by the operator. This would assume that there is some network configurability of this relaxation.

Our view: If such configuration possibilities are limited, then this it could be acceptable. The UE could also additionally verify that it is adhering to regulations.

With regards to the type of signalling, we propose to define UE specific RRC signalling because:

- Different UEs may be operating in different parts of the Base Station bandwidth.

- An operator strategy may be to only allow the extension for certain UEs within its network.

In the case of a multi-carrier network, the UE would not know whether the adjacent carrier is from the same operator until it was configured with CA.

Suggested RRC (network to UE) signalling concept:

Network to convey to UE (UE specific) whether shifted OOBE is permitted, and include the following additional information:

- Whether the shifted OOBE permitted is “symmetrical” or “asymmetrical”.

  - If symmetrical, default OOB shift multiplier value = 0.5

  - If asymmetrical, a default 0.5 shift applies on one side, and zero on the other side. At least an indication of whether the shift applies to left or right side needed here.

- If more multiplier values specified, then RAN4 needs to decide whether:

  - Different non-zero shifts may apply to both sides.

  - In such case, the explicit values of the shifts would need to be signalled for each side.

Proposal 7: Request RAN2 to define UE specific RRC signalling to indicate to the UE whether shifted OOBE is permitted, including the following additional information and RRC parameters:

- Whether the shifted OOBE permitted is “symmetrical” or “asymmetrical”.

  - If symmetrical, default OOB shift multiplier value = 0.5

  - If asymmetrical, a default 0.5 shift applies on one side, and zero on the other side. At least an indication of whether the shift applies to left or right side needed here.

- If more multiplier values specified, then RAN4 needs to decide whether:

  - Different non-zero shifts may apply to both sides.

  - In such case, the explicit values of the shifts would need to be signalled for each side.

The UE could also additionally verify that it is adhering to regulations.

Detailed RRC parameters:

| Name | Values | Conditions | Comments | UE or cell specific |
| --- | --- | --- | --- | --- |
| permittedShifted OOBE-Type | CHOICE (Symmetrical, Asymmetrical) |  | Default shift value = 0.5*. | UE specific |
| asymmetricalShifted OOBE-Applicability | CHOICE (Left side, Right side) | Ignored by UE if permittedShifted OOBE-Type value = “Symmetrical” | Default shift value = 0.5* | UE specific |
| FFS other parameters and whether this changes the structure above |  |  |  |  |

Other potential pre-requisites could be that “dedicated channel Bandwidth” is configured to the UE.

Proposal 8: RAN4 to decide whether Dedicated Channel Bandwidth configuration is a pre-requisite.

UE capability signalling

Agreement in RAN4#114bis:

- An optional UE capability to indicate whether it supports the CBW extension based MPR reduction feature

- To discuss the following aspects

  - Whether to have separate UE capabilities for scenario 1 and scenario 2

  - Whether to indicate multiple values of fixed extension capability

We do not really see the difference between scenario 1 and scenario 2 from a UE implementation perspective. It more comes down to what is permitted in the operating environment.

We do think that we should have separate capabilities for the default 0.5* shift and any other shifted values, as more shifted values add more testing complexity and IODT, and the benefits may be limited to very specific deployments.

RAN4 has not explicitly discussed whether the capability would apply for specific power classes. Our starting point was RedCap so we had assumed 23dBm. However, 26dBm power class is also being defined for RedCap. Our simulation analysis has also been focused on PC3 so far. We would be open to apply for PC2 but we would need more time to evaluate whether the same assumptions can apply for PC2, also considering that “edge RB allocations” are defined for PC2.

Proposal 9: Define 0.5* shift as the default capability for the supported feature. If additional shifts of <0.5* are specified, then define separate UE capability(ies) for additional shifts.

Proposal 10: Discuss further until RAN4#116 whether to apply also this feature also for PC2.

- 3. Conclusion & Proposals

Proposal 1: Shifted OOBE can be defined for any existing UE CBW up to the maximum CBW supported by UE.

Proposal 2: If smaller values other than 0.5* UE CBW are specified, they should be limited to a maximum of 2 discrete points.

Proposal 3: Either 0.5 * CBW and ceil(0.5 * NRB) shift are acceptable to be adopted, with the following requirements applicable for modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms for PC3 (with applicability for PC2 FFS):

- Maximum inner RB allocation of LCRB = NRB for symmetrical shift of 0.5* on each side, and LCRB = 2*NRB/3 for an asymmetrical shift of 0.5* on one side and zero on the other side. Maximum inner RB allocation of LCRB for any potential additional multiplier values smaller than 0.5*NRB are FFS.

- If the 0.5*NRB approach is selected, a guardband of the “original UE CBW” guardband should be added between the last RB of 0.5*NRB and the start of the shifted OOBE region.

Proposal 4: Define the term in the specifications as “shifted OOBE” rather than “extended CBW”

Proposal 5: Adopt Case 3 for IBE with IBE starting from the last allocated RB, unless there is clear evidence and explanation as to why inner RB allocation MPR cannot be achieved. If it is proven that inner RB allocation MPR cannot be achieved, then consider a relaxation as necessary.

Proposal 6: Consider the example spec changes in this section as a starting point for modification to the MPR section of TS 38.101-1.

Proposal 7: Request RAN2 to define UE specific RRC signalling to indicate to the UE whether shifted OOBE is permitted, including the following additional information and RRC parameters:

- Whether the shifted OOBE permitted is “symmetrical” or “asymmetrical”.

  - If symmetrical, default OOB shift multiplier value = 0.5

  - If asymmetrical, a default 0.5 shift applies on one side, and zero on the other side. At least an indication of whether the shift applies to left or right side needed here.

- If more multiplier values specified, then RAN4 needs to decide whether:

  - Different non-zero shifts may apply to both sides.

  - In such case, the explicit values of the shifts would need to be signalled for each side.

The UE could also additionally verify that it is adhering to regulations.

Detailed RRC parameters proposed:

| Name | Values | Conditions | Comments | UE or cell specific |
| --- | --- | --- | --- | --- |
| permittedShifted OOBE-Type | CHOICE (Symmetrical, Asymmetrical) |  | Default shift value = 0.5*. | UE specific |
| asymmetricalShifted OOBE-Applicability | CHOICE (Left side, Right side) | Ignored by UE if permittedShifted OOBE-Type value = “Symmetrical” | Default shift value = 0.5* | UE specific |
| FFS other parameters and whether this changes the structure above |  |  |  |  |

Proposal 8: RAN4 to decide whether Dedicated Channel Bandwidth configuration is a pre-requisite.

Proposal 9: Define 0.5* shift as the default capability for the supported feature. If additional shifts of <0.5* are specified, then define separate UE capability(ies) for additional shifts.

Proposal 10: Discuss further until RAN4#116 whether to apply also this feature also for PC2.

- 4. References

- R4-2505099, “WF on power domain enhancements”, Moderator (Huawei)

- R4-2504240, “Discussion on power domain enhancement for NR single carrier”, MediaTek