# 3GPP TSG-RAN WG4 Meeting #115r		R4-2506535

# Malta, 07th – 11th April, 2025

Title:	Views on BS FR1 transmitter co-existence spurious emission requirements

Source:	Qualcomm Incorporated

Agenda item:	7.13.3.3

Document for:	Approval

# 1.	Introduction

It was agreed in [1] that RAN4 Add the new objective of co-existence transmitter spurious emission limits according to the request from ETSI/TFES to the ongoing Rel-19 NR BS RF requirement evolution work item, with the following objectives.

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed.

In [2], a WF was agreed on the preliminary views on how to study and assess the co-existnece Tx spurious limits. In this contribution, we share our views on the open items that were agreed in [2].

# 2 	Discussion

In [2] two options regarding the evaluation methodology adopted within RAN4 to derive the Tx spurious emission requirements were agreed as shown below.

- Consider MCL methodology, including AAS and non-AAS implementations. - TR 25.942 is used as reference for the methodology derivation. - System level evaluations are not precluded.

In our contribution, we will investigate the protection of UE receiver (Downlink, BS-to-UE) and protection of BS receiver (Uplink, BS-to-BS). In order to determine the proposed emission level, the following link budget following TR 25.942 framework is adopted

pemissions dBmMHz=KT dBmHz+BMHz+NF dB+LdB-M[dB]

Where the below parameters are assumed in this contribution

- KT= -174dBmHZ.

- B=1 MHz.

- Noise figure for UE or BS depending on the scenario.

- desensitisation assumption: 3 dB and 0.8 dB for protection of UE receiver and BS receiver, respectively.

- L is the coupling loss between Tx and Rx, which will be evaluated depending on the scenario.

## 2.1 Protection of UE receiver (Downlink, BS-to-UE)

This scenario considers the downlink protection of other operators in the same geographical area. Following TR 25.942, we consider minimum distance between aggressor BS and victim UE to be equal 5m. Assuming operation at n104 (operational frequency = 6425 MHz), the FSPL at 5m equals 62.5 dB. Assuming UE noise figure to be either as [9 dB – 13 dB] (agreed for n79 in RAN4, see R4-2410576 or agreed for 7.125 – 8.4 GHz study item in RAN4, see R4-2414302) and 3 dB desensitisation, the evaluated pemissions dBmMHz= [-42.5, -38.5] dB. Compared to legacy emission requirements of -52 dBm/MHz, only by considering relevant FSPL and noise figure values for upper FR1 bands (i.e., n104), one can observe a relaxation of [9.5 dB, 13.5 dB].

Observation 1: For protection UE receiver, only by considering relevant FSPL and noise figure values for upper FR1 bands (i.e., n104), relaxation of [9.5 dB, 13.5 dB] could be realized depending on the underlying assumptions.

If one would like to consider a less conservative value of the coupling loss between BS and UE, one can resort to system level modelling. However, assuming 100% grid shift, propagation model of TR 38.901, and ISD of 500m will lead to even more relaxed requirements, since the coupling loss values will be much higher compared to the 5m conservative assumption that was adopted in TR 25.942.

Observation 2: With system level assumptions such as 100% grid shift, 38.901 propagation model and ISD = 500m, conservative assumptions to define emission requirements that were considered TR 25.942 would not be applicable anymore.

In order 	to account for the impact of AAS in the link budget calculation, a similar methodology as presented in Section 2.2.1 in [3] could be adopted, where RAN4 would agree on vertical and azimuthal antenna discrimination values based on the subarray pattern in the OOB domain. Such gain values would be plugged into the link budget calculation to derive the required emission requirement. For example, Figure 1 below shows the radiation pattern of an AAS BS (θsubtilt=3o) in the OOB domain assuming that the BF gain diminishes and only the subarray and element gain is residual in the OOB domain. Offsets for the vertical and azimuthal planes could be decided and plugged into the above link budget analysis to derive the requirements with AAS impacts considered.

Figure 1 Subarray gains assuming three elements per subarray in the vertical plane

Proposal 1: To account for AAS impacts, RAN4 to account for vertical and azimuthal offsets of the subarray antenna gain in the link budget analysis to derive the emission requirements.

## 2.2 Protection of BS receiver (Uplink, BS-to-BS)

This scenario considers the uplink protection of other operators in the same geographical area. Similar to the Section 2.1, the evaluation of the protection emission requirements mainly boils down to the correct evaluation of the BS2BS coupling loss. Based purely on geographical deployment, Figure 2 shows the BS-to-BS coupling loss assuming UMa scenario with UMa 38.901 propagation model, BS antenna height = 25m, ISD=500m, and 100% grid shift between operator A and operator B. Note that in TR 25.942, worst case assumption of 288m was assumed to derive the inter-BS pathloss. Following TR 25.942 methodology, FSPL pathloss at 6425 GHz would be equal to 97.7 dB, which represents the 8% of the CDF shown in Figure 2. Intuitvely, the below figure does not account for the victim BS and aggressor BS antenna gains. It is important to agree within RAN4 if the antenna gains entail only the subarray and single element component or full AAS model. Additionally, RAN4 needs to further discuss which percentile of the curve might be meaningful compared to the approach that was adopted in TR 25.942.

Figure 2 inter-BS pathloss for UMa assuming 100% grid shift and UMa 38.901 propagation model

Proposal 2: For the protection of BS receiver, RAN4 to further discuss if the full AAS gain or only the subarray gain to be account for when evaluating the BS-to-BS coupling loss.

Proposal 3: For the protection of BS receiver, RAN4 to further discuss if statistical approach (i.e., SLS-inspired approach) to be followed or the MCL approach in TR 25.942. If the former, RAN4 to discuss which percentile of the BS-to-BS coupling loss CDF to be utilized when evaluating the updated emissions.

# Conclusion

In this contribution we provide our views on BS FR1 transmitter co-existence spurious emission requirements. Our observations and proposals can be summarized as follows:

Observation 1: For protection UE receiver, only by considering relevant FSPL and noise figure values for upper FR1 bands (i.e., n104), relaxation of [9.5 dB, 13.5 dB] could be realized depending on the underlying assumptions.

Observation 2: With system level assumptions such as 100% grid shift, 38.901 propagation model and ISD = 500m, conservative assumptions to define emission requirements that were considered TR 25.942 would not be applicable anymore.

Proposal 1: To account for AAS impacts, RAN4 to account for vertical and azimuthal offsets of the subarray antenna gain in the link budget analysis to derive the emission requirements.

Proposal 2: For the protection of BS receiver, RAN4 to further discuss if the full AAS gain or only the subarray gain to be account for when evaluating the BS-to-BS coupling loss.

Proposal 3: For the protection of BS receiver, RAN4 to further discuss if statistical approach (i.e., SLS-inspired approach) to be followed or the MCL approach in TR 25.942. If the former, RAN4 to discuss which percentile of the the BS-to-BS coupling loss CDF to be utilized when evaluating the updated emissions.

# Reference

[1]		RP-242432, “Moderator's summary of discussion about R19-RAN4-BSRF”, RAN4 Chair (Huawei), RAN #106, Madrid, Spain.

[2]	R4-2504769, “Way Forward for Tx spurious emissions co-existence limits re-evaluation”, RAN4#114bis, Huawei.

[3]	R4-2504298, “Transmitter co-existence spurious emission requirements”, RAN4#114bis, Huawei.