3GPP TSG-RAN WG4 Meeting#115		R4-2506543

Malta, MT, May 19th -23rd, 2025

Agenda item:	7.5.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

Introduction

In RAN4#114bis meeting, a WF [1] was approved. In this contribution, we continue to provide some initial views on methods for reducing the number of UE RX chains.

Discussion

Triggering condition to enable “One RF Rx chain”

- Issue 2-3: Triggering condition to enable ”One Rx RF chain” RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode: - Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed - Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified - Note: detailed conditions to enable/disable can be further discussed and decided separately

According to the last meeting agreement, there are two options regarding the triggering condition to enable the one Rx RF chain mode. For option 1, no new indication from UE is needed. For option 2, UE needs to do the measurement. Based on simulation results from other companies, the in-gap interference maybe 20dB higher than wanted signals. In our understanding, UE need to measure this in-gap interference to determine and indicate to the network whether it can support non-contiguous CCs with one Rx RF chain or not, otherwise, it will cause ping-pong release and reconfiguration, which result in more interruption. Regarding the measurements, gNB triggers UE or UE autonomously to measure. UE needs to measure the wanted PCC/SCC, in-gap interference, Tx leakage, image of LO and other intermodulation products. When the DL SNR degradation is no greater than allowed degradation or total interferences are lower than threshold, UE indicates to the network it can support non-contiguous CCs with one Rx RF chain and enables it.

Proposal 1: Support option 2. UE needs to measure the wanted PCC/SCC, in-gap interference, Tx leakage, image of LO and other intermodulation products to enable or disable the one Rx RF chain mode.

How to indicate UE capability for supporting “one Rx RF chain” for DL fragmented carriers

#### Issue 2-1: How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers - Consider the following alternatives - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication - Alternative 1 has no modification of CA configurations in RAN4 - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers. - Alternative 1a has no modification of CA configurations in RAN4 - Alternative 2: use the new notation for CA configurations with single Rx RF Chain - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

In terms of indicating UE capability, in order to reduce the RAN2 impact and RAN4 spec work, Alternative 1a is preferred.

Proposal 2: Alternative 1a are preferred. Use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

Discussion on semi-statically switch hardware resources procedure and fall back considerations

- Further discussions on - CA fallback with/without UE architecture switching - Consider the fall back - from high order CA to lower order CA - from CA to single carrier mode - Decided by network - UE architecture switching: - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA - FFS on whether it is decided by UE itself or decided by network - FFS on whether the additional indication from UE side is needed

UE fallback behaviours were discussed in several meetings, in last meeting, an agreement was reached to further discuss CA fallback and UE architecture switching separately.

- UE architecture switching

For single band operation, there are two CCs of intra-band DL NCCA in one Rx RF chain. When UE does the measurement and judges that it cannot support intra-band DL NCCA in one Rx RF chain, UE can decide that choose it is changed from 1RX to 2RX to keep CA configuration or choose it is fallback to single CC. Similarly, when UE does the measurement and judges that it can support intra-band DL NCCA in one Rx RF chain, UE can decide that choose it changes from 1RX to 2RX. However, UE architecture switching will cause additional interruption, which needs to be considered.

Proposal 3: UE architecture switching can be decided by UE. Interruption caused by UE architecture switching needs to be considered.

- CA fallback without UE architecture switching

For inter-band CA, we can assume two bands operation, there are two CCs of intra-band DL NCCA in band X and single DL CA in band Y. When CA fallback without UE architecture switching, which means that there is nX(2A) just can be changed into nX(A). CA fallback from higher order to low order.

Case 1: CA_nX(2A)_nY(A) --> CA_nX(A)_nY(A)

For single-band CA, two CCs fall back to single carrier mode.

Case 2: CA_nX(2A) --> nX(A)

- CA fallback with UE architecture switching

For inter-band CA, we can assume two bands operation, there are two CCs of intra-band DL NCCA in band X and single DL CA in band Y. When CA fallback with UE architecture switching, which means that nX(2A) can be kept.

Case 1: CA_nX(2A)_nY(A) --> CA_nX(2A)_nY(A)

For single-band CA, there is no involved to CA fallback. It will reduce the Rx number. Such as 4RX to 2RX.

Case 2: CA_nX(2A) --> nX(2A)

Conclusions

In this paper, we provide our views on methods for reducing the number of UE RX chains, we have made the following proposals.

Proposal 1: Support option 2. UE needs to measure the wanted PCC/SCC, in-gap interference, Tx leakage, image of LO and other intermodulation products to enable or disable the one Rx RF chain mode.

Proposal 2: Use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

Proposal 3: UE architecture switching can be decided by UE. Interruption caused by UE architecture switching needs to be considered.

- Reference

- R4-2505109, WF on fragmented DL carriers study, Mediatek, Apple, Qualcomm. TSG-RAN WG4 114bis meeting