3GPP TSG RAN WG4 Meeting #115	R4-2506547

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.23.2

Source:	vivo

Title:	Discussion on general aspects for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

Based on this SID, RAN4 had further general discussion on priorities from scenarios-level, use-cases level and sub-use -cases level in the past meetings. In this contribution, we provide our views on general aspects for AI/ML mobility based on the related progress captured in WF[2].

- Discussion

- 2.1 general aspects in AI/ML for mobility

RAN2 agreement in RAN2#129bis meeting Things to continue study for specification and RAN2 impact UE sided model; Network sided model. Measurement event prediction; Yes,; No (up to NW implementation). Temporal domain case A; Yes, all sub cases; Yes. Temporal domain case B; Yes , all sub cases; Yes. Frequency domain; Yes , all sub cases; Yes. Spatial domain; Pending; Pending. L3 beam level measurement prediction; Yes,; Yes.

At the latest RAN2 meeting, RAN2 listed the scenarios requiring analysis of spec impacts, with the conclusions as shown above. Both the UE-sided model and the NW-sided model are within the scope.

For the UE-sided model, in addition to the scenarios corresponding to the green-highlighted parts already under discussion in RAN4, according to the conclusions from the previous RAN2 meeting, the previously pending L3 beam-level scenario has also been confirmed as one of the study scenarios, and RAN4 needs to conduct a study on this scenario as well. We have proposed corresponding suggestions in the companion paper. As for the spatial domain, since it remains pending in RAN2, we can await further RAN2 conclusions to decide whether RAN4 should study it.

RAN4's previous discussions mainly focused on analyzing potential requirements predicted by the UE-sided model. For the spec impacts corresponding to the newly agreed NW-sided model in RAN2, RAN4 also needs to conduct further analysis.

Observation 1: Based on RAN2 latest agreement, for AI/ML for mobility, both the UE-sided model and the NW-sided model are within the scope

Proposal 1: In addition to UE sided model, RAN4 also needs to conduct further analysis on potential RRM impact for NW-sided model

RAN4 agreements in RAN4#112bis Issue 1-1-1: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

As for the priority of use cases, RAN4 had the initial working plan in RAN4#112bis meeting as above.

For the RLF prediction, according to the agreement concluded in RAN2#129 meeting as follows, it will not be studied in Rel-19 due to the limited time for SI discussion. As per this working plan, the part of RRM impact study will be excluded in RAN4 accordingly.

RAN2 Agreements on priority - RLF prediction will not be studied in Rel-19 - Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed). - Continue with generalization across cell configurations for RRM prediction - Continue with measurement events

Proposal 2: RAN4 not to study RRM impact on RLF prediction in Rel-19

Issue 1-1-2: Sub-use-case priorities - RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model). In RAN2, the following sub-use cases were agreed for RRM measurement prediction: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Proposals and Observations: - Proposal 1 (vivo, OPPO, Huawei, CMCC): RAN4 to consider all three sub-use cases with no prioritization. - Recommended WF: - RAN4 to consider all three sub-use cases with no prioritization.

First, according to the conclusions reached at the latest RAN2 meeting as mentioned in the first issue, for all agreed scenarios, the three sub-use cases will undergo further analysis of spec impact. From this perspective, RAN4 should not exclude any sub-use case.

Additionally, from a technical standpoint, regarding the prediction performance of the above three sub-use cases, based on simulation evaluations in RAN2, performance differences have been observed: In a specific scenario (with a shorter prediction window), Case 2 demonstrates higher prediction accuracy than the other two cases. Moreover, compared to the other two cases, the input and output of Case 2 both involve predictions at the same point (Point C), and the implementation process is relatively simpler. Therefore, concerning Case 2, it can be considered as one of the cases in the RAN4 study.

For L3 filtering, RAN2 discussed and agreed upon two L1 filtering options, namely sliding and non-sliding L1 filtering. These two methods remain under consideration for further evaluation.

Figure 1 Sliding L1/L3 filtering Figure 2 non-sliding L1/L3 filtering In sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every sample period. In non-sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every measurement period. In both L1/L3 filtering options, the filtered L1 measurement result is obtained based on the non-filtered L1 measurement results within one measurement period. And the filtered L3 measurement result is obtained as specified in section 5.5.3.2 of TS 38.331

In our understanding, these two implementations may differ in prediction performance. For the sliding L1 filtering, as it utilizes previously measured L3-RSRP values for calculation, This could be what makes the L1 prediction results smoothed. For the non-sliding L1 filtering, only the predicted L1-RSRP participated in the subsequent L3 filtering. Considering the implementation method has a certain impact on the performance of case 1, it is difficult to say whether case 1 or case 3 performs better for prediction scenarios with L1 results as input and L3 results as output. Therefore, we believe that neither case can be excluded or deprioritized before conducting the necessary simulation evaluation or a concrete conclusion on how to choose the implementation of these two cases.

Proposal 3: Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases

- Summary

Observation 1: Based on RAN2 latest agreement, for AI/ML for mobility, both the UE-sided model and the NW-

sided model are within the scope

Proposal 1: In addition to UE sided model, RAN4 also needs to conduct further analysis on potential RRM impact for NW-sided model

Proposal 2: RAN4 not to study RRM impact on RLF prediction in Rel-19

Proposal 3: Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases

- References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2504901, WF on RRM requirements for FS_NR_AIML_Mob, Nokia