3GPP TSG RAN WG4 Meeting #115	R4-2506551

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.31.5.1

Source:	vivo

Title:	Discussion on RRM requirements on (e)RedCap for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on Rel-19 NR NTN WI, which mainly included the objectives of downlink coverage enhancements, support of regenerative payload and Redcap NTN. The considerations from companies and the latest progress on (e)RedCap NTN were summarized in WF [1]. In this contribution, we would like to provide our further analysis on aspects on RRM requirements on (e)RedCap NTN.

- 2.	Discussion

  - General consideration on RRM requirements

#### Issue 1-1-1: SDT requirements - Background RAN4#114: #### Issue 1-1-2: SDT requirements Agreement: - For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is, - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Further discuss whether to send RAN2 LS. <Way Forward>: - Proposal 1: For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2. - The agreement reached in the last meeting only focus on RAN4’s requirements. - Proposal 2: RAN4 to send LS to RAN2 informing them about the RAN4 agreement on the removal of the RSRP change criterion for TA validation in CG-SDT for (e)RedCap UE in FR1-NTN and on the reuse of the TA validation criteria as specified for PUR in IOT-NTN.

Regarding the conclusion reached by RAN4 at the RAN4#114-bis meeting that "TA validation based on the RSRP change criterion for RedCap in TN shall be removed in the SDT requirement for (e)RedCap with NTN," whether this will impact the RAN2 spec primarily depends on how the RSRP change criterion is described in the existing RAN2 spec for TA Validation for CG-SDT (e.g., whether this criterion corresponds to cg-SDT-RSRP-ChangeThreshold as an optional or mandatory configuration).

Based on the existing TS 38.213 specifications and the IE design in TS 38.331, summarized as follows, from RAN2's procedural perspective, for the highlighted MAC-layer behavior described below, in addition to the condition that the "TAT timer is running," the UE is still required to:

- Store the current RSRP for TA validation (highlighted in gray).

- Furthermore, compare it with the reference RSRP value (highlighted in cyan).

TS38.321 ### 5.27.2 TA Validation for CG-SDT RRC configures the following parameters for TA validation for CG-SDT: - cg-SDT-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation. The MAC entity shall, upon the reception of CG-SDT configuration: 1> store the current RSRP of the downlink pathloss reference for TA validation as defined in TS 38.331 [5] clause 5.7.17. The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and - cg-SDT-TimeAlignmentTimer is running. TS38.331 <Relevant part 1> cg-SDT-TA-ValidationConfig Configuration for the RSRP based TA validation. If this field is not configured, then the UE does not perform RSRP based TA validation. <Relevant part 2> CG-SDT-TA-ValidationConfig-r17 ::= SEQUENCE { cg-SDT-RSRP-ChangeThreshold-r17 ENUMERATED { dB2, dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare5, spare4, spare3, spare2, spare1} <Relevant part 3> - cg-SDT-TA-ValidationConfig-r17 SetupRelease { CG-SDT-TA-ValidationConfig-r17 } OPTIONAL, -- Need M

Regarding the cyan-highlighted portion, after internal alignment with our RAN2 colleagues, our interpretation is that the RSRP change condition is an optional configuration. If the threshold is not configured, the UE would not be required to check the RSRP-based condition.

As for the gray-highlighted portion, our interpretation is that these two steps are merely preparatory steps for RSRP-based TA validation. Therefore, if RSRP-based TA validation is interpreted as an optional configuration, the purpose of performing these two steps becomes unclear.

Thus, given the ambiguity in the RAN2 specifications regarding TA validation based on the RSRP change criterion, and to ensure consistency between RAN4 and RAN2 in describing UE behavior at the specification level, we can also accept sending an LS to RAN2 to inform them of RAN4's conclusion on TA validation for the SDT requirement for (e)RedCap with NTN. RAN2 can then decide whether modifications to the RAN2 specifications are necessary and how to reflect this conclusion in the RAN2 specifications.

Proposal 1: Given the ambiguity in the RAN2 specifications regarding TA validation based on the RSRP change criterion, RAN4 to send an LS to RAN2 to inform them of RAN4's conclusion on TA validation for the SDT requirement for (e)RedCap with NTN

#### Issue 1-2-3: Other features Online agreement Agreement: - SMTC adjustment in idle/inactive is kept [mandatory] for (e)RedCap UEs with NTN. - From RAN4 perspective, location-based measurement report trigger is kept optional for (e)RedCap UEs with NTN.

According to the R17 design, SMTC adjustment in idle/inactive is an optional UE capability. The purpose of this issue is to make it mandatory for (e)RedCap UEs with NTN.

34-11; SMTC adjustment in idle/inactive; It's optional for UE to perform SMTC adjustment in RRC_IDLE/RRC_INACTIVE; n/a; n/a; n/a; n/a; Optional without capability signalling.

From technical perspective, since RedCap NTN UEs have weaker processing capabilities, and based on the existing conclusion that mandates support for two SMTCs, when the network (NW) alone controls the positions of the two SMTCs (i.e., the UE does not support autonomous SMTC adjustment), the UE's processing becomes less flexible if the two SMTCs are close to each other. In this case, supporting UE-autonomous adjustment of the SMTC offset offers greater flexibility.

From spec perspective, the existing RAN2 mechanism for UE-autonomous SMTC adjustment is reflected in the highlighted description of `smtc4list`, which means that if the UE reports support for autonomous adjustment, it may or may not adjust the SMTC offset based on the actual PDD. If this feature is to be enhanced to mandatory support, the corresponding RAN2 spec may also need to be modified. Since this UE feature was introduced by RAN2 in R17, one way to trigger RAN2 discussion is to send an LS notification. Alternatively, after RAN4 agrees to this enhancement, it could be included in the UE feature list.

smtc4list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

Proposal 2: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN

  - Others

#### Issue 4-1-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands? <Way Forward>: - Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 2: Introduce new capability to declare support over RedCap over NTN. - Proposal 3: RAN4 to not discuss or conclude on UE capability signaling and structure for RedCap NTN, unless specific requests are received from other working groups via LSs.

For flexibility considerations, we support Proposal 2. The drawback of Proposal 1 is that if a UE declares support for RedCap and support for NTN, it will only be treated as a RedCap-NTN-capable UE and must inherently meet the corresponding RRM requirements defined for RedCap-NTN, without allowing the UE to operate as a RedCap-TN UE or an NTN non-RedCap UE. This lacks sufficient flexibility.

On the other hand, R19 has also defined new priority rules for collision handling of overlapping DL and UL in HD-FDD for RedCap-NTN. According to P1, these rules would also need to be implicitly supported. However, for legacy R17/R18 UEs, it is possible to support both RedCap and NTN without necessarily following these corresponding rules.

Considering these two aspects, to avoid potential inflexibility in the future, choosing P2 is a more straightforward approach.

Proposal 3: On how to declare the capabilities for (e)RedCap UE with FR1-NTN bands, RAN4 to introduce new capability to declare support over RedCap over NTN

  - UE feature list

As we proposed in the previous issue, for the capabilities of (e)RedCap UE with FR1-NTN bands, RAN4 needs to introduce a new capability to declare support for RedCap over NTN. Below is the corresponding UE feature list:

- Summary

Proposal 1: Given the ambiguity in the RAN2 specifications regarding TA validation based on the RSRP change criterion,RAN4 to send an LS to RAN2 to inform them of RAN4's conclusion on TA validation for the SDT requirement for (e)RedCap with NTN

Proposal 2: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN

Proposal 3: On how to declare the capabilities for (e)RedCap UE with FR1-NTN bands, RAN4 to introduce new capability to declare support over RedCap over NTN

- References

- R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT