3GPP TSG RAN WG4 Meeting #115	R4-2506555

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.29.3.3

Source:	vivo

Title:	Further discussion on Conditional Intra-CU LTM

Document for:	Discussion

- Introduction

After the discussion of the past three meetings, the general aspect on defining delay requirements on CLTM is determined. In this paper, we will further provide our thinking on the details of the definition of delay requirements for supporting conditional Intra-CU LTM based on the latest WF [1].

- Discussion

<conditional LTM>

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.. < Way forward > FFS the following options: Conclude this issue in the next meeting. - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Option 3: Not define requirement for any of them.

For the UE-based TA mechanism, the UE calculates the TA value of the candidate cell by measuring the Reference Signal Time Difference (RSTD) between the current serving cell and the candidate cell (as shown by the black and green solid lines below) and combining it with the TA value of the current serving cell. However, it is important to note that this calculation is valid only when the difference between the starting points of the first symbol of the serving cell and the candidate cell is known (i.e., the offset value in the figure). In practice, we cannot accurately obtain this offset value or guarantee that the offset = 0, and this part of the error cannot be eliminated.

Potentially, the applicable scenarios for UE-based TA may mainly include: when the network supports TA calculation for the candidate cell through a PDCCH order and delivers it to the UE via a MAC CE. Since the TA has already been obtained through the PDCCH order, the RTD (also with some error) between the current serving cell and the neighboring cell can be calculated. After the TA obtained through the PDCCH order becomes outdated, UE-based TA could be considered as an alternative option for use.

Based on the above analysis, and considering that the core part only has two remaining meeting discussions, it is difficult to define relevant requirements for UE-based TA measurement or L1 measurement on deactivated Scell in this release. Therefore, we tend to favor Option 3: Not define requirement for any of them.

Proposal 1: RAN4 not to define requirement for L1 measurement on deactivated Scell and UE based TA measurement in Rel-19

Issue 1-2-3: The components of conditional LTM delay Issue 1-2-4: Tinterrupt < Agreement > - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19. < Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not. Issue 1-2-5: TRRC < Way forward > FFS the following proposals: - Proposal 1: TRRC is the RRC procedure delay.

Regarding TRRC, based on the conclusion from the last RAN2 meeting, the UE starts execution condition evaluation upon receiving the CLTM configuration, including the C-LTM execution condition. This behavior aligns with the existing CHO UE procedure, where the corresponding RRC processing time can reuse the current processing time defined for RRC reconfiguration messages containing CHO configuration.

Proposal 2: For CLTM, the definition and corresponding value of exsisting TRRC in CHO can be reused

For measurement time, CSI-RS based L1 measurement is introduced in Rel-19 LTM, and the detailed requirements are currently being discussed in RAN4 in parallel. From a feature completeness perspective, we prefer to define the corresponding requirements for CSI-RS based L1 measurement for CLTM in Rel-19, provided that the requirements on CSI-RS based L1 measurement can be fully defined.

Proposal 3: If CSI-RS based L1-RSRP measurement requirements is defined in Rel-19 LTM, it can be reused in Tmeasure

<subsequent conditional LTM>

Issue 1-3-1: General aspects of Subsequent Conditional LTM delay < Way forward > FFS the following options: - Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. - Proposal 1b: TEvent_DU needs to be updated. - Proposal 1c: Apart from the starting point of the delay. - Proposal 2: The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell. Issue 1-3-3: The components of subsequent conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. (Apple) - Tmeasure is measurements time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. - Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption + Tfirst-RS + TRS-proc, where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU. - Proposal 3: DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, where the TCLTM_interrupt = TCLTM-processing + TCLTM-IU.

From delay requirement point of view, compared with conditional LTM, the only difference for subsequent conditional LTM is the starting point. Apart from this, the delay requirement on conditional LTM can be reused to define the remaining components.

Proposal 4: For subsequent conditional LTM, Apart from the starting point of the delay, the delay requirement on conditional LTM can be reused to define the remaining components of subsequent CLTM.

- Summary

Proposal 1: RAN4 not to define requirement for L1 measurement on deactivated Scell and UE based TA measurement in Rel-19

Proposal 2: For CLTM, the definition and corresponding value of exsisting TRRC in CHO can be reused

Proposal 3: If CSI-RS based L1-RSRP measurement requirements is defined in Rel-19 LTM, it can be reused in Tmeasure

Proposal 4: For subsequent conditional LTM, Apart from the starting point of the delay, the delay requirement on conditional LTM can be reused to define the remaining components of subsequent CLTM.

- References

- R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom