3GPP TSG-RAN WG4 Meeting # 115                                R4-2506565

# Malta, MT, 19th – 23rd May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on impacts for measurement event prediction

Agenda Item:	7.23.3.2

Document for:	Approval

- Introduction

In RAN plenary #105 meeting, a revised SID[1] for Rel-19 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this SID are listed as follows.

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this document, we will provide some views on requirements impacts based on RAN2 assumptions of AI/ML for mobility from the RAN4 perspective.

- Discussion

In previous meeting, we discussed the priorities of different use cases and have reached consensus that RAN4 to start the discussion with RRM measurement prediction use-case first. For measurement event prediction use-case and RLF use-case, more progress is needed from RAN2, and RAN4 to only start the discussion of measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. In this paper, we will provide our views on measurement event prediction use case.

As we known, RAN2 has discussed measurement event prediction use-case many times and reached some agreements. To our knowledge, RAN2 focused on discussing simulation assumptions and results based on measurement event prediction in previous meetings and the progress of RAN2 is replicated as follows.

RAN2#127bis Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference. 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

In RAN2, indirect measurement event prediction and direct measurement event prediction are both within the scope of their research. Indirect measurement event prediction means that intermediate results are needed. That is to say, the prediction of RRM measurement is first performed to obtain the predicted RSRP, and then based on these prediction results, the calculation results of measurement events are further obtained. Correspondingly, direct measurement event prediction does not require to obtain the prediction results of RSRP.

Based on RAN2’s progress, we perfer RAN4 to discuss both indirect measurement event and direct measurement event prediction. Meanwhile, RAN2 has an agreement that the simulation assumption of RRM measurement prediction can be reused unless otherwise specified, so we can apply the analysis results of the simulation parameters for RRM measurement prediction to measurement event prediction. In the discussion on RRM measurement prediction, we have considered the measurement error in high level that RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy, while RAN4 has not agreed yet on the exact error model(s) in the SI phase. On the basic, measurement error should be considered into measurement event perdiction when we discuss the prediction accuracy.

Proposal 1: Measurement error should be considered into measurement event prediction, which will impact the prediction accuracy.

- Indirect prediction: Case A Most of the results show that the F1 score for prediction of measurement event is very good. For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate. - Indirect prediction: Case B Measurement event case B can have very good F1 score depending on filtering approach/PW F1 score is good for the PW window for case B (i.e. low PW) F1 score decreases with an increasing MRRT value We will focus on finding cases that bring benefits to the system rather than trying to compare With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism. - Other Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required. - Direct prediction F1 score for direct measurement is very good based on the current simulation results.

Based on the current discussion of RAN2, we suggest that similar to RRM measurement prediction and discuss the impact of temproal domain and frequency domain first. As for spatial domain, if time permits or work needs, we can further discuss the impacts.

Proposal 2: For measurement event prediction, RAN4 should at least consider scenarios 2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A).

For indirect event measurement prediction, measurement result(s) is predicted by a RRM measurement prediction model first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Therefore, the RRM impact on RSRP difference prediction accuracy in RRM measurement prediction use case can be directly reused for indirect event measurement prediction. In the last meeting, we have discussed the performance metric for event prediction use case and reached consensus blew [2].

Issue 3-1-2: Performance metric for event prediction use case Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

As to the issue whether need to define requirement for the predicted RSRPs or not if the report includes the predicted RSRPs, we prefer to define requirement for reported results and need to ensure that all reported results meet the relevant requirements. In our understanding, the requirements for measurement prediction can be used as baseline for intermediate output.

Proposal 3: For indirect case, if the report includes the predicted RSRPs, RAN4 should define requirements for the predicted RSRPs and the requirements for measurement prediction can be used as baseline.

For the metric of event prediction, RAN2 has consiered the timing fidderence between predicated event and ground truth event, i.e., EDT, which is not considered in legacy. To ensure the accuracy of the measurement, RAN4 should consider whether and how to define prediction accuracy for event prediction use case based on RAN2’s progress.

Proposal 4: RAN4 should consider whether and how to define prediction accuracy for event prediction use case based on RAN2’s progress.

- Conclusion

In this contribution, we have the following proposals for the AI/ML mobility on measurement event prediction:

Proposal 1: Measurement error should be considered into measurement event prediction, which will impact the prediction accuracy.

Proposal 2: For measurement event prediction, RAN4 should at least consider scenarios 2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A).

Proposal 3: For indirect case, if the report includes the predicted RSRPs, RAN4 should define requirements for the predicted RSRPs and the requirements for measurement prediction can be used as baseline.

Proposal 4: RAN4 should consider whether and how to define prediction accuracy for event prediction use case based on RAN2’s progress.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR , OPPO (rapporteur)

- R4-2504901, WF on RRM requirements for FS_NR_AIML_Mob, Nokia