3GPP TSG-RAN WG4 Meeting # 115                                    R4-2506566

Malta, MT, 19th – 23rd May, 2025

Agenda item:	5.19.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Further NR mobility enhancements

Document for:	Approval

Introduction

In this contribution, we discuss the remaining open issues for mobility core part requirements based on the RAN4#114 way forward [1].

Discussion

In this contribution, we will provide our views on some issues based on the WF which agreed in RAN4#114 meeting.

- Sub-topic 1-2 RAN1 LS on collision between SSB and RA occasion for LTM Issue 1-2-1: whether/how to handle the collision between an SSB and a valid RO for LTM Agreement on reply LS - RACH shall be prioritized over SSB, if the collision is outside MG. - If collision happens within MG, it is up to UE implementation, i.e., UE may transmit or drop RACH when the RACH occasion colliding with the MG occasion. < Agreement > - Further discuss the definition of RACH occasion colliding with the MG occasion.

For the first issue, we have discussed how to handle the collision between an SSB and a valid RO. As a result, this issue is divided into two parts that collision is outside MG and collision with the MG occasion. For the first case, RACH occsion has higer priority and SSB measurement should be dropped. For the second case, it’s up to UE how to handle the cillision, transmit or drop RACH. The issue we should discuss in this meeting is how to define the collision between RACH occasion and MG occasion.

First, if RACH occasion and MG occasion overlap in the time domain, including partially and fully overlapping, we can understand it as a collision between RACH occsion and MG occasion at this time. In addition, if RO arrives before MG and the time difference between the two is not sufficient to complete MG rescheduling or cancellation, MG still exists in this situation, which may also be considered as a collision scenario. When we discussed the value of TSSB if the condition of TSSB equal to zero is not met in PDCCH ordered RACH delay requirements, the case that SSB outside actibe DL BWP is considered and margin (2ms) is added for MG resheduling.

Proposal 1: RACH occasion and MG occasion are considered colliding if the two occasions are fully or partially overlapping in time domain.

Issue 1-3-1: PDCCH-order PRACH delay Table 8.6.2-1: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. 5; 0.03125; 20; 65. 6; 0.015625; 39; 129. NOTE 1: Depends on UE capability. NOTE 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. < Way forward > Further discuss the following proposals - Proposal 1 (Ericsson): - If PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell, for PRACH transmission using 1.25kHz or 5 kHz SCS, UE determines BWP switch delay assuming µ=0. - Proposal 2 (QC): - Update the value of T_SSB-proc defined for SSB processing in PDCCH-order PRACH delay requirement to 7 ms from 2 ms

This issue is about requirements on PDCCH order RACH delay. We can disciss one by one.

For proposal 1, proponent mentioned that RACH transmission can use 1.25kHz or 5kHz in LTM but BWP switch delay do not refer to 1.25KHz or 5KHz. As mentioned in TS 38.213, ∆RF/BB preparation=TBWPSwitchDelay refers to TS 38.133 and we should make it clear which one should be considered. From TS 38.213, we can observed that RAN1 defined the value of N2 assuming SCS configuration μ=0 for a PRACH transmission using 1.25 kHz or 5 kHz SCS. Therefore, the same method can be used for TBWPswitchDelay.

TS 38.213 clause 8.1 If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+∆BWPSwitching+∆Delay+Tswitch+TSSB+∆RF/BB preparation msec, where - NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission - ∆BWPSwitching=0 if the active UL BWP does not change, or if a cell indicator field in the PDCCH order indicates a non-serving cell [5, TS 38.212], and ∆BWPSwitching is a time duration of TBWPswitchDelay defined in [10, TS 38.133] otherwise - ∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2 - Tswitch is a switching gap duration as defined in [6, TS 38.214] - TSSB=0 if a cell indicator field in the PDCCH order indicates a serving cell or if cell indicator field is not present, and TSSB is defined in [10, TS 38.133] otherwise - ∆RF/BB preparation=0 if a cell indicator field in the PDCCH order indicates a serving cell or if cell indicator field is not present, and ∆RF/BB preparation is defined in [10, TS 38.133] otherwise For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines N2 assuming SCS configuration μ=0. TS 38.133 clause 6.2.2C.2 - ∆RF/BB preparation is the time for RF and baseband preparation. The following apply: - If PRACH bandwidth is outside active UL BWP of serving cell(s) but within one of configured UL BWPs of an active serving cell, the ∆RF/BB preparation is the DCI based BWP switching delay specified in clause 8.6. - If PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell, ∆RF/BB preparation equals to capability value reported in pdcch-RACH-PrepTimeList-r18. - Otherwise, ∆RF/BB preparation = 0. TS 38.133 clause 8.6.2 Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in table 8.6.2-1. Table 8.6.2-1: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. 5; 0.03125; 20; 65. 6; 0.015625; 39; 129. NOTE 1: Depends on UE capability. NOTE 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch..

Proposal 2: If PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell, for PRACH transmission using 1.25kHz or 5 kHz SCS, UE determines BWP switch delay assuming µ=0.

For proposal 2, proponent proposed to update the value of TSSB-proc defined for SSB processing in PDCCH-order PRACH delay requirement from 2 ms to 7 ms due to the fine timing of the SSB may not always be estimated immediately after the reception of the SSB indicated in the PDCCH order. We have no strong view on this issue and we slightly prefer to reuse the legacy value which is used widely in RAN4 requirements.

Conclusions

- In this contribution, we put forward the following proposals and observations on L1/L2 triggered mobility.

- Proposal 1: RACH occasion and MG occasion are considered colliding if the two occasions are fully or partially overlapping in time domain.

Proposal 2: If PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell, for PRACH transmission using 1.25kHz or 5 kHz SCS, UE determines BWP switch delay assuming µ=0.

References

- R4-2502584, WF on RRM requirements for NR_Mob_enh2, MediaTek