# 3GPP TSG-RAN WG4 Meeting #115	R4-2506576

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on interruption requirements for NonCol_intraB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some descriptions overlap with existing content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete duplicate descriptions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description is duplicated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.3, 8.2.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.2.1.2.3	Interruptions at SCell addition/release

The requirements in this clause shall apply for the UE configured with PSCell.

When one E-UTRA SCell in MCG is added or released:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to X1 slot, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released and UE not supporting requirementTypeIndication-r18 indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and is not provided with nonCollocatedTypeMRDC-r18, or

-	of up to X1 slot, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released and UE not supporting [MRDCrequirementTypeIndication -r19 ] indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting [MRDC NW signalling-r19 ] is capable of [interBandMRDC-WithOverlapDL-Bands-r16] and [MRDC UE capability signalling-r19] on this band pair and is not provided with [nonCollocatedTypeMRDC-r19], or

-	of up to max{Y1 slot + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being added or released, and UE does not indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE capable of requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and provided with nonCollocatedTypeMRDC-r18, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG;

-	of up to max{Y1 slot + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being added or released, and UE does not indicates it is capable of [MRDC UE capability signalling-r19] on this band pair or UE capable of  [MRDCrequirementTypeIndication -r19 ] indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 and  [MRDC UE capability signalling-r19] on this band pair and provided with [nonCollocatedTypeMRDC-r19], provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG;

Where X1 and Y1 are specified in Table 8.2.1.2.3-1.

When one SCell in SCG is added or released:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X1 slot, if the active serving cell and the SCell being added or released are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair

-	of up to X1 slot, if the active serving cell is non-contiguous to the SCell being added or released in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

or

-	of up to Y1 slot + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being added or released in the same FR1 band, or if the active serving cells are in the same FR1 band as any of the SCells being added or released and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

-	of up to Y1 slot + TSMTC_duration if the active serving cells are in the same FR2 band as any of the SCells being added or released, provided the cell specific reference signals from the active serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being added when one SCell is added. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being added, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being added is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being added. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being added, TSMTC duration for the SCell being added is 0 ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is released.

Where X1 and Y1 are specified in Table 8.2.1.2.3-2.

Table 8.2.1.2.3-1: Interruption length X1 and Y1 at E-UTRA SCell addition/Release

|  | NR Slot length | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 2 | 3 | 2 | 3 |
| 2 | 0.25 | 5 |  | 4 | 5 |
| 3 | 0.125 | 9 |  | N/A | - N/A |

Table 8.2.1.2.3-2: Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) |  | Interruption length Y1 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 2 |  | 2 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 4 | 4 |
|  |  | Either aggressor cell or victim cell is on FR1 | 5 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 8 | 8 |
|  |  | Aggressor cell is on FR1 | 9 |  |

8.2.1.2.4	Interruptions at SCell activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and one SCell.

When one E-UTRA SCell in MCG is activated from deactivated or dormant state, or deactivated from activated or dormant state:

-	the UE is allowed an interruption on any active serving cell in SCG:

-	of up to X2 slot, if the active serving cell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to X2 slot, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being activated or deactivated and UE not supporting requirementTypeIndication-r18 indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and not provided with nonCollocatedTypeMRDC-r18, or

-	of up to X2 slot, if the active serving cell is in the band overlapping or partially overlapping with the E-UTRA SCell being added or released and UE not supporting [MRDCrequirementTypeIndication -r19 ] indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting [MRDC NW signalling-r19 ] is capable of [interBandMRDC-WithOverlapDL-Bands-r16] and [MRDC UE capability signalling-r19] on this band pair and is not provided with [nonCollocatedTypeMRDC-r19], or

-	of up to max{Y2 slot + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being activated or deactivated, and UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair or UE supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 on this band pair and is provided with nonCollocatedTypeMRDC-r18, provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being activated or deactivated are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG.

-	of up to max{Y2 slot + TSMTC_duration, 5ms}, if the active serving cells are in the same band or in the band overlapping or partially overlapping with the E-UTRA SCells being activated or deactivated, and UE does not indicates it is capable of [MRDC UE capability signalling-r19] on this band pair or UE capable of  [MRDCrequirementTypeIndication -r19 ] indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 and  [MRDC UE capability signalling-r19] on this band pair and provided with [nonCollocatedTypeMRDC-r19], provided the cell specific reference signals from the active serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above active serving cells in SCG;

Where X2 and Y2 are specified in Table 8.2.1.2.4-1.

When one SCell in SCG is activated or deactivated:

-	an interruption on any active serving cell in SCG:

-	of up to X2 slot, if the active serving cell and the SCell being activated or deactivated are in a FR1 band pair or in a FR1+FR2 band pair.

-	of up to X2 slot, if the active serving cells and the SCells being activated or deactivated are in a FR2 band pair and UE is capable of independent beam management on this FR2 band pair.

-	of up to X1 slot, if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

-	of up to X1 slot, if the active serving cell is non-contiguous to the SCell being activated or deactivated in the same FR1 band and UE is capable of [intraBandNR-CA-non-collocated-r19] and [nonCollocatedTypeNR-CA-r19] is not provided.

or

-	of up to Y1 slot + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being activated or deactivated in the same FR1 band, or if the active serving cells are in the same FR1 band as any of the SCells being activated or deactivated and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, or

-	of up to Y1 slot + TSMTC_duration, if the active serving cells are contiguous to any of the SCells being activated or deactivated in the same FR1 band, or if the active serving cells are in the same FR1 band as any of the SCells being activated or deactivated and UE is not capable of [intraBandNR-CA-non-collocated-r19] or UE is capable of [intraBandNR-CA-non-collocated-r19] and [intraBandNR-CA-non-collocated-r19] is provided,

provided the cell specific reference signals from the active serving cells and the SCells being activated ordeactivated are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

-	of up to Y2 slot + TSMTC_duration if the active serving cells are in the same FR2 band as any of the SCells being activated or deactivated, provided the cell specific reference signals from the active serving cells and the SCells being activated or deactivated are available in the same slot, where, TSMTC_duration is

-	the longest SMTC duration among all above active serving cells in SCG and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms;

-	the longest SMTC duration among all above active serving cells in SCG when one SCell is deactivated.

Where X2 and Y2 are specified in Table 8.2.1.2.4-2.

Table 8.2.1.2.4-1: Interruption length X2 and Y2 at E-UTRA SCell activation/deactivation

|  | NR Slot length | nterruption length X2 (slots) |  | Interruption length Y2 (slots) |  |
| --- | --- | --- | --- | --- | --- |
|  | (ms) | Sync | Async | Sync | Async |
| 0 | 1 | 1 | 2 | 1 | 2 |
| 1 | 0.5 | 1 | 2 | 1 | 2 |
| 2 | 0.25 | 3 |  | 2 | 3 |
| 3 | 0.125 | 5 |  | N/A | N/A |

Table 8.2.1.2.4-2: Interruption length X2 and Y2 at SCell activation/deactivation

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 |  | 1 |
| 1 | 0.5 | 1 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |  |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |  |

<End of Change 1>