# 3GPP TSG-RAN WG4 Meeting #115                              R4-2506587

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.24.4.2

Source: 	CMCC

Title: 			 Discussion on other RRM requirements for NR MIMO Phase 5

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR MIMO Phase 5 was approved [1]. In RAN#105 meeting, the revised WID are approved [2], the objectives are duplicated as following.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last meeting, there was discussion on NR MIMO Phase 5, and a WF was agreed [3]. This contribution provides discussion on RRM requirements on enhancements for asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

## 2.1 Enhancements for asymmetric DL sTRP/UL mTRP scenarios

According to RAN1 agreements [8], if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s). It can be seen that if PL offset is not configured, UL TRP can transmit SSB.

From deployment point of view, UL TRP(s) transmit SSB is one of our typical deployment in HetNet scenario.

ICBM is in use since normal TRP instead of UL-only TRP are deloyed in our HetNet deployment. As shown in Fig.1, indoor pico site and outdoor macro site are deployed at same frequency with different cell ID (Inter-cell). Both indoor pico site and outdoor macro site transmit SSBs with different cell ID. UE transmit PUCCH/PUSCH/SRS to the TRP with additional PCI, and UE receive PDCCH/PDSCH only from normal TRP. The reason to have this deployment is that outdoor macro site has severe interference to indoor pico site, then outdoor macro site can be used for DL transmission, while indoor pico site has better coverage for UL, then indoor pico site can be used for UL reception.

Fig.1 Inter-cell deployment with asymmetric DL and UL transmission

Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2.

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

According to the discussion in RAN4 #114 meeting, RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.”are currently only applied for Scenario #1. Companies are not clear on whether this RAN1 agreements are applied for Scenario #2. RAN4 sent LS to RAN1 for clarification [5].

According to RAN1 reply LS [6], it can be seen that the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s).

Draft Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP (R1-2503091) 1. Overall Description: RAN1 would like to thank RAN4’s LS on DL reference timing to support two TA enhancement for asymmetric DL sTRP/UL mTRP. With regard to the RAN4’s question, RAN1 provides the corresponding answer as below: Question: Based on RAN1 LS (R1-2410870), RAN4 discussed timing requirement for two TA enhancement for asymmetric DL sTRP/ UL mTRP. RAN4 would like to check with RAN1 whether RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). And RAN1 made the following new agreement for further clarification: Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI. 2. Actions: To RAN4 ACTION: RAN1 respectfully asks RAN4 to take the above replied information into consideration for their future work.

For scenario#2, when the UE is configured with rel-19 2 TAs, RAN1 agreed that the UE maintains two downlink reference timings. If “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. If “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG. The agreements applies for the case UE is configured with SSB-MTC-additionalPCI. It can be seen that RAN1 design is same as that for Rel-18 m-DCI two TA, legacy timing requirements (Rel-18 m-DCI two TA) can be reused.

Proposal 1: for 2TA enhancement, for senario #2, it is proposed to follow RAN1 agreements that two downlink reference timings are maintained. And the reference point for PRACH transmission is

- if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

- if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

Issue 2-6: Whether to define UL TCI state activation requirement for Scenario#2? Way forward: discuss the following options in the next meeting for Scenario#2 - Option 1: No RRM impacts on TCI state switching requirement. - Option 2: Legacy SSB based UL TCI state activation requirement can be re-used.

For scenario 2 (UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s)), the solution to derive pathloss for UL TRP baed on the pathloss from RSs from normal TRP and pathloss offset is not applicable. While for this case, since SSB is transmitted from UL TRP (i.e. TRP with additional PCI), the SSB can be used as pathloss RS when UE transmits PUCCH and PUSCH to UL TRP. RAN1 also agreed that SSB from UL-TRP can be used for PL RS for PRACH. Taking above into account, legacy requirements can be reused.

Agreement in RAN1#120 meeting For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH: - The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH; - The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH: - In this case, the PRACH configuration associated with addition PCI is used. - This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured.

Proposal 2: for scenario 2, it is proposed that legacy UL TCI state switching delay requirements can be reused.

- Conclusion

This contribution provides discussion on RRM requirements on asymmetric DL sTRP/UL mTRP scenarios. The proposals are:

Proposal 1: for 2TA enhancement, for senario #2, it is proposed to follow RAN1 agreements that two downlink reference timings are maintained. And the reference point for PRACH transmission is

- if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

- if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

Proposal 2: for scenario 2, it is proposed that legacy UL TCI state switching delay requirements can be reused.

- Reference

- RP-234007, New WID: NR MIMO Phase 5

- RP-242394, WID revision: NR MIMO Phase 5

- R4-2504903, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2502596, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP

- R1-2503091, Draft Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP

- R4-2420108, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R1-2410870, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement

- R4-2416921, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung