# 3GPP TSG-RAN WG4 Meeting #115                              R4-2506590

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.29.3.2

Source: 	CMCC

Title: 			 Discussion on CSI-RS based L1 measurement

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #105 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meetin, there was discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on CSI-RS based L1 measurement.

- Discussion

- Issue 4-4: CSI-RS based intra-frequency/inter-frequency measurement definition Candidate options: - Option 1: Apple, CMCC A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the center frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: MediaTek, ZTE, Ericsson, Nokia, Qualcomm A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP. - Option 3: Huawei If L1 CSI-RS RSRP measurement with gap is not supported, either option of intra-frequency CSI-RS based L1 RSRP measurement definition has no big difference. If CSI-RS measurement with gap is supported, to define measurement requirements, the “CSI-RS frequency layer” concept will be touched. Then the intra-frequency definition based on center frequency is more preferred. - Option 4: vivo - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resources of both the neighbour cell and serving cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource are confined within the active DL BWP.

For L3 measurement, a measurement is defined as a CSI-RS based intra-frequency L3 measurement provided that SCS/CP type/centre frequency of the CSI-RS resource of the neighbour cell configured for measurement are same as that of the serving cell indicated for measurement. For the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, our initial consideration is to follow the definition of CSI-RS based L3 measurement. Based on this consideration, it is proposed to consider center frequency.

From RAN4 perspective, the definition of intra-frequency and inter-frequency has impact on RRM requirements. The delay/accuracy requirements are different for intra-freqeucny and inter-frequency. However, in last meeting, it was agreed that from RAN4 perspective, not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. The above agreements mean that RAN4 requirements only apply to measurement without gap, which imply the CSI-RS configured for measurement is within the active BWP. In last meeting, it was agreed the definition of inside active BWP, which is at least 48 RBs of the CSI-RS configured for measurement is within the active BWP. And in last meeting, it was agreed no to introduce definition of frequency layer. With current situation, we do not see the necessity to further discuss CSI-RS based intra-frequency/inter-frequency measurement definition.

Proposal 1: considering that RAN4 requirements only apply to measurement without gap, no need to introduce definition of CSI-RS based intra-frequency/inter-frequency measurement.

- Conclusion

This contribution provides discussion on CSI-RS based L1 measurement. The proposals are:

Proposal 1: considering that RAN4 requirements only apply to measurement without gap, no need to introduce definition of CSI-RS based L1 intra-frequency/inter-frequency measurement.

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R1-2409283, LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

- R3-247911, Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4