# 3GPP TSG-RAN WG4 Meeting #115                               R4-2506591

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.29.3.3

Source: 	CMCC

Title: 			 Discussion on conditional intra-CU LTM

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #107 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meeting, there was discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on conditional intra-CU LTM.

- Discussion

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.. < Way forward > FFS the following options: Conclude this issue in the next meeting. - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Option 3: Not define requirement for any of them.

According to the discussion in previous meeting, whether CLTM requirements apply to candidate cells on deactivated SCC is related with the discussion on Rel-18 LTM. In Rel-18 LTM, LTM candidate cells on deactivated SCC is supported in RAN2, but RAN4 related requirements, e.g. interruption, are not specified due to limited timeline. It is necessary to define requirements to fix this mismatch between RAN2 and RAN4. And after the LTM requirements are applied to candidate cells on deactivated SCC, the Rel-19 CLTM requirements can also be defined.

Proposal 1: For the case which CLTM is triggered by L1 measurement, it is proposed to define measurement requirements for L1 measurement on deactivated SCC and the cells with same frequency as deactivated SCell.

According to RAN2 agreements, UE based TA measurement mechanism is supported for conditional intra-CU LTM. Autonomous TA adjustment for LTM is introduced in Rel-18 LTM. However, no dedicated requirements for UE based TA adjustment are specified in RAN4 in R18 due to limited timeline. In order to guarantee the performance, it is necessary to define related RAN4 requirements, e.g.timing requirements, for UE based TA adjustment.

Proposal 2: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements for UE based TA adjustment.

Issue 1-2-3: The components of conditional LTM delay Issue 1-2-4: Tinterrupt < Agreement > - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19. < Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not. Issue 1-2-5: TRRC < Way forward > FFS the following proposals: - Proposal 1: TRRC is the RRC procedure delay. Issue 1-2-6: TEvent_DU - The issue is merged into issue 1-2-3. Issue 1-2-7: Tmeasure - The issue is merged into issue 1-2-3. Issue 1-2-8: TCLTM_execution - The issue is merged into issue 1-2-3. Issue 1-2-9: TLTM-RRC-processing - The issue is merged into issue 1-2-3. Issue 1-2-10: TLTM-processing < Way forward > FFS the following proposal: - Proposal 1: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Proposal 1a: TLTM-processing can be up to 20ms. Issue 1-2-11: Tfirst-RS / TRS Issue 1-2-12: TRS-proc - The issues are merged into issue 1-2-3. Issue 1-2-13: TLTM-IU < Way forward > FFS the following proposal: - Proposal 1: TLTM-IU is the interruption uncertainty during LTM cell switch. Issue 1-2-14: Twait_RS < Way forward > FFS the following proposal: - Proposal 1: Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. In Rel-18 LTM, both are included in Tinterrupt. As for Rel-19 CLTM, some companies propose not to be included. On one hand, we understand the motivation to minimize the interruption since network does not know when the condition is met for C-LTM. On the other hand, it is up to UE implementation whether these procedure or part of procedures (e.g. waiting time of Tfirst-RS is not included in T_interrupt and RS tracking time of Tfirst-RS is included in T_interrupt) will cause interruption.

According to Rel-18 LTM, the value of Tfirst-RS and TRS-proc can be zero under certain conditions, which are duplicated as following. In our view, we can follow similar way as that for Rel-18 LTM, Tfirst-RS andTRS-proc are included in Tinterrupt, and the value is zero under certain conditions. As for the detailed condition, the conditions under which Tfirst-RS =0 and TRS-proc = 0 for Rel-18 LTM can be reused for CLTM. As for whether additional conditions is needed can be further consdiered.

TS 38.133, Interruption time for LTM (6.3.1.3) Tfirst-RS = 0 and TRS-proc= 0 under the following conditions: - The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or - The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.25.3 and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or - the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or - The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8. 25.3, and not more than TCI state activation delay stated in section 8. 25.3 + [480 ms], or [NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.] - The time between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms]. Otherwise, Tfirst-RS is the time to the first SSB transmission on the target cell after Tcmd + TLTM-RRC-processing + TLTM-processing. TRS-proc = 2 ms.

Proposal 3: it is proposed that Tfirst-RS and TRS-proc are included in Tinterrupt. And the conditions under which Tfirst-RS =0 and TRS-proc = 0 for Rel-18 LTM can be reused for CLTM. FFS whether additional conditions is needed.

Rel-18 LTM cell switch delay DLTM is specified as the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.  DLTM = Tcmd + TLTM-interrupt. Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement. TLTM-interrupt is interruption time, and TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms. In detail, TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. TLTM-IU is the interruption uncertainty during LTM cell switch which is related with whether RACH-based LTM cell switch or RACH-less LTM cell switch is in use.

For CLTM, TLTM-processing and TLTM-IU are needed. Similar as Rel-18 LTM, TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. TLTM-IU is the interruption uncertainty during CLTM cell switch.

Proposal 4: for CLTM, TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

Proposal 5: for CLTM, TLTM-IU is the interruption uncertainty during CLTM cell switch.

- Based on discussion, the overall cell switch delay for CLTM is proposed as DCLTM = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution or TLTM-RRC-processing + TLTM-interrupt , and TLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. And the value of Tfirst-RS and TRS-proc can be zero under certain conditions.

Proposal 6: the cell switch delay for CLTM is proposed as

DCLTM = TRRC_delay + TEvent_DU + Tmeasure + TLTM-RRC-processing + TLTM-interrupt ,

TLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU.

- TRRC_delay is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell.

- TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

- TLTM-interrupt is the interruption time

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. The value of Tfirst-RS is zero under certain condition.

- TRS-proc is the time for SSB processing. The value of TRS-proc is zero under certain condition.

- TLTM-IU is the interruption uncertainty during CLTM cell switch

In last meeting, it was agreed that the starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. No need to consdier TRRC in subsequent conditional LTM delay. The cell switch delay for subsequent conditionl LTM is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM-RRC-processing +TLTM-interrupt. TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. Tmeasure is measurements time. TLTM-interrupt is the interruption time as specified in Rel-18, and TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms. In detail, TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. TLTM-IU is the interruption uncertainty during LTM cell switch which is related with whether RACH-based LTM cell switch or RACH-less LTM cell switch is in use.

- Proposal 7: for subsequent conditional LTM, cell switch delay is proposed as:

- DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM-RRC-processing + TLTM-interrupt. ,

- TLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ,

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurement time.

- TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

- TLTM-interrupt is the interruption time

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. The value of Tfirst-RS is zero under certain condition.

- TRS-proc is the time for SSB processing. The value of TRS-proc is zero under certain condition.

- TLTM-IU is the interruption uncertainty during cell switch

- Conclusion

This contribution provides discussion on conditional LTM. The proposals are:

Proposal 1: For the case which CLTM is triggered by L1 measurement, it is proposed to define measurement requirements for L1 measurement on deactivated SCC and the cells with same frequency as deactivated SCell.

Proposal 2: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements for UE based TA adjustment.

Proposal 3: it is proposed that Tfirst-RS and TRS-proc are included in Tinterrupt. And the conditions under which Tfirst-RS =0 and TRS-proc = 0 for Rel-18 LTM can be reused for CLTM. FFS whether additional conditions is needed.

Proposal 4: for CLTM, TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

Proposal 5: for CLTM, TLTM-IU is the interruption uncertainty during CLTM cell switch.

Proposal 6: the cell switch delay for CLTM is proposed as

DCLTM = TRRC_delay + TEvent_DU + Tmeasure + TLTM-RRC-processing + TLTM-interrupt ,

TLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU.

- TRRC_delay is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell.

- TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

- TLTM-interrupt is the interruption time

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. The value of Tfirst-RS is zero under certain condition.

- TRS-proc is the time for SSB processing. The value of TRS-proc is zero under certain condition.

- TLTM-IU is the interruption uncertainty during CLTM cell switch

- Proposal 7: for subsequent conditional LTM, cell switch delay is proposed as:

- DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM-RRC-processing + TLTM-interrupt. ,

- TLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ,

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurement time.

- TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

- TLTM-interrupt is the interruption time

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. The value of Tfirst-RS is zero under certain condition.

- TRS-proc is the time for SSB processing. The value of TRS-proc is zero under certain condition.

- TLTM-IU is the interruption uncertainty during cell switch

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2420102, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4