# 3GPP TSG-RAN WG4 Meeting #115                               R4-2506594

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.25.4.2

Source: 	CMCC

Title: 			 Discussion on RRM impacts for SBFD operation

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on SBFD, and a WF was agreed [2]. This contribution provides discussion on RRM impacts for SBFD operation.

- Discussion

- Issue 2-1-1a: CSI-RS L1 measurement – collision with UL - Agreements (adhoc session on Thursday) - Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement. - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission following the R16 NR-U approach. - Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Background: - Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - RAN1 agreements related to CSI processing time Agreement [RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.. - Agreements (adhoc session on Thursday with additions) - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE.

For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, according to RAN1 agreements, there are two UE capabilities. UE capability 1 is CSI processing timeline and CPU counting are the same as legacy. UE capability 2 is to scale the CSI processing timeline Z/Z’by factor of 2 and keep the same CPU counting as legacy. As dicsussed in [3], the existing legacy CSI computation time can be up to 4ms. For UE support capability 2, the CSI processing time is up to 8 ms.

Following is the legacy CSI-RS based L1-RSRP measurement period. TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. According to TS38.331, it is possible that the value of TCSI-RS is smaller than 8 ms. TReport is configured periodicity for reporting. According to TS38.331, it is possible that the value of Treport is smaller than 8 ms. Taking above into account, it is reasonable to add a lower bound of 8ms on top of existing delay requirements, e.g. max (8ms, TReport, ceil(M*P)*TCSI-RS) for non-DRX case.

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: K = 1 when TCSI-RS ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 or measurementEnhancementCA-r17. |  |

From TS 38.331 Clause 6.3.2: CSI-ResourcePeriodicityAndOffset ::= CHOICE { slots4 INTEGER (0..3), slots5 INTEGER (0..4), slots8 INTEGER (0..7), slots10 INTEGER (0..9), slots16 INTEGER (0..15), slots20 INTEGER (0..19), slots32 INTEGER (0..31), slots40 INTEGER (0..39), slots64 INTEGER (0..63), slots80 INTEGER (0..79), slots160 INTEGER (0..159), slots320 INTEGER (0..319), slots640 INTEGER (0..639) } CSI-ReportPeriodicityAndOffset ::= CHOICE { slots4 INTEGER(0..3), slots5 INTEGER(0..4), slots8 INTEGER(0..7), slots10 INTEGER(0..9), slots16 INTEGER(0..15), slots20 INTEGER(0..19), slots40 INTEGER(0..39), slots80 INTEGER(0..79), slots160 INTEGER(0..159), slots320 INTEGER(0..319) } -- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP -- ASN1STOP

Proposal 1: for UE indicating that CSI processing time is scaled by 2, it is proposed to add a lower bound of 8ms on top of existing CSI-RS based L1 measurement period, e.g. max (8ms, TReport, ceil(M*P)*TCSI-RS) for non-DRX case.

- Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Agreements - For L1-RSRP and L1-SINR - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

In last meeting, it was agreed that for CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network. As for the impact on L1-RSRP and L1-SINR measurement period, it is necessary to define the delay requirements, and it is reasonable to follow NR-U approach to extend measurement period by excluding the resource occasions on invalid symbols. Similar issue exist for CSI-RS L1 measurement colliding with UL. It was agreed that dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement. And measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission following the R16 NR-U approach. From the perspective on how to handle the unusable resource occasions, the case of colliding with UL and the case of measurement on valid symbols are same. It is preferred to follow the same way.

Proposal 2: it is proposed that measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol following the R16 NR-U approach.

Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Proposals for requirements for DUD case - Proposal 1 (CATT, Nokia, E///, ZTE): - RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighboring cells. - UE needs to puncture the CSI-RS resources on SBFD symbols in frequency domain when target neighbour cell is a SBFD cell. (CATT, ZTE) - RAN4 can send LS to RAN2 to ask for supporting the configuration (E///) - RE-use the conclusion of L1-RSRP/SINR CSI-RS resource in different symbol types (ZTE) - Proposal 2 (Samsung, CMCC, HW): - For DUD case, use the same principle as agreed in last meeting for DU case - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS. (HW) - Proposal 3 (MTK): - RAN4 shall deprioritize CSI-RS based L3 measurements.

For the requirements for CSI-RS based L3 measurement, in our view, there is no difference between DU case DUD case. For DU case, it was agreed that UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. The agreements for DU case can be reused for DUD case. In detail, for DUD case, with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols, legacy requirements are applied.

Proposal 3: for DUD case, UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols.

Issue 2-1-3: Requirements for scheduling restriction - Proposals related to UL/DL transition - Proposal 1 (CATT, Nokia, E///, CMCC, HW, CTC, MTK): - Do not define new scheduling restriction requirements for DL/UL transition. - Proposals related to inter-frequency SSB measurement without gap - Proposal 1 (vivo, MTK): - For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed - the scheduling restriction in section 9.3.9.4 can be used as a reference (vivo) - UE is not required to receive DL in DL subbands on SBFD symbols (MTK) - Proposal 2 (CATT, Nokia, Samsung, HW): - RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point.

There is no scheduling restriciton for legacy L1 measurement, since there is guard period for the DL-to-UL transition. However, guard period is absent during the DL to UL transition. The potential issue is whether scheduling restriction is needed. RAN1 is discussing similar issue, as highlighted in blue as following. To avoid duplicated discussion, the issue can be left to RAN1.

Agreements in RAN1#116 meeting For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Proposal 4: it is proposed that RAN4 scheduling restriction requirement is not needed for DL/UL transition.

- Conclusion

This contribution provides discussion on RRM requirements for SBFD. The proposals are:

Proposal 1: for UE indicating that CSI processing time is scaled by 2, it is proposed to add a lower bound of 8ms on top of existing CSI-RS based L1 measurement period, e.g. max (8ms, TReport, ceil(M*P)*TCSI-RS) for non-DRX case.

Proposal 2: it is proposed that measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol following the R16 NR-U approach.

Proposal 3: for DUD case, UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols.

Proposal 4: it is proposed that RAN4 scheduling restriction requirement is not needed for DL/UL transition.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504480, Discussion on SBFD Operation, MediaTek inc.

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2420165, LS on CSI-RS measurement with SBFD operation

- R1-2501560, Reply LS on CSI-RS measurement with SBFD operation