# 3GPP TSG-RAN WG4 Meeting #115                             R4-2506600

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.18.5

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements of FR2-1 SSB based L3

measurement delay reduction for connected mode

Document for:	Discussion

- Introduction

In RAN #103 meeting, the WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- Objective of core part - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Objective of performance part - Specify RRM performance requirements and test cases for the Core part enhancements.

This was initial discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

Issue 2-1-1: TC list for Rx BSF optimization Agreement in online session: Further discuss based on the following table: Index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CMCC. 3; Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CATT. 2; Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Huawei. 4; Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Samsung. 5; Scenario 5: Handover; 1 or 2 TCs; TBA(e.g., Intra-frequency or inter-frequency, unknown); OPPO. 6; Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection; 1 or 2 TCs; TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown); Intel.

L3 measurement delay reduction by optimizing Rx BSF is to reduce measurement delay by enabling multi-Rx simultaneous reception. It is agreed that serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) is the triggering condition to activate/deactivate multi-Rx for L3 measurement. If the serving cell’s signal quality is below the threshold, multi-Rx for L3 measurement is activated. If the serving cell’s signal quality is above the threshold, multi-Rx for L3 measurement is deactivated.

According to the core part discussion, scenario 1/2/3/4/5/7 are considered as first priority. For scenario 1/2/3/4, (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. For scenario 5/7, for UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch) and RRC re-establishment.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

The SSB processing delay/time for processing multiple beams received in a SMTC is different for different scenarios. The details are as below.

- For scenario 1/2/3/4

  - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

    - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms

    - where TSSB_processing = 2 ms

- For scenario 7

  - to consider an additional processing delay (2ms) after the last SMTC periodicity

- For scenario 5

  - No need to add additional processing time in the delay requirement

It can be seen that the applied trigger condition and SSB processing delay/time are different for different scenarios. To guarantee the test coverage, it is proposed to define test cases for scenario 1/2/3/4/5/7.

Proposal 1: for Rx beam sweeping factor optimization, it is proposed to define test cases for scenario 1/2/3/4/5/7.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

It is agreed that L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), which means that test cases need to cover both non-DRX and DRX.

Proposal 2: for Rx beam sweeping factor optimization, it is proposed to define test cases for both non-DRX and DRX.

Issue 2-2-1: TC list for CSSF optimization - TC for solution 1: Agreement in online session: - Define the test cases for solution 1 of CSSF enhancement following the table below: TC index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; ZTE. - TC for solution 3: [Way forward]: - FFS: - Define the test cases for solution 3 of CSSF enhancement following the table below: TC index; Aspects; CA/DC mode; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA.

According to the core part discussion on optimizing CSSF outside gap in CA/DC, there are three candidate solutions. Solution 1 is to measure one serving CC per band. Solution 2 is to change the searcher occupancy ratio of measurement among the measured CCs. Solution 3 is to support 3 searchers for CSSF. For now, it is agreed not to support solution 2 in Rel-19. And solution 1 and solution 2 are supported.

For solution 1: measure one serving CC per band, the whole solution is as below. It can be seen that new network indication is introduced. One network indication is to enable/disable this CSSF enhancement. Another network indication is to indicate which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. It is necessary to define test cases to verify whether UE follow network indication to perform corresponding measurement. In last meeting, the test cases for solution 1 of CSSF enhancement is agreed as above. While FFS test cases for solution 3.

In prebious meeting, it was agreed to support solution 3 (perform CSSF enhancement based on 3 searchers). For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing by default.

For legacy 2 searcher case, one searcher is dedicated for PCC measurement. For FR1, all the SCCs share the 2nd searcher. For FR2, 50% of the 2nd searcher is for FR2 SCC where neighbour cell measurement is required, and the other 50% of the 2nd searcher is for FR2 SCC where neighbour cell measurement is not required, inter-frequency MO with no measurement gap and E-UTRA inter-RAT MO with no measurement gap.

It was agreed that the 3rd searcher resource sharing is same as the second searcher sharing. With this assumption, for FR1, all the SCCs share the 2nd and 3rd searcher. For FR2, 50% of the 2nd searcher and 50% of the 3rd searcher are for FR2 SCC where neighbour cell measurement is required, and the other 50% of the 2nd searcher and the other 50% of the 3rd searcher are for FR2 SCC where neighbour cell measurement is not required, inter-frequency MO with no measurement gap and E-UTRA inter-RAT MO with no measurement gap. Based on above consideration, it is necesary to define test cases to guarantee the performance for the case of three searchers.

Proposal 3: for solution 3 (perform CSSF enhancement based on 3 searchers), it is proposed to define test cases as following table:

| TC index | Aspects | CA/DC mode |
| --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them |

- Conclusion

This contribution provides discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode. The proposals are:

Proposal 1: for Rx beam sweeping factor optimization, it is proposed to define test cases for scenario 1/2/3/4/5/7.

- Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

Proposal 2: for Rx beam sweeping factor optimization, it is proposed to define test cases for both non-DRX and DRX.

Proposal 3: for solution 3 (perform CSSF enhancement based on 3 searchers), it is proposed to define test cases as following table:

| TC index | Aspects | CA/DC mode |
| --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them |

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements