3GPP TSG-RAN WG4 Meeting #115	R4-2506630

St Julian’s, Malta, 19th – 23rd May 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR NTN less than 5MHz SAN demodulation requirements

Agenda item:	7.10.4.6

Document for:	Discussion

# Introduction

Agreement:

Issue 1-3: PRACH – Scope of the SAN demodulation requirements

- No new PRACH requirements

In RAN#106, a revised WID [1] on NR NTN less than 5MHz was approved for Release 19. In RAN4#114-bis meeting, the discussions were kicked-off for SAN and UE demodulations requirements. Some agreements were achieved by the end of the meeting [2], copied as follows:

Open Issues:

Issue 1-1: PUSCH – Scope of the SAN demodulation requirements

Way forward:

- Option 1: No new PUSCH requirements

- Option 2: FFS

Inputs from NTN operators can be considered

Issue 1-2: PUCCH – Scope of the SAN demodulation requirements

Way forward:

- Option 1: FFS to introduce SAN PUCCH requirement with 3MHz bandwidth under NR NTN channel condition

- Option 2: Define PUCCH format 2 cases for SAN

Inputs from NTN operators can be considered

Issue 1-3: PRACH – Scope of the SAN demodulation requirements

- Discuss further whether to introduce the test applicability rule

In this contribution, we discuss on the remaining open issues, copied as follows:

# SAN Demodulation Requirements

2.1 General

According to the WID, the Rel-18 feature ‘Terrestrial Networks (TN) to support less than 5 MHz channel bandwidth (CBW)’ facilitates more efficient deployment of FR1 Non-Terrestrial Network (NTN) bands, especially in scenarios where satellite spectrum is limited. Currently, the minimum channel bandwidth defined for NTN FR1 is 5 MHz, as specified in 3GPP TS 38.108. Since the objective of this WI is to extend the Rel-18 efforts to NTN FR1 bands, it may be practical to define requirements for 3 MHz channel bandwidth—provided there is a significant performance difference compared to 5 MHz, or if only 3 MHz bandwidth support is needed for Satellite Access Networks (SAN), based on the current applicability rule for CBW in 3GPP TS 38.181.

Proposal 1: Introduce new PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS, if performance difference is observed from simulations between 3 and 5 MHz CBW, or 3MHz only support of SAN is needed.

Observation 1: It seems unlikely that only 3MHz channel bandwidth support would be needed for SAN.

2.2 Scope of the Requirements

From the open issues of PUSCH and PUCCH, and observation 1, companies need to discuss whether to define PUSCH and PUCCH requirements, based on the simulation results. Further, as per the WID [1], NR-NTN requirements should be defined for FR1 bands- n254, n255 and n256. However, given there are no physical layer enhancements in RAN1, the limited test cases of PUSCH and PUCCH requirements with 3 MHz could be checked.

Observation 2: It may be feasible to simulate limited test cases of PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS.

To check the 3MHz capability, only normal PUSCH requirements could be enough to simulate, e.g., CP-OFDM and DFT-s-OFDM requirements only.

Observation 3: The following cases for PUSCH FR1 3 MHz demodulation simulations for SAN can be considered:

- Requirements for PUSCH with transform precoding disabled

- Requirements for PUSCH with transform precoding enabled

Based on our simulation results [3], we did not observe performance difference between 3 and 5MHz CBW for normal SAN PUSCH requirements, particularly for Line-of-Sight (LoS) channel NTN-TDLC5-200. We observe <0.5 dB performance different for non-LoS channel NTN-TDLA100-200. However, this is not enough to create a requirement.

Observation 4: Based on our simulation result, the performance difference is very small between 3MHz and 5MHz in SAN normal PUSCH requirements. We expect similar variation in performance for other SAN PUSCH requirements.

Proposal 2: It does not seem necessary to simulate SAN PUSCH requirements for 3 MHz CBW and further not to create PUSCH requirements.

Regarding PUCCH simulations, following Rel-18 feature ‘Terrestrial Networks (TN) to support less than 5 MHz channel bandwidth’, we simulated PUCCH format 2 as the starting point and as expected, we observed obvious performance difference between 3 and 5 MHz CBW. This is because the frequency diversity provided by intra-slot frequency hopping is reduced when operating over narrow bandwidths. The limited frequency hopping range leads to a noticeable degradation in PUCCH detection performance. While some performance loss is inherent due to the narrow bandwidth, this is significantly observed in PUCCH Format 2. Frequency bins with hopping can overlap with PRB allocation. In that case, requirement for 3MHz would be needed.

Table 2.2-1: Minimum requirements for PUCCH format 2, 15 kHz SCS and 3/5 MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex D) | SNR (dB) for 3 MHz CBW | SNR (dB) for 5 MHz CBW |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 8.42 | 4.21 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 0.79 | -1.49 |

Observation 5: There is obvious performance difference on PUCCH format 2 for UCI BLER test case between 5MHz and 3MHz.

Proposal 3: Introduce new PUCCH format 2 for UCI BLER requirements for 3MHz.

Further, it might be feasible to simulate other SAN PUCCH requirements also to verify if there is a performance difference between 3 and 5 MHz requirement.

Proposal 4: The following cases for PUCCH FR1 3 MHz demodulation simulations for SAN can also be considered: Format 1//3/4 and multi-slot PUCCH.

Furthermore, it would be needed to define the applicability rule for the SAN PUCCH requirements created with 3MHz channel bandwidth. To define the applicability rule, a similar approach can be followed for “Rel-18 TN less than 5MHz requirement”, where separate applicability rule was added in Sec. 8.1.2.2.3 of TS 38.141-1. It stated that a BS declaring to support 3MHz CBW shall be tested with respective performance test case, and 5MHz CBW test can be skipped, if applicable.

Proposal 5: Define applicability rule for SAN PUCCH requirements with 3MHz channel bandwidth, if the requirements are created.

Regarding PRACH, no new requirements are agreed in the previous 3GPP meeting. Also, as per applicability rule for different CBW of PRACH in TS 38.181, the test requirements apply for any CBW that is supported by SAN. Therefore, no applicability rule is needed for PRACH.

Proposal 6: It is not required to define applicability rule for SAN PRACH requirements with 3MHz channel bandwidth.

2.3 Simulation Assumptions

For PUCCH requirements, we can refer to the existing specifications for 15 kHz SCS and 5 MHz channel bandwidth provided in TS 38.108, while adjusting the “first PRB after frequency hopping” to reflect the reduced 3 MHz channel bandwidth. We can consider the cases given in the WF [2].

Proposal 7: Consider simulation assumptions for SAN PUCCH requirements for 3MHz channel bandwidth as given in the way forward:

PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |

PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

PUCCH format 3

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2:11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |

PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

# UE Demodulation Requirements

The remaining issue on UE demodulation requirements for NTN 3MHz CBW is whether to define new (puncturing) PDCCH demodulation requirements or not [4].

Way forward: - Option 1: No new PDCCH requirements - Option 2: Discuss the feasibility of introducing PDCCH requirements

In Rel-18 NR less than 5MHz, RAN4 defined PDCCH demodulation requirements with puncturing assuming the gNB schedule PDCCH on CORESET0 [5].

On the other hand, TS 38.101-5 Table 8.2.1.1.2-1 specifies that:

- “For UE supporting NR NTN access (nonTerrestrialNetwork-r17), the requirements in TS38.101-4 also applies to UE according to applicability rules in TS38.101-4 Clause 5.1, 6.1, 7.1 and 8.1”

This means if NTN access UE supports less than 5MHz feature (support3MHz-ChannelBW-Symmetric-r18), the UE needs to pass the punctured PDCCH demodulation requirements specified in TS 38.101-4 5.3.

Observation 6: NTN UE supporting the less than 5MHz feature need to pass the punctured PDCCH demodulation requirements specified in TS 38.101-4 5.3.

Figure 1 shows our simulation results of puncturing PDCCH demodulation with TN/NTN channel models. The simulation uses the Rel-18 puncturing PDCCH demodulation requirement parameters specified in TS 38.101-4 5.3.2.7, that is, one of 8 RBs are punctured. This figure also compares the performance difference between puncturing and non-puncturing scenarios.

Figure 1	Simulation results of PDCCH demodulation with/without puncturing.

Table 1	Summary of PDCCH demodulation simulation results.

| Channel model | SNR for 1% BLER with non-puncturing | SNR for 1% BLER with puncturing | SNR difference between non-puncturing and puncturing |
| --- | --- | --- | --- |
| TDLA30-10 | 2.4 | 3.5 | 1.1 |
| NTN-TDLA30-200 | 3.4 | 4.4 | 1.0 |
| NTN-TDLC5-200 | -3.7 | -2.7 | 1.0 |

Table 1 shows the SNR difference regarding SNR to achieve 1% BLER between puncturing and non-puncturing cases. The result shows the degradation of SNR with puncturing is about 1dB regardless of TN channel model (TDLA30-10) or NTN channel models (NTN-TDLA30-200 or NTN-TDLC5-200).

Observation 7: Same SNR difference between non-puncturing case and puncturing case for TDLA30-10 and NTN-TDLA30-200/NTN-TDLC5-200.

We think the existing puncturing PDCCH demodulation requirements specified in TS 38.101-4 5.3 are sufficient to verify the NTN UE demodulation performance. We therefore don’t think RAN4 need to add new PDCCH demodulation requirements with puncturing in TS 38.101-5.

Proposal 8: No new PDCCH demodulation requirements with puncturing in TS 38.101-5.

# Conclusion

4.1	SAN Demodulation requirements

Based on the above discussion, following observations are captured:

Observation 1: It seems unlikely that only 3MHz channel bandwidth support would be needed for SAN.

Observation 2: It may be feasible to simulate limited test cases of PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS.

Observation 3: The following cases for PUSCH FR1 3 MHz demodulation simulations for SAN can be considered:

- Requirements for PUSCH with transform precoding disabled

- Requirements for PUSCH with transform precoding enabled

Observation 4: Based on our simulation result, the performance difference is very small between 3MHz and 5MHz in SAN normal PUSCH requirements. We expect similar variation in performance for other SAN PUSCH requirements.

Observation 5: There is obvious performance difference on PUCCH format 2 for UCI BLER test case between 5MHz and 3MHz.

Based on the above discussion, following proposals are captured:

Proposal 1: Introduce new PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS, if performance difference is observed from simulations between 3 and 5 MHz CBW, or 3MHz only support of SAN is needed.

Proposal 2: It does not seem necessary to simulate SAN PUSCH requirements for 3 MHz CBW and further not to create PUSCH requirements.

Proposal 3: Introduce new PUCCH format 2 for UCI BLER requirements for 3MHz.

Proposal 4: The following cases for PUCCH FR1 3 MHz demodulation simulations for SAN can also be considered: Format 1//3/4 and multi-slot PUCCH.

Proposal 5: Define applicability rule for SAN PUCCH requirements with 3MHz channel bandwidth, if the requirements are created.

Proposal 6: It is not required to define applicability rule for SAN PRACH requirements with 3MHz channel bandwidth.

Proposal 7: Consider simulation assumptions for SAN PUCCH requirements for 3MHz channel bandwidth as given in the way forward.

4.2	UE Demodulation requirements

Observation 6: NTN UE supporting the less than 5MHz feature need to pass the punctured PDCCH demodulation requirements specified in TS 38.101-4 5.3.

Observation 7: Same SNR difference between non-puncturing case and puncturing case for TDLA30-10 and NTN-TDLA30-200/NTN-TDLC5-200.

Proposal 8: No new PDCCH demodulation requirements with puncturing in TS 38.101-5.

# References

[1] RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, 3GPP TSG RAN Meeting #106

[2] R4-2504753, “WF for NR IoT NTN less than 5MHz demod”, Ericsson, RAN4#114-bis, Wuhan, China.

[3] R4-2506631, “Simulations on NR NTN less than 5MHz SAN demodulation requirements”, Ericsson, RAN4#115, Malta.

[4] R4-2504753 “Way Forward for [114bis][324] NR_IoT_NTN_less_than_5MHz_demod”, Ericsson.

[5] R4-2409843, “BigCR for 38.101-4 on Lessthan5MHz UE demod perf and CSI requirements”, Nokia.