3GPP TSG-RAN WG4 Meeting #115	   R4-2506664

# St.Julian’s, Malta, 12th – 17th , May , 2025

Agenda Item:	7.18.6

Source: 	Ericsson

Title:	On performance aspect of the Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

From RAN #103 meeting the WID for RRM enhancement phase 5 is listed in [1]. In this paper we provide our view for the initial thoughts on RRM performance for fast Scell activation for UE supporting Rel-18 EMR.

Background: WID objective #2

- Fast SCell activation for UE supporting Rel-18 EMR

- Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

- Apply fast scell activation in FR1 and FR2-1

- Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

The Rel-19 RRM enhancement builds upon two new UE capabilities. The updated Scell activation requirements, which are part of this enhancement, have been agreed upon and endorsed in R4-2502685.

For UEs supporting Rel-18 EMR, several new test cases have been introduced based on two distinct new capabilities. The Scell activation requirements have also been further enhanced for these UEs, covering RRC-based DL Scell activation, MAC CE-based DL Scell activation, and PUCCH Scell activation.

Additionally, legacy Scell activation features and Rel-18 Idle/Inactive eEMR measurement reporting are supported with existing test cases that address various UE behaviors and scenarios which can be seen from table 1 and table 2.

| Feature | Side Condition | FR-group | Test case |
| --- | --- | --- | --- |
| Scell activaiton/deactivation | known | FR2 | A5.5.3.2 |
| Scell activaiton/deactivation | unknown | FR2 | A5.5.3.5 |
| Direct Scell activation | known | FR2 | A5.5.3.7 |
| PUCCH Scell activation/deactiation | known | FR2 | A5.5.3.9 |
| PUCCH Scell activation/deactivation | unknown | FR2 | A5.5.3.10 |
| Scell activaiton/deactivation | known | FR1 | A6.5.3.1 |
| Scell activation/deactivaiton | unknown | FR1 | A6.5.3.3 |
| Direct Scell activation | known | FR1 | A6.5.3.4 |
| PUCCH Scell activation/deactiation | known | FR1 | A6.5.3.6 |
| PUCCH Scell activation/deactivation | unknown | FR1 | A6.5.3.7 |

Table 1. Existing Scell activation test cases

| Feature | UE behaviour | FR group | Test case |
| --- | --- | --- | --- |
| Legacy EMR test | With reporting | FR1 | A6.6.9.1 |
| Idle mode fast CA/DC eEMR | Without reporting | FR1 | A6.6.9.2 |
| Idle mode fast CA/DC cell reselection | Without reporting | FR1 | A6.6.9.3 |
| Idle mode fast CA/DC cell reselection | With reporting | FR1 | A6.6.9.4 |
| Idle mode fast CA/DC eEMR | Without reporting | FR2 | A7.6.23.1 |
| Idle mode fast CA/DC cell reselection | Without reporting | FR2 | A7.6.23.2 |
| Idle mode fast CA/DC cell reselection | With reporting | FR2 | A7.6.23.3 |

Table 2. Existing EMR/Cell reselection measurement report test case

- Observation 1: Existing test cases are available for both Scell activation and Rel-18 Idle/Inactive state measurement reporting.

- Observation 2: The Legacy EMR with reporting for FR2 test case is not available from the specification.

- Observation 3: The Direct Scell activation for unknown test case is not available from the specification.

The Rel-19 fast scell activation enhancement is based on the EMR report can further enhance the unknown target cell activation requirement. The test case can be mainly focus on the UE have a valid EMR/Reselection report and further activate an unknown target cell.

- Proposal 1: RAN4 further discuss whether to introduce test case for the scenario where UE report EMR in FR2 and unknown Scell for direct Scell activation.

From UE procedure aspect, performance evaluation begins when the UE receives the measurement configuration for EMR measurement and concludes at when UE sent the CSI report for a successful Scell activation. Detail can be seen from figure 1.

Figure 1 signal flow of the Fast scell activation for UE supporting Rel-18 EMR

From a performance perspective, if the UE sends a valid measurement report 5 seconds prior to receiving the Scell activation command, the target cell is considered known. The known condition will apply, and specific delay activation requirements will be verified during the test. Conversely, if the UE does not send a valid measurement report before the activation command, the target cell is treated as unknown, and the unknown activation requirements will be verified during the test case.

From a test case timeline perspective, fast Scell activation for UEs supporting Rel-18 EMR requires a combination of two existing test cases, as illustrated in Figure 2.

Figure 2. Test case timelines to combine existing test cases.

- Observation 4: The EMR/Cell reselection test case and the Scell activation test case follow relatively independent timelines.

Since the two feature sets follow independent timelines, the performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated using a configuration that combines the two relevant test cases. This is feasible because there is no overlap in test behavior or verification requirements between them.

If the EMR or cell reselection behavior under verification involves the UE sending a valid measurement report, then the known Scell activation test cases should be used to evaluate the activation requirements.
Conversely, if the UE behavior does not include sending a valid measurement report, the unknown Scell activation test cases should be used for evaluation.

Based on different EMR reporting UE behavior and the Scell known or unknown condition listed in Table 1. We think the test case can be configured by combine the two test cases and follow the suggested timeline from figure.2 The EMR report will enhance the unknown target cell activation, so the case can be verified from different test configuration is that when there is an EMR report being sent combined with unknown target cell activation.

The test configuration can be seen from table.2

|  | Scenario | Test case configuration |  | Expected test outcome |
| --- | --- | --- | --- | --- |
| Rel-19 fast Scell activation FR1 | EMR UE report + unknown Scell activation | A6.6.9.1 | A6.5.3. 3 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | EMR UE report + unknown PUCCH Scell activation/deactivation | A6.6.9.1 | A6.5.3.7 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | Cell reselection UE report + unknown Scell activation | A6.6.9.4 | A6.5.3. 3 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR1 | Cell reselection UE report + unknown PUCCH Scell activation/deactivation | A6.6.9.4 | A6.5.3.7 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR2 | Cell reselection UE report + unknown Scell activation | A7.6.23.3 | A5.5.3.5 | Activation delay will be enhanced. |
| Rel-19 fast Scell activation FR2 | Cell reselection UE report + unknown PUCCH Scell activation/deactivation | A7.6.23.3 | A5.5.3.10 | Activation delay will be enhanced. |

- Proposal 2: The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle state fast CA/DC reporting test case such as take table.2 as test configuration.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: RAN4 further discuss whether to introduce test case for the scenario where UE report EMR in FR2 and further activate unknown Scell for direct Scell activation.

	Proposal 2: The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle state fast CA/DC reporting test case such as take table.2 as test configuration.

# References

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, VIVO, March 2024

- R4-2504900 “RRM pha5 part II WF”, CATT, February 2025