3GPP TSG-RAN WG4 Meeting #115	   R4-2506669

# St, Julian’s, Malta, 12th – 19th , May , 2025

Agenda Item:	7.25.5

Source: 	Ericsson

Title:	Discussion on RRM performance requirements for SBFD

Document for:	Discussion

# Introduction

In this paper we provide our view on RRM performance requirement within related WID [1] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

Scenarios

The Rel-19 SBFD RRM core requirement mainly focus on two objectives, the L1-CLI handling mechanism and the RRM core requirement for SBFD operation.

The L1-CLI measurement due to SBFD operation have similarity with the Rel-16 L3-CLI measurement from measurement procedure point of the view. However certain differences can be observed for the L1-CLI measurement for SBFD.

The L1-CLI measurement resource can be configured with either DL subband or UL subband. While the L3-CLI measurement resource was configured with the active BWP.

The measurement timing and measurement period for L1-CLI measurement will be different from the Rel-16 L3-CLI measurement. On top of that the scheduling restriction is also different for L1-CLI-RSSI measurement.

The L1-SRS-RSRP scheduling restriction was specified based on different SCS, different number of symbol will be restricted before the SRS resource.

The legacy L3-CLI measurement test case has been specified both in FR1 and FR2, considering SA and DC scenario. While for SBFD, the current working assumption is that the SBFD operation only with single cell.

Also to mention the Legacy L3-CLI measurement test case combined the scheduling restriction within the test case for different SCS.

- Proposal 1: The SBFD RRM performance requirement for L1-CLI handling and other RRM measurement impact due to SBFD operation need to take consideration for both FR1 and FR2 for single cell operation.

- Proposal 2: RAN4 further discuss how to specify L1-CLI measurement test case such as potential re-use of the L3-CLI measurement test configuration.

Test methodology and configuration

The SBFD operation impact on the RRM measurement requirement mostly reflected on the CSI-RS based L1-RSRP/SINR measurement, the RLM/BFD/CBD measurement and the L3 RRM measurement due to the fundamental difference between a SBFD symbol and non-SBFD symbol.

Currently the impact on CSI-RS based L1-RSRP/SINR measurement requirement is being discussed for the core requirement, however the measurement requirement impact is most likely on the measurement accuracy and measurement period due to the minimum number of CSI-RS resource will be impacted due to punctuation of CSI-RS resources. Another impact is that the UE will perform the CSI-RS based measurement during SBFD operation based on network indication of valid symbol type.

- Proposal 3: RAN4 further discuss how to specify CSI-RS based L1-RSRP/SINR measurement test case due to the CSI-RS resource punctuation and the valid symbol type indication.

Regarding the RLM/BFD/CBD measurement, the impact due to CSI-RS punctuation is quite similar as the L1-RSRP/SINR, the difference is that the valid symbol type indication is not applicable to the RLM/BFD/CBD measurement.

The legacy RLM/BFD test case have been specified for both SA and DC scenario, for SBFD the working assumption for Rel-19 is considering SBFD operation for single cell.

The legacy test procedure for Out of sync and In sync for RLM/BFD can be reused, the details for the test requirement for how long the time duration should be and how UE should behave due to SBFD impact on CSI-RS resources needs to be further discussed when the core requirement is stable.

- Proposal 4: RAN4 further discuss how to specify CSI-RS based RLM/BFD/CBD test case and test requirement due to the CSI-RS resource punctuation for SBFD symbols.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: The SBFD RRM performance requirement for L1-CLI handling and other RRM measurement impact due to SBFD operation need to take consideration for both FR1 and FR2 for single cell operation.

	Proposal 2: RAN4 further discuss how to specify L1-CLI measurement test case such as potential re-use of the L3-CLI measurement test configuration.

	Proposal 3: RAN4 further discuss how to specify CSI-RS based L1-RSRP/SINR measurement test case due to the CSI-RS resource punctuation and the valid symbol type indication.

	Proposal 4: RAN4 further discuss how to specify CSI-RS based RLM/BFD/CBD test case and test requirement due to the CSI-RS resource punctuation for SBFD symbols.

# References

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024