3GPP TSG-RAN WG4 Meeting #115	R4-2506673

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.1.4

Source:	Samsung

Title:	Discussion on UE demodulation performance requirements for 6Rx

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The WID for performance part has been updated as below [2] according latest RAN4#114 meeting agreements [3].

The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

In last RAN4#114bis meeting, work plan and general scopes are agreed [4]. In this contribution, we provide our further analysis on the open issues of 6Rx PDSCH demodulation performance and CSI requirements, and 6Rx SDR requirements.

- Discussion

  - General

In last meeting, below agreements are achieved [4].

Issue 1-1: Single carrier test - Agreement - Only define single carrier test for 6Rx, i.e. no-CA related tests. Issue 1-2: FR1 TDD test - Agreement - Only define test cases for FR1 TDD - 30kHz SCS - 40MHz CBW - TDD UL-DL pattern: 7DS2U, S: 6D+4G+4U Issue 1-3: PDCCH test - Agreement - Not define PDCCH demodulation requirements Issue 1-4: PBCH test - Agreement - Not define PBCH demodulation requirements Issue 1-5: Test applicability rule for 6Rx test cases - Way forward - Option 1: RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification. - Option 2: - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices. Issue 1-6: Specification structure - Proposals - Option 1: Add a new clause for 6Rx - Other options - Recommended WF Postpone the discussion to the CR drafting stage

Test applicability rule for 6Rx test cases

For the test applicability rule for 6Rx test cases, since we already have conclusion and updated WID as

    - 6 MIMO layers performance requirements will be only defined for FWA.

    - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices.

The facts about 6 MIMO layers only for FWA should be clearly captured in the applicability rules. In addition, for UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage.

Proposal 1: The facts about 6 MIMO layers performance requirements are only for FWA should be clearly captured in the applicability rules.

Proposal 2: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage.

  - PDSCH demodulation performance requirements

The agreement for introducing 6Rx PDSCH demodulation performance requirements for FR1 TDD single carrier has been achieved. Therefore, related test assumptions and parameters should be discussed and aligned.

Test scenarios

The way forward for test scenarios as below.

Test scenarios - Way forward - Option 1: Test scenarios without interference: PDSCH Mapping Type A - Option 2: Test scenarios with interference - PDSCH demodulation requirements with inter-cell interference - PDSCH demodulation requirements with intra-cell inter-user interference

It seems no need to define 6Rx PDSCH demodulation requirements for every scenario, we prefer to define limited test cases for 6Rx UE, e.g., only define 6Rx PDSCH demodulation requirements for scenarios without interference with PDSCH Mapping Type A.

Proposal 3: Prefer to only define 6Rx PDSCH demodulation requirements for scenarios without interference with PDSCH Mapping Type A.

Test cases for 4 MIMO layers for FWA and Handheld UE

Considering FWA and Handheld UE usually have different product forms and different typical parameters, such as different antenna correlations. We think it’s more reasonable to define two separate cases for FWA and Handheld UE with 4 MIMO layers.

Proposal 4: Prefer to define two separate test cases for 4 MIMO layers for FWA and Handheld UE.

Propagation channel and MIMO correlation for 6 MIMO layers

For the propagation channel and MIMO correlation matrices for 6 MIMO layers, we think follow current 8 layers is enough, use TDLA 30-10 ULA Low.

Proposal 5: Prefer to use TDLA 30-10 ULA Low as the propagation channel and MIMO correlation matrices for 6 MIMO layers.

Propagation channel for 4 MIMO layers

For the propagation channel for 4 MIMO layers, different selection for different UE types is perfect if separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, TDLA 30-10 is a good option.

Proposal 6: Different propagation channel selection for different UE types is perfect if separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, i.e., TDLA30-10 for FWA and TDLC300-100 for Handheld. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, TDLA 30-10 is a good option.

MIMO correlation for 4 MIMO layers

This issue also has relationship with the number of test cases for 4 MIMO layers. If separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to use ULA Low for FWA and ULA Medium B for Handheld UE. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, we prefer to use ULA Low.

Proposal 7: If separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to use ULA Low for FWA and ULA Medium B (alpha=0.3, beta=0.005154) for Handheld UE. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, we prefer to use ULA Low.

Meanwhile, the UE correlation matrix for 6Rx should be introduced as

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

Proposal 8: UE correlation matrix for 6 Rx should be introduced as

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

DMRS configuration for 6 MIMO layers case

Per our understanding and the description of DMRS configurations described in 38.211 and 38.331, DMRS type 1 with one additional OFDM symbol for DMRS ports and maxLength=1 is the typical DMRS configuration (Rel-15 1+1 DMRS symbols) for 4 MIMO layers as Figure 2-1 (a), it is straightforward to configure maxLength=2 in order to support 6 MIMO layers as Figure 2-1 (b), this is the typical configuration for real network.  For Rel-18 1+1(6L), it only could be supported when the feature of Rel-18 enhanced DMRS is supported.

38.212, DMRS ports configurations

- Rel-15 typical 4 ports configuration

- Rel-15 typical 6 ports configuration

- Rel-18 6 ports configuration

Figure 2-2. DMRS ports configurations for Rel-15 DMRS and Rel-18 DMRS

Proposal 9: For DMRS configuration for 6 MIMO layers case, use the typical configuration Rel-15 Type 1 DMRS 2+2 and DRMS ports {1000-1003, 1004, 1006}.

Antenna configuration for 4 MIMO layers case

For 4 MIMO layers, both 4Tx6Rx and 8Tx6Rx antenna configurations could achieve the purpose.

Proposal 10: For DMRS configuration, we think both 4Tx6Rx and 8Tx6Rx are acceptable.

MCS for 6 MIMO layers case

For PDSCH demodulation performance requirements, MCS17 and MCS Table 1 are assumed for Rel-18 8 Rx which is introduced for FWA. Hence, we prefer to use MCS17 and MCS Table 1 as the starting point for 6Rx.

Proposal 11: For PDSCH demodulation performance requirements of 6Rx with 6 MIMO layers, select MCS17 and MCS Table 1 as the starting point.

MCS for 4 MIMO layer case

For PDSCH demodulation performance requirements, MCS17 and MCS Table 1 are assumed for Rel-18 8 Rx which is introduced for FWA. Hence, we prefer to use MCS17 and MCS Table 1 as the starting point for 6Rx.

Proposal 12: For PDSCH demodulation performance requirements of 6Rx with 4 MIMO layers, select MCS17 and MCS Table 1 as the starting point.

Other test parameters for PDSCH test

For other test parameters, we prefer to use option 1 as below as the starting point for 6Rx.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Test metric |  |  | SNR@70% max TP |

Proposal 13: For other test parameters, we prefer to use option 1 as the starting point for 6Rx.

  - SDR requirements

Static channel and precoding for 6Rx

For the static channel matrices, for rank1, rank2, rank4, and rank6, the number of Singular Values should be at least 1, 2, 4 and 6 separately. According this rule, we propose following static channel matrices.

For 1 port transmission the channel matrix is defined in the frequency domain by

H=111111

For 2 port transmission the channel matrix is defined in the frequency domain by

H=111j-jj-1-1-1j-jj

For 4 port transmission the channel matrix is defined in the frequency domain by

H=111111111-1-1-1     j-jjj-j-j-jj-j-j-jj

For 8 port transmission the channel matrix is defined in the frequency domain by

H=1111111111-1-1     11-111-1-111-1-1-1   j-jjj-jj-jj-j-j-jj    j-j-jj-j-jjj-jj-jj

Proposal 14: For UE Receiver with 6Rx, following static channel matrices are proposed.

- For 1 port transmission the channel matrix is defined in the frequency domain by

H=111111

- For 2 port transmission the channel matrix is defined in the frequency domain by

H=111j-jj-1-1-1j-jj

- For 4 port transmission the channel matrix is defined in the frequency domain by

H=111111111-1-1-1     j-jjj-j-j-jj-j-j-jj

- For 8 port transmission the channel matrix is defined in the frequency domain by

H=1111111111-1-1     11-111-1-111-1-1-1   j-jjj-jj-jj-j-j-jj    j-j-jj-j-jjj-jj-jj

SDR requirements for 64QAM and 256QAM

Reuse the MCS values for 2 and 4 layers agreed in last RAN4#114bis meeting. While for MIMO layer as 6, new requirements of SDR for up to maximum 256QAM need to be defined. Furthermore, in current spec, for MIMO layer 4 and 8, only the requirements with scaling factor 1 for 256QAM and 64QAM changed, hence we only need to do maximum MCS feasibility for MIMO layer 6 with 256QAM and 64QAM with scaling factor 1, for other scaling factors, reuse the MCS values for 4 layers or 8 layers.

Proposal 15: Define new SDR requirements of 6Rx with MIMO layer 6 as below table, and do maximum MCS feasibility for 256QAM and 64QAM with scaling factor 1.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | TBD |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | TBD |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |

SDR requirements for 1024QAM

For 1024QAM, current spec only define SDR requirements up to MIMO layer 2 even for 8Rx, hence new SDR requirements for 1024QAM with MIMO layer 2 should be defined, and no need to do the feasibility of 1024QAM with 4 MIMO layers.

Proposal 16: For the SDR requirements of 6Rx with 1024QAM, no need to do the feasibility of 1024QAM with 4 MIMO layers.

Proposal 17: For 1024QAM, define SDR requirements for MIMO layer 2 as below, and study if MCS24 is achieveable for scaling factor 1.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | TBD |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

  - CSI reporting performance requirements

CQI reporting requirements

For the CQI estimation processing, there is no UE handling difference for 6Rx and 4Rx/8Rx, hence we don’t think new CQI reporting requirements are necessary for 6Rx. In current spec, CQI requirements with rank 4 under static channel are defined for 8Rx, from this point of view, it is also acceptable to define CQI requirements with rank 4 under static channel for 6Rx. No need to define wideband CQI reporting requirements with inter-cell interference under fading conditions at all.

Proposal 18: For CQI reporting requirements, prefer not to define new CQI reporting requirements for 6Rx as first priority. It is also acceptable to only define CQI requirements with rank 4 under static channel for 6Rx as second priority. No need to define wideband CQI reporting requirements with inter-cell interference under fading conditions at all.

RI reporting requirements

Similar reasons as above for CQI and PMI reporting requirement, we don’t prefer to define new test requirements for RI reporting for 6Rx.

Proposal 19: For RI reporting requirements, prefer not to define new RI reporting requirements for 6Rx.

- Conclusion

In this contribution, we provide our further analysis and views on the PDSCH demodulation performance, SDR requirements and CSI reporting performance requirements on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

For General

Proposal 1: The facts about 6 MIMO layers performance requirements are only for FWA should be clearly captured in the applicability rules.

Proposal 2: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage.

For PDSCH demodulation performance requirements

Proposal 3: Prefer to only define 6Rx PDSCH demodulation requirements for scenarios without interference with PDSCH Mapping Type A.

Proposal 4: Prefer to define two separate test cases for 4 MIMO layers for FWA and Handheld UE.

Proposal 5: Prefer to use TDLA 30-10 ULA Low as the propagation channel and MIMO correlation matrices for 6 MIMO layers.

Proposal 6: Different propagation channel selection for different UE types is perfect if separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, i.e., TDLA30-10 for FWA and TDLC300-100 for Handheld. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, TDLA 30-10 is a good option.

Proposal 7: If separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to use ULA Low for FWA and ULA Medium B (alpha=0.3, beta=0.005154) for Handheld UE. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, we prefer to use ULA Low.

Proposal 8: UE correlation matrix for 6 Rx should be introduced as

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

Proposal 9: For DMRS configuration for 6 MIMO layers case, use the typical configuration Rel-15 Type 1 DMRS 2+2 and DRMS ports {1000-1003, 1004, 1006}.

Proposal 10: For DMRS configuration, we think both 4Tx6Rx and 8Tx6Rx are acceptable.

Proposal 11: For PDSCH demodulation performance requirements of 6Rx with 6 MIMO layers, select MCS17 and MCS Table 1 as the starting point.

Proposal 12: For PDSCH demodulation performance requirements of 6Rx with 4 MIMO layers, select MCS17 and MCS Table 1 as the starting point.

Proposal 13: For other test parameters, we prefer to use option 1 as the starting point for 6Rx.

For SDR requirements

Proposal 14: For UE Receiver with 6Rx, following static channel matrices are proposed.

- For 1 port transmission the channel matrix is defined in the frequency domain by

H=111111

- For 2 port transmission the channel matrix is defined in the frequency domain by

H=111j-jj-1-1-1j-jj

- For 4 port transmission the channel matrix is defined in the frequency domain by

H=111111111-1-1-1     j-jjj-j-j-jj-j-j-jj

- For 8 port transmission the channel matrix is defined in the frequency domain by

H=1111111111-1-1     11-111-1-111-1-1-1   j-jjj-jj-jj-j-j-jj    j-j-jj-j-jjj-jj-jj

Proposal 15: Define new SDR requirements of 6Rx with MIMO layer 6 as below table, and do maximum MCS feasibility for 256QAM and 64QAM with scaling factor 1.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | TBD |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | TBD |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |

Proposal 16: For the SDR requirements of 6Rx with 1024QAM, no need to do the feasibility of 1024QAM with 4 MIMO layers.

Proposal 17: For 1024QAM, define SDR requirements for MIMO layer 2 as below, and study if MCS24 is achieveable for scaling factor 1.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | TBD |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

For CSI reporting requirements

Proposal 18: For CQI reporting requirements, prefer not to define new CQI reporting requirements for 6Rx as first priority. It is also acceptable to only define CQI requirements with rank 4 under static channel for 6Rx as second priority. No need to define wideband CQI reporting requirements with inter-cell interference under fading conditions at all.

Proposal 19: For RI reporting requirements, prefer not to define new RI reporting requirements for 6Rx.

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2503014, WF on 6Rx UE requirements, RAN4#114 meeting, AT&T

[4] R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#114bis meeting, Huawei, HiSilicon