3GPP TSG-RAN WG4 Meeting #115	R4-2506674

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.3.5

Source:	Samsung

Title:	Discussion and simulation results on UE demodulation performance requirements for less than 5MHz FR1 Phase 2

Document for:	Discussion

- Introduction

In the RAN#107 meeting, the latest WID [1] for “NR channel BW less than 5MHz for FR1 Phase 2” has been revised as below. This is the second meeting for demodulation performance part, and the performance part is targeting to be finished at June/2025 [2], only two meetings scheduled for demodulation performance part.

- Objective of SI or Core part WI or Testing part WI - Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz or 5MHz in band n100 and 5MHz or 10MHz in band n101 NOTE: Consideration of 5MHz channel bandwidth in the above “3MHz or 5MHz” is specific to band n100 and does not apply to other bands. - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified. - Objective of Performance part WI - Specify necessary performance requirements (RAN4)

In this contribution, we provide our analysis and simulation results on demodulation performance impacts for less than 5MHz phase 2.

- Discussion

  - PDSCH CA demodulation requirements for 3MHz CHBW

Whether to define PDSCH CA demodulation requirements for 3MHz CHBW

For less than 5MHz phase 1 in Rel-18 with single carrier scope, no PDSCH demodulation test requirements defined for less than 5MHz since almost no performence impact and generally requirements only based on typical channel bandwidth on PDSCH demodulation. Meanwhile, 3MHz PDCCH demodulation test requirements are introduced since the CCE resource puncturation has impact on PDCCH demodulaiotn performance. In addition, 3MHz PDCCH demodulation test requirements are introduced only for FR1 FDD since 3MHz is only supported by some FR1 FDD bands listed as below Table 2-1.

Table 2-1 3MHz supported Bands defined in 38.101-1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n26 | 814 MHz – 849 MHz | 859 MHz – 894 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n31 | 452.5 MHz – 457.5 MHz | 462.5 MHz – 467.5 MHz | FDD |
| n72 | 451 MHz – 456 MHz | 461 MHz – 466 MHz | FDD |
| n85 | 698 MHz – 716 MHz | 728 MHz – 746 MHz | FDD |
| n10021 | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |
| n106 | 896 MHz – 901 MHz | 935 MHz – 940 MHz | FDD |

For less than 5MHz phase 2 in Rel-19 with CA scope, co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW are introduced, whether corresponding PDSCH demodulation requirements should be defined or not need further discussion. Firstly, in current spec, less than 5MHz bandwidth only supported CA bandwidth combination are not defined in 38.101-1. Secondly, it is hard to find opportunity to test the 3MHz case if follow existing test applicability rules for inter-band CA defined as Table 2-2. Hence if PDSCH CA demodulation requirements for 3MHz bandwidth will be defined, it’s better to update existing applicability rules for inter-band CA configuration selection.

Table 2-2 Existing applicability rules for inter-band CA

- Existing applicability rules for inter-band CA configuration selection: - Step 1: Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. - Step 2: Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. - Step 3: Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. - Step 4: Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3.

Observation 1: Less than 5MHz bandwidth only supported CA bandwidth combination are not defined in 38.101-1. It is hard to find opportunity to test the 3MHz case if follow existing test applicability rules for inter-band CA defined as Table 2-2.

Proposal 1: For less than 5MHz phase 2, consider introducing PDSCH CA demodulation requirements and update existing applicability rules for 3MHz.

  - SDR requirements for 3MHz CHBW

Whether to define SDR requirements for 3MHz CHBW

If PDSCH CA demodulation requirements for 3MHz channel bandwidth will be defined in Rel-19, corresponding configuration for SDR requirements in Table 5.5A-1 and Table 5.5A-4 should be updated accordingly.

Proposal 2: Prefer to update corresponding configuration for SDR requirements in Table 5.5A-1 and Table 5.5A-4 to include 3MHz, if PDSCH CA demodulation requirements for 3MHz channel bandwidth are agreed be defined in Rel-19.

  - CA CQI reporting requirements for 3MHz CHBW

Whether to define CA CQI reporting requirements for 3MHz CHBW

Similar as PDSCH CA demodulation requirements and SDR requirements for 3MHz channel bandwidth, If PDSCH CA demodulation requirements for 3MHz channel bandwidth will be defined in Rel-19, corresponding configuration for CA CQI requirements in 38.101-4 Table 6.1.2-1 and Table 6.2A.3.1.1-1 should be updated accordingly.

Proposal 3: Prefer to update corresponding configuration for CA CQI requirements in 38.101-4 Table 6.1.2-1 and Table 6.2A.3.1.1-1 to include 3MHz, if PDSCH CA demodulation requirements for 3MHz channel bandwidth are agreed to be defined in Rel-19.

  - Simulation results

Simulation assumptions are as below according the agreements in RAN4#115 meeting [5].

Test parameters and simulation assumptions for 3MHz CC (if introduced) - <Agreement>: - Number of PRBs: 15PRBs - PDCCH aggregation level: If the 3MHz CC is PCell, reuse R18 PDCCH test configuration for 3MHz, i.e., AL4 (CCE 4 to 7) with 3 symbols. If it is SCell, - Receiver assumption: MMSE-IRC - Rank: 2 - Propagation condition: TDLA30-10 - MCS 13 - Cover both 2Tx2Rx and 2Tx4Rx with ULA Low correlation - For the other parameters: Follow the Common test parameters for CA in Table 5.2A-1 in TS38.101-4

Simulation results for PDSCH demodulation performance with FR1 FDD 3MHz as Figure 2-1.

Figure 2-1 PDSCH demodulation performance with FR1 FDD 3MHz

The simulation results for SNR (dB) at 70% max throughput with impairment margin could be summarized as

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3 | R.PDSCH.XX | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.74 |
| 3 | R.PDSCH.XX | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 6.75 |

- Conclusion

In this contribution, we provide our analysis and views on the NR channel BW less than 5MHz for FR1 Phase 2 topic.

The observations and proposals could be summarized as:

Observation 1: Less than 5MHz bandwidth only supported CA bandwidth combination are not defined in 38.101-1. It is hard to find opportunity to test the 3MHz case if follow existing test applicability rules for inter-band CA defined as Table 2-2.

Proposal 1: For less than 5MHz phase 2, consider introducing PDSCH CA demodulation requirements and update existing applicability rules for 3MHz.

Proposal 2: Prefer to update corresponding configuration for SDR requirements in Table 5.5A-1 and Table 5.5A-4 to include 3MHz, if PDSCH CA demodulation requirements for 3MHz channel bandwidth are agreed be defined in Rel-19.

Proposal 3: Prefer to update corresponding configuration for CA CQI requirements in 38.101-4 Table 6.1.2-1 and Table 6.2A.3.1.1-1 to include 3MHz, if PDSCH CA demodulation requirements for 3MHz channel bandwidth are agreed to be defined in Rel-19.

- Reference

[1] RP-250463, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, RAN#107 meeting, Intel Corporation

[2] RP-250462, Status Report for NR channel BW less than 5MHz for FR1 Phase 2, RAN#107 meeting, Intel Corporation

[3] RP-250626, RAN4 CRs to Rel-19 NR channel BW less than 5MHz for FR1 Phase 2, RAN#107 meeting

[4] RP-221496, Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1), RAN Meeting #96, Huawei, HiSilicon

[5] R4-2504754, Way Forward for [114bis][326] NR_less_than_5MHz_Ph2_demod, RAN4#114bis meeting, China Telecom