3GPP TSG-RAN WG4 Meeting #115	R4-2506677

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.16.3

Source:	Samsung

Title:	Simulation results on spatial channel model

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#109 in Sep 2025 [2]. The objectives are as below:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In RAN4#113-RAN4#114bis meetings, parameter assumptions, CDL methodologies and AAV configurations has been agreed for simulations [3][4][5].

In this contribution, we share our simulation results for CDL-based model, TDL extended model and baseline TDL channel model.

- Simulation assumptions

  - Agreed simualtion assumptions

For CDL based channel modeling, we combine the simulation assumptions argeed from RAN4#113 to RAN4#114bis meetings [3][4][5] as below.

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |  | MU-MIMO PDSCH (FR1) |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  | 40MHz |  |  |  |
| Subcarrier spacing |  | 30kHz |  |  |  |
| Duplex mode |  | TDD |  |  |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |  |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |  |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |  | IRC, E-IRC |
| Number of Tx Ports |  | 8,4 | 8 |  | 4 |
| Number of Rx Ports/Antennas |  | 8,4 | 4 |  | 4 |
| Number of layers |  | 8,4 | 4,2 |  | 2+2 |
| PMI |  | Random, FFS Fixed | Follow, Random |  | UE1: Random UE2: Random and not equal to UE1, Orthogonal |
| Waveform |  | CP-OFDM with normal CP |  |  |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |  |  |
| MCS |  | 13 (64 QAM table) |  |  |  |
| PDSCH configuration | Mapping type | Type A |  |  |  |
|  | k0 | 0 |  |  |  |
|  | Starting symbol (S) | 2 |  |  |  |
|  | Length (L) | 12 |  |  |  |
|  | PDSCH aggregation factor | 1 |  |  |  |
|  | Resource allocation type | Type 0 |  |  |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |  |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |  |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |  |  |
|  | Number of additional DMRS | 1 |  |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |  |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |  | type-I-SP |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) [ paramCombination-r16=6 (L=4, pν=1/2, 1/4, β=1/2) ] |  | (N1,N2,O1,O2) = (2,1,4,1) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD |  | For Random precoding: for every PRB Bundle (size=2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer | 8 CSI-RS Ports (4,1) 4 CSI-RS Ports (2,1) |  | TBD |
|  | Density (ρ) | 1 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (5) |  |  |  |
| CQI/RI/PMI delay |  | N/A | 7 ms | N/A |  |
| Number of HARQ Processes |  | 8 |  |  |  |
| Maximum HARQ transmissions |  | 4 |  |  |  |
| Channel Models |  | The purpose of the study item. |  |  |  |
| Testing Metric |  | Full Throughput Curves |  |  |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |  |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |  |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |  |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |  |  |
|  | BS Antenna Polarisation | (+45, -45) |  |  |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |  |  |
|  | UE Antenna Polarisation | (0, 90) |  |  |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |  |  |
|  | Antenna Panel Placement | YZ Plane |  |  |  |

  - Other parameters and configurations

Channel Model Configurations

For the channel model configurations, we use CDL option A2, CDL option A4, TDL cluster model 1 and baseline TDLC300 as below.

- CDL Option A2:  CDL (TR 38.753) with Table 7.2.1-8 in TR 38.901

- CDL Option A4:  CDL (TR 38.753) with Table 7.2.1-8 in TR 38.901 with truncation

- TDL cluster model 1

- Baseline: TDLC300 defined in TS38.101-4

AAV Configurations

For AAV configurations of CDL based channel modeling, below options are used for initial simulation

- Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

Doppler frequencies

- 10Hz

- 100Hz

Correlation matrix for baseline TDLC cases

- ULA Low

- X-Pol Medium

- X-Pol High

Extended TDL parameters

- Four clusters with power [0, 0, -9, -9] dB, phase shift angles [0, -π/2, -π, -3π/2]

- Simulation results

  - PDSCH demodulation performance for 4Tx4Rx4layer Doppler 10Hz configuration

Legacy TDL and 38.753 based CDL comparison

Firstly, 38.753 based CDL without truncation (named A2 below) is one of the down selected models, hence it is important to compare the simulation results between legacy TDL channel models and 38.753 based CDL channel model in Figure 3-1. For legacy TDL, three correlation matrixes are deployed including Low, XP Medium, XP High. For 38.753 based CDL, AAV model option 3 and option 1Y with AAV gain (including AE pattern) normalized are simulated.

Figure 3-1 4T4R4Layer PDSCH demodulation performance comparison between legacy TDL and 38.753 based CDL

From Figure 3-1, we have below observations:

- Peak throughput is hard to reach for legacy TDLC300-10 with XP High correlation matrix.

- 38.753 based CDL option 1Y has similar performance as option 3 with AAV gain (including AE pattern) normalized.

- 38.753 based CDL options have similar PDSCH demodulation performance as legacy TDL300-10 XP Medium configuration.

38.753 CDL A2 (without truncation) and A4 (with truncation) comparison

According our discussion in last RAN4#114bis meeting, the truncation solution A4 is selected as one of 38.753 CDL solution. Therefore, the performance comparison between A2 and A4 should be analysed in order to facilitate further down selection.

Figure 3-2 4T4R4Layer PDSCH demodulation performance comparison between A2 and A4

From Figure 3-2, we have below observations:

- For 4T4R 4layer PDSCH demodulation performance, there are almost no performance difference observed for 753 CDL A2 and A4.

Legacy TDL and Extended TDL comparison

The simulation results for PDSCH demodulation performance comparison between legacy TDL and extended TDL models as Table 3-1.

Table 3-1 Simulation results for PDSCH 4T4R4Layer

| Metric (SNR [dB] @% Max TP) | TDLC300-Low | TDLC300-High | Extended TDL cluster model 1 with high corr |
| --- | --- | --- | --- |
| 30% | 5.5 | 12.1 | 17 |
| 70% | 13.5 | 27.5 | 26.5 |

From Table 3-1, we have below observations:

- Extended TDL with 4 clusters and high correlation has similar performance as legacy TDLC300-XPHigh case.

  - PDSCH demodulation performance for 8Tx8Rx8layer Doppler 10Hz configuration

38.753 CDL A2 (without truncation) and A4 (with truncation) comparison

According our discussion in last RAN4#114bis meeting, the truncation solution A4 is selected as one of 38.753 CDL solution. Therefore, the performance comparison between A2 and A4 should be analysed in order to facilitate further down selection. The SNR working points comparison between A2 and A4 are summarize in Table 3-2.

Table 3-2. SNR working points comparison between A2 and A4 for 8T8R8Layer

| Metric (SNR [dB] @% Max TP) | AAV option 3 |  |  |  | AAV option 1Y |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A2 |  | A4 |  | A2 |  | A4 |  |
|  | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 |
| 30% | 9.1 | 9.1 | 8.7 | 8.8 | 9.1 | 9.1 | 8.6 | 8.2 |
| 70% | 22.9 | 22.4 | 19.9 | 19.2 | 23.1 | 22.7 | 22.2 | 21.8 |

From Table 3-2, we have below observations:

- For 8T8R 8layer PDSCH demodulation performance, there are obvious performance difference observed for 753 CDL A2 and A4, e.g., ~3dB for AAV option3 and 1dB for AAV option1Y.

Extended TDL comparison

The simulation results for 8T8R8Layer PDSCH demodulation performance as Figure 3-3 and the SNR working points could be summarized as Table 3-3.

Figure 3-3 8T8R8Layer PDSCH demodulation performance of Extended TDL model

Table 3-3 Simulation results for PDSCH 8T8R8Layer

| Metric (SNR [dB] @% Max TP) | CW0 | CW1 |
| --- | --- | --- |
| 30% | 5.9 | 7 |
| 70% | 14.8 | 16.4 |

From Figure 3-3 and Table 3-3, we have below observations:

- For 8T8R 8layer PDSCH demodulation performance, Extended TDL with 4 clusters could perform well.

  - PMI reporting performance for 8Tx4Rx2layer Doppler 10Hz configuration

38.753 CDL A2 (without truncation) and A4 (with truncation) comparison

For 8Tx4Rx2Layer results in Figure 3-4, the PMI reporting performance are compared between 38.753 CDL A2 (without truncation) and A4 (with truncation), for single panel Type I codebook and eType II codebook. The SNR working points could be summarized as Table 3-4.

Figure 3-4 PMI reporting performance comparison between 38.753 CDL A2 and A4

Table 3-4 The SNR working points comparison between A2 and A4

| Metric (SNR [dB] @% Max TP) | Type-I |  |  |  | eType-II |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AAV option 3 |  | AAV option 1Y |  | AAV option 3 |  | AAV option 1Y |  |
|  | A2 | A4 | A2 | A4 | A2 | A4 | A2 | A4 |
| 70% | 4 | 4.1 | 2.8 | 2.4 | 2.3 | 3.5 | 2.3 | 3.5 |
| 90% | 6.3 | 6.8 | 4.7 | 3.9 | 3.8 | 5 | 3.8 | 4.9 |

From Figure 3-4 and Table 3-4, the observations could be summarized as below

- For 8T4R2Layer PMI performance, obvious performance difference could be observed between 38.753 CDL A2 and A4, especially for eType-II codebook.

PMI reporting performance of Extended TDL

The PMI reporting performance of extended TDL model with XP High correlation as Figure 3-5. From Figure 3-5, the observations could be summarized as below

- For 8T4R2Layer with extended TDL300-10 and XP High correlation, the performance of eType II codebook outperform Type I codebook.

Figure 3-5 PMI reporting performance of extended TDL model

  - PMI reporting performance for 8Tx4Rx4layer Doppler 10Hz configuration

38.753 CDL A2 (without truncation) and A4 (with truncation) comparison

For 8Tx4Rx4Layer results in Figure 3-6, the PMI reporting performance are compared between 38.753 CDL A2 (without truncation) and A4 (with truncation), for single panel Type I codebook and eType II codebook. The SNR working points could be summarized as Table 3-5.

Figure 3-6 PMI reporting performance comparison between 38.753 CDL A2 and A4

Table 3-5 The SNR working points comparison between A2 and A4

| Metric (SNR [dB] @% Max TP) | Type-I |  |  |  | eType-II |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AAV option 3 |  | AAV option 1Y |  | AAV option 3 |  | AAV option 1Y |  |
|  | A2 | A4 | A2 | A4 | A2 | A4 | A2 | A4 |
| 70% | 9.7 | TBD | 9.8 | 9.9 | 13.4 | 15.3 | 13.4 | 15.1 |
| 90% | 12.4 | TBD | 11.7 | 11.8 | 15.6 | 17.5 | 15.2 | 16.8 |

From Figure 3-6 and Table 3-5, the observations could be summarized as below

- For 8T4R4Layer PMI performance, obvious performance difference could be observed between 38.753 CDL A2 and A4 with eType-II codebook, the difference of SNR working points is ~2dB.

PMI reporting performance of Extended TDL

Figure 3-7 PMI reporting performance of Extended TDL model

The PMI reporting performance of extended TDL model with XP High correlation as Figure 3-7. From Figure 3-7, the observations could be summarized as below

- For 8T4R4Layer with extended TDL300-10 and XP High correlation, the performance of eType II codebook outperform Type I codebook.

- For 8T4R4Layer with extended TDL300-10 and XP High correlation, the performance of Type I codebook is even worse than random PMI case at some SNR range.

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] RP-250577, Revised SID: Study on spatial channel model for demodulation performance requirements, RAN#107 meeting, Nokia, BT Plc

[3] R4-2419782, Way Forward for [113][320]NR_SCM, RAN4#113 meeting, Nokia

[4] R4-2502378, Way Forward for [114][322]NR_SCM, RAN4#114 meeting, Nokia

[5] R4-2504756, Way Forward for [114bis][320] NR_SCM, RAN4#114bis meeting, Nokia

[6] R4-2504762, Simulation assumptions for SCM, RAN4#114bis meeting, Huawei, HiSilicon, MediaTek