3GPP TSG-RAN WG4 Meeting #115							         		     R4-2506685

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	6.3.3	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 2 DL with up to 2UL (NR_CADC_R19_2BDL_xBUL)	[NR_CADC_SUL_R19-Core]

Title: 	On CA_n28-n71 2UL operation

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In R4#114b 2UL support was requested on top of the existing 1UL CA_n28-n71 combination. This challenging LBLB combination was flagged, and issues were captured in the way forward [1]. In this contribution, we provide our position on this 2UL challenge and make a proposal for a way forwards.

# Discussion

## 2.1 Way Forward input

Way Forward [1] provided the options to be studied for CA_n28-n71 2UL reproduced here in italics.

Proposal 1: RAN4 can investigate the following RF architectures in order to enable the UL CA_n28-n71.

Option 1: Three-antenna RF architecture below as an example. (NOTE: it isn’t precluded to implement Full band duplexer for band n28.)

Option 2: Two-antenna RF architecture with two triplexers.

Option 3: Other RF architectures are not precluded, e.g. studies in TR38.872 on CA_n28-n105.

Proposal 2: RAN4 can investigate the impacts on the following RF requirements in order to enable the UL CA_n28-n71.

- The attenuation between UL band n71 and UL band n28.

- MSD for both DL band n28 and n71 due to IMD3 and IMD5.

- Potential revisit on the existing MSD due to cross band isolation.

- Delta Tib and Rib

- MPR requirements based on the investigation whether intermodulation interference will excceed the SEM or spurious emission requirements.

- The band protection requirements of -50dBm/MHz.

NOTE: Whether to specify 2UL is based on the outcome of the studies.

## 2.2 CA_n28-n71 2UL

In Release 17, RAN4 studied thoroughly many LBLB combinations including combinations very similar to CA_n28-n71 like CA_n28-n105 and CA_n71-n85. Even with multiple companies in the study it was concluded that 2UL could not be supported in these cases due to:

- The small or inexistent gap between the two UL

- The large MSD due to IMD3/5 falling in both DL bands.

And thus, as of today there is no LBLB combinations supporting 2UL when in close proximity.

CA_n28-n71 with 2UL suffer from the same issues and DC_28-n71 has been introduced with SUO only. Thus, we do not see a reason to make an exception and introduce 2UL for CA_n28-n71. Additionally, CA_n28-n71 1UL BCS0 is already specified with the same CBW support as the new request and the associated cross-band MSDs as shown below with extracts of the BCS and MSDs from 38.101-1. The only new aspect is the 2UL support:

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n71A | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n71 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 13.3 | ACLR2 |
| n71 | n28 | 688 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | 6.5 | >ACLR2 |

In terms of implementation, for the specified cross-band MSD, like for other LBLB cases, a two-antenna architecture with dual triplexers and assume the reuse of a CA_n28-n105 and full Band n28 implementation was assumed:

- Antenna 1 with a n105DL+n105UL+n28DL triplexer

- Antenna 2 with a n105DL+n28UL+n28DL triplexer.

It should be noted that n105 and n28 full band duplexers are the most challenging LBLB technically and triplexing with the other band DL only increases the challenge, and thus moderate isolation into the other band DL must be expected.

Consequently, 1UL MSD is in the order of 6 to 13dB and low order IMD 2UL MSD is probably >40dB resulting in a 2UL overall performance being lower than 1UL.

A better approach to the support of 2UL for better balancing of the bands would be to implement FDD-FDD LBLB CA based on switching as proposed for Release 20.

Proposal for CA_n28-n71:

- Baseline architecture for MSD evaluation is a dual-antenna with dual triplexers.

- 2UL CA_n28-n71 operation is not specified in Release 19 and the specified 1UL configuration should be used.

- 2UL CA_n28-n71 is part of the Release 20 FDD-FDD LBLB CA based on switching.

# Conclusion

In this contribution, we have discussed the 2UL operation aspect of CA_n28-n71 and have the following proposal.

Proposal for CA_n28-n71:

- Baseline architecture for MSD evaluation is a dual-antenna with dual triplexers.

- 2UL CA_n28-n71 operation is not specified in Release 19 and the specified 1UL configuration should be used.

- 2UL CA_n28-n71 is part of the Release 20 LBLB FDD-FDD CA based on switching.

# References

[1] R4-2505216 WF for uplink CA n28-n71, Huawei, R4#114b