3GPP TSG-RAN WG4 Meeting #115	R4-2506702

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The enhancement on FR2-1 L3 measurement by optimizing Rx beam sweeping factor was further discussed in last meeting with agreements and issues captured in [1] and an LS [2] is sent to RAN2 about the RAN4 agreements so far. In this paper, we further provide our views on remaining issues.

# Discussion

The agreements achieved in last meeting are summarized as follows:

### Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in RRM session (Monday): - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell. - Leave the decision to RAN2. Further discuss the details for the LS to RAN2, and the LS can only be approved if the content is agreeable. - Details of the LS can refer to R4-2504962. ### Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement in the ad-hoc: - Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4. ### Issue 1-2-3: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF Agreement in the ad-hoc: - For each scenario for PC3 UE, the described factors in the table should be replaced with capability value. Scenarios; Factors. 1; Mpss/sss_sync_w/o_gaps = [reduced N]*3 Mmeas_period_w/o_gaps = [reduced N]*3. 2; Mpss/sss_sync_with_gaps = [reduced N]*3 Mmeas_period_with_gaps = [reduced N]*3. 3; Mpss/sss_sync_inter = [reduced N]*3 Mmeas_period_inter = [reduced N]*3 MSSB_index_inter = [reduced N]*3. 4; Mpss/sss_sync_inter = [reduced N]*5 Mmeas_period_inter = [reduced N]*5 MSSB_index_inter = [reduced N]*3. 5; If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then N in the HO requirement will be replaced by [reduced N] when the target cell is in FR2-1 and the PC3 UE supports [Rel-19 Rx FBS].. 7; RRC Re-establishment/redirection Can be discussed directly in the draft CR..

The issues about whether to introduce quit conditions are not discussed in last meeting. We provide our analysis on the necessary in this contribution. Regarding whether and how to define a condition to trigger UE to quit multiple panel L3 measurement, there is discussion but without agreements. The discussion could be found in the summary [3]. The reason why the conditions is important is that the power consumption in FR2 with multiple panel is extremely high compared with FR1 measurement. We should try to avoid UE is force to perform multiple panel measurement forever. We can find the evaluation about UE power consumption in TR 38.840

#### 8.1.4.1 For intra-frequency measurements Table 22: UE power consumption for the RRM measurements N: Number of cells for intra-frequency measurement; Synchronous case; Asynchronous case. FR1; FR2; FR1; FR2. N=8; 150; 225; 170; 285. N=4; 120; 195; 140; 255. - All above values are slot-averaged power (Pfr1 or Pfr2 for FR1 or FR2 respectively). - Synchronous case means actual SSB transmissions from cells are time-aligned e.g., timing of SSBi from celli is aligned with timing of SSBi from cellj. - Asynchronous case means actual SSB transmissions from cells are not time-aligned e.g., timing of SSBi from celli is not aligned with timing of SSBi from cellj - The following maximum number of cells for intra-frequency measurement within a slot: Table 23: Maximun numer of cells for intra-frequency measurements FR1; FR2. Nmax = 19; Nmax = 9. For combined measurement and search: To obtain combined neighboring cell measurement and search power, add the difference between full neighboring cell search state power and SSB/CSI-RS proc. state power to the neighboring cell measurement power, as shown below: Table 24: UE power consumption of the combined neighbor cell measurements and cell search N: Number of cells for intra-frequency measurement & search; Synchronous case; Asynchronous case. FR1; FR2; FR1; FR2. N=8; 200; 320; 220; 380. N=4; 170; 290; 190; 350.

If we look at the power consumption model for intra-frequency measurement, the power consumption per slot for RRM measurement for FR2 is 1.5~1.85 times of that of FR1. Then, ratio is 6~7.4 times per second (30KHz FR1 and  120KHz FR2). Then, if we consider UE activating multiple panels for FR2 measurement, this ratio will be at least doubled since the most power consuming component is the RF part.

Observation 1: The UE power consumption of FR2 RRM measurement is several times more than that of FR1 RRM measurement. The value will be even larger (10+ times) when activating multiple panels simultaneously.

From under understanding, it is essential to have a quit method from UE perspective. If the activation condition is only based on serving cell quality (e.g. RSRP and/or RSRQ threshold). If UE moves to the cell edge and stops for a long time, UE will activate multiple panels for fast L3 measurement for a long time. Another reason is that, the channel condition may vary from UE to UE. For instance, the threshold conditions are met due to some blocking but the UE is still in the cell centre. It does not necessarily require to UE to perform FBS all the time.

Observation 2: Based on agreed triggering condition (serving cell’s signal quality), it is possible that UE will always activating multiple panels for L3 measurement, which is problematic for at least following case:

- Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving.

- Case 2: UE is still in the cell centre and the triggering conditions are met due to block.

If without UE quit conditions, the above cases can not be avoided. Then most likely that the consequence will be UE will not support this feature. The reason is that how fast/how many Rx beams are used for RRM measurement is UE implementation issues and gNB does not needs to know. For instance, though the beam sweeping factor is defined as 8 in the spec, UE can use smaller beam sweeping factor for better performance since Rel-15. In other words, if the above case cannot be avoided, UE can choose not to support the feature but to speed up the L3 measurement by activation multiple panels by implementation with the balance between fast measurement and power consumption.

Observation 3: UE can choose not to support the feature but to speed up the L3 measurement by activation multiple panels by implementation with the balance between fast measurement and power consumption.

Proposal 1: UE autonomous quitting should be supported in Rel-19.

Based on the online/offline discussion, values comments are received that some cases are still unsolved. The questions are recorded as follows:

- Q1: For case#1, if UE is in the cell edge and stops, UE shall not quit fast measurement forever as UE may starting moving again at the cell edge.

- Q2: For case#2, UE will not have qualified neighbour cell measurement results, then UE is till performing fast measurement.

Based on the discussion and proposals in last meeting, we think a timer could be considered which can help solve the issues mentioned above. For instance, if the UE has performed fast measurement for at least T1, UE is allowed to fall back to normal measurement, which can solve the issue mentioned in Q2. And then, if UE quits the fast measurement for power saving, it shall reactivate fast measurement within T2, which can solve the issue mentioned in Q1.

Observation 4: A timer can solve the issues as mentioned in Q1 and Q2.

Based on the analysis above, details of UE autonomous quitting condition are proposed as follows:

- UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 OR UE has reported qualified measurement results.

- After quitting fast measurement, UE shall re-activated fast measurement no longer than T2 from the quitting time.

Proposal 2: Details of UE autonomous quitting condition are proposed as follows:

- UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 OR UE has reported qualified measurement results.

- After quitting fast measurement, UE shall re-activated fast measurement no longer than T2 from the quitting time.

Another issue is to discuss the details of UE capabilities. It is agreed to introduce the UE capability of reduced N as {2,4,6} in last meeting. From our understanding, supporting these features requires additional UE capability for both RF and baseband resources. And the capacity of RF/baseband resource varies for different band combinations. As clarified in last meeting, the applicable scenario is single FR2-1 PCell with one carrier in FR2-1 for L3 measurement [4]. There are following cases to be considered:

- Case 1: Intra-band infra-frequency: PCell and target carrier for L3 measurement are in the same band and intra-frequency.

- Case 2: Intra-band inter-frequency: PCell and target carrier for L3 measurement are in the same band and inter-frequency.

- Case 3: Inter-band: PCell and target carrier for L3 measurement are in different bands and inter-frequency.

From our understanding, case 1 is the most typical case as it is more likely that UE is configured with one carrier for L3 measurement for one band. Case 2 is less realistic than case 1 in the real deployment. As for case 3, it is also not the typical cases that gNB trigger the fast measurement on one band based on the quality of another band as the coverage of these two bands could be different. And from UE implementation perspective, enabling the feature for inter-frequency measurement will require more baseband resources.

Proposal 3: For the feature FR2-1 L3 measurement delay by optimizing, for scenario 1/2/3/4, define separate UE capabilities for intra-frequency measurement and inter-frequency measurement with the granularity of “per band”.

The details of the proposed UE features could be found in the table in the annex.

# Conclusions

Observation 1: The UE power consumption of FR2 RRM measurement is several times more than that of FR1 RRM measurement. The value will be even larger (10+ times) when activating multiple panels simultaneously.

Observation 2: Based on agreed triggering condition (serving cell’s signal quality), it is possible that UE will always activating multiple panels for L3 measurement, which is problematic for at least following case:

- Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving.

- Case 2: UE is still in the cell centre and the triggering conditions are met due to block.

Observation 3: UE can choose not to support the feature but to speed up the L3 measurement by activation multiple panels by implementation with the balance between fast measurement and power consumption.

Proposal 1: UE autonomous quitting should be supported in Rel-19.

Observation 4: A timer can solve the issues as mentioned in Q1 and Q2.

Proposal 2: Details of UE autonomous quitting condition are proposed as follows:

- UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 OR UE has reported qualified measurement results.

- After quitting fast measurement, UE shall re-activated fast measurement no longer than T2 from the quitting time.

Proposal 3: For the feature FR2-1 L3 measurement delay by optimizing, for scenario 1/2/3/4, define separate UE capabilities for intra-frequency measurement and inter-frequency measurement with the granularity of “per band”.

# References

[1] R4-2504900 WF on RRM requirements for NR_RRM_Ph5_Part2

[2] R4-2504962 LS on Rx BSF optimization for NR RRM Ph5

[3] R4-2415656 Topic summary for [112bis][209] NR_RRM_Ph5_Part1

[4] R4-2414026 WF for NR_RRM_Ph5_Part1

# Annex

# NR_RRM_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_RRM_Ph5 | 51-1 | FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor | Support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor for: - SSB based Intra-frequency measurement without MG - SSB based Intra-frequency measurement with MG - Handover - RRC Re-establishment - RRC Connection Release with Redirection | n/a | Y | n/a | UE does not meet the requirements of FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Per Band | n/a | The feature only applies to FR2-1 | n/a | Candidate value of N in L3 measurement delay requirements: {2,4,6}. The feature applies to the scenario when UE is configured with only one FR2-1 Cell (i.e. FR2-1 PCell without CA/DC) and is configured with only one FR2-1 carrier for L3 SSB measurement. | Optional with capability signalling |
|  | 51-2 | FR2-1 inter-frequency L3 measurement delay reduction by optimizing Rx beam sweeping factor | Support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor for: - SSB based Inter-frequency measurement without MG - SSB based Inter-frequency measurement with MG | 51-1 | Y | n/a | UE does not meet the requirements of FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Per Band | n/a | The feature only applies to FR2-1 | n/a | Candidate value of N in L3 measurement delay requirements: {2,4,6}. The feature applies to the scenario when UE is configured with only one FR2-1 Cell (i.e. FR2-1 PCell without CA/DC) and is configured with only one FR2-1 carrier for L3 SSB measurement. | Optional with capability signalling |