3GPP TSG-RAN WG4 Meeting # 115	R4-2506707

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.27.3.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES on adaptation

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN4 discussed the RRM impacts of SSB adaption in last meeting with agreements and issues captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

In last RAN1 meeting, RAN1 further achieved following agreements and progress, which are summarized as follows:

Agreement For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell, - The DCI is scrambled a new RNTI, - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell - the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter - the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell Agreement For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell. - Xmax=2 Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst

Based on latest RAN1 agreements, it could be observed that at most two additional SSB burst periodicity could be configured for an SCell. Regarding how determine the first SSB burst with new SSB periodicity, RAN1 will further discuss it.

The agreements and progress in last RAN2 meetings are summarized as follows:

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Based on above RAN1/2 progress, we provide our views on remaining issues in following part.

## SSB adaptation for L1 measurement

For L1 measurement, RAN4 achieved following agreements in last meeting:

Issue 1-2-1: L1 requirements at transition Agreement - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline. - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs.

Based on the latest progress in RAN1, RAN1 is still discussing timeline about SSB adaptation on determining the first SSB burst position with new SSB periodicity. Following TP are proposed for impacted L1 requirements (taking L1-SINR as example)

---------------------------------------------------------- Start Text proposal ------------------------------------------------------------

### 9.8.7	Minimum requirement at transitions

For UE supporting ncd-SSB-BWP-Wor-r18, when the L1-SINR measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching during one measurement period, the UE shall use a L1-SINR measurement period that is the maximum of the L1-SINR measurement period corresponding to the first SSB type and the second SSB type after the BWP switching.

Subsequent to the above-mentioned measurement periods, the L1-SINR measurement period requirements are based on the SSB type after the BWP switch.

For UE supporting [UE capability for SSB adaptation], when UE receives DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission, the L1-SINR measurement period requirements are based the indicated SSB burst periodicity which applies from the first SSB burst as defined in TBD.

-----------------------------------------------------------End Text proposal ------------------------------------------------------------

Proposal 1: For L1 requirements impacts for L1-RSRP/L1-SINR/CBD, adopt above TP to define requirements at transitions.

Another issue to be clarified is about whether to differentiate the case w.r.t DRX configuration. From our understanding, there is no reason to change the legacy principle when DRX is configured. In other words, if DRX is configured, UE still follow legacy DRX based requirements. The same principle is also adopted in OD-SSB discussion with following agreements.

Issue 1-4-4: L1 measurement after SCell activation when DRX is configured Agreement - For Scenario 2 and Scenario 2a, when DRX is configured, following the legacy requirements based on DRX with the consideration of OD-SSB is not deactivated.

Proposal 2: For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period.

Another issue we would like to discuss is the potential impacts on measurement restrictions. Taking L1-RSRP as example, when SSB-based L1-RSRP and CSI-RS are overlapped, UE is only required to measure one of them.

#### 9.5.5.1 Measurement restriction for SSB based L1-RSRP For FR1, when the SSB for L1-RSRP measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, - If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction; - If SSB and CSI-RS have different SCS, - If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction; - If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. For FR2, when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined. For FR2, if the network configures same or mixed numerology between SSB for L1-RSRP measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair. #### 9.5.5.2 Measurement restriction for CSI-RS based L1-RSRP For both FR1 and FR2, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB. For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement without restrictions. For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities: - If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions. - If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined. For FR1, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction. For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

Based on existing requirements, it is up to UE to choose either SSB or CSI-RS for measurement. However, for SSB adaption (maybe the same situation for OD-SSB), SSB are occasionally triggered by MAC-CR or DCI. Since the Cell specific CSI-RS is configured based on normal SSB pattern without collision, the newly triggered the SSB may colliding the CSI-RS and gNB may not guarantee that both SSB and CSI-RS are available to UE when they are overlapping. Thus, if UE choose CSI-RS for measurement, it may cause wrong measurement results (e.g. wrong L1-RSRP results/ wrongly triggered BFD). Thus, it should be discussed whether legacy measurement restriction can apply when adapted SSB colliding with CSI-RS.

Observation 1: The occasionally triggered SSB adaption (maybe also for OD-SSB) may collide pre-defined Cell specific CSI-RS, and gNB may not be able to guarantee that both SSB and CSI-RS are available to UE. Per existing requirements, if UE chooses to measurement CSI-RS, it may cause wrong UE behaviour.

Proposal 3: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

## SSB adaptation for L3 measurement

For L3 measurements, RAN4 achieved following agreements in last meeting,

Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations.

Based on the agreements in previous RAN2 agreements, it could be observed that UE will select one of configured SMTC accordingly upon reception of DCI format 2_9, which is mapped with the indicated SSB burst periodicity.

Observation 2: Based on RAN2 agreements, UE will select one of configured SMTC accordingly upon reception of DCI format 2_9, which is mapped with the indicated SSB burst periodicity.

For L3 measurement impacts, same principle as L1 requirements shall be considered which is clearer on expected UE behaviour. Thus, following TP are propose for minimum requirements at transitions.

---------------------------------------------------------- Start Text proposal ------------------------------------------------------------

### 9.1.6	Minimum requirement at transitions

When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply.

When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within NCSG or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply.

The carrier-specific scaling factor specified in clause 9.1.5 that applies to the other impacted measurement objects will also apply based on the longer measurement or cell identification delay before or after the transition.

When the UE transitions between DRX and non-DRX or when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

Subsequent to this measurement period, the cell identification and measurement period requirements on each measurement object are corresponding to the second mode after transition.

For UE supporting ncd-SSB-BWP-Wor-r18, when the intra-frequency measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching or SCG activation/deactivation during one cell identification period or measurement period, the UE shall use a cell identification period that is the maximum of the cell identification periods or measurement periods corresponding to the first SSB type and the second SSB type after the BWP switching or SCG activation/deactivation.

Subsequent to the above-mentioned measurement periods and/or cell identification periods, the cell identification and measurement period requirements on each measurement object are based on the SSB type after the BWP switch or SCG activation/deactivation.

For UE supporting [UE capability for SSB adaptation], when UE receives DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission, the cell identification and measurement period requirements apply based on the SMTC which is mapped with the indicated SSB burst periodicity from the first SMTC after the first SSB as defined in TBD.

-----------------------------------------------------------End Text proposal ------------------------------------------------------------

Proposal 4: For L3 requirements impacts for SSB adaption, adopt above TP to define requirements at transitions.

## SSB adaptation for SCell activation

Based on latest progress on SSB adaption scenario, it is almost the same as OD-SSB which is only for SCell, and at least SSB periodicity adaption is supported. Based on that, RAN4 achieves following agreements in previous meeting:

Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements Agreement: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - FFS Rel-18 SCell activation with L3 reporting - FFS Multiple SCells activation

However, one thing needs to be clarified is that what is the condition for “SSB adaptation command comes together with SCell activation command”. Since RAN2 has concluded that RAN2 will not introduce MAC CE for SSB adaption, thus the only way for SSB adaption is DCI format 2_9. It is not clear what is the condition for a MAC CE and DCI by saying they come together. This is something different from OD-SSB when both SCell activation and OD-SSB activation are triggered by MAC CE. Considering that DCI decoding is much faster than MAC CE decoding, UE shall be use new SSB periodicity for SCell activation when DCI is received at the same slot as MAC CE. As mentioned above, since they are messages from different layers, it is too restricted to say that there are received from the same slot. When the DCI is received early than SCell activation MAC CE or RRC (for direct SCell activation), UE shall also use the new SSB periodicity. One may comment that if the SSB adaption is long before SCell activation, UE has no ideas about the SSB adaption is for fast SCell activation. Then, it is reasonable to also limited that the DCI received no early than X ms before SSB activation command.

Proposal 5: For SCell activation requirements based on the SSB periodicity after SSB adaptation, the requires apply the slot m when UE receives DCI format 2_9 for SSB adaption is no later than the slot n when UE receives SCell activation command (MAC CE or RRC), and no early than X ms before slot n.

For the applicable SCell activation procedure, it is still FFS whether to define requirements Rel-18 SCell activation with L3 reporting. Based on conclusion in previous meeting, the impacts on SCell activation are common, which is to activate the SCell with the indicated SSB periodicity. Thus, the requirements for Rel-18 SCell activation with L3 reporting shall also be defined.

Proposal 6: SCell activation requirements based on the SSB periodicity after SSB adaptation also apply to Rel-18 SCell activation with L3 report.

# Conclusions

Proposal 1: For L1 requirements impacts for L1-RSRP/L1-SINR/CBD, adopt above TP to define requirements at transitions.

Observation 1: The occasionally triggered SSB adaption (maybe also for OD-SSB) may collide pre-defined Cell specific CSI-RS, and gNB may not be able to guarantee that both SSB and CSI-RS are available to UE. Per existing requirements, if UE chooses to measurement CSI-RS, it may cause wrong UE behaviour.

Proposal 2: For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period.

Proposal 3: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

Observation 2: Based on RAN2 agreements, UE will select one of configured SMTC accordingly upon reception of DCI format 2_9, which is mapped with the indicated SSB burst periodicity.

Proposal 4: For L3 requirements impacts for SSB adaption, adopt above TP to define requirements at transitions.

Proposal 5: For SCell activation requirements based on the SSB periodicity after SSB adaptation, the requires apply the slot m when UE receives DCI format 2_9 for SSB adaption is no later than the slot n when UE receives SCell activation command (MAC CE or RRC), and no early than X ms before slot n.

Proposal 6: SCell activation requirements based on the SSB periodicity after SSB adaptation also apply to Rel-18 SCell activation with L3 report.

# References

[1] R4-2502689 WF on RRM requirements for Netw_Energy_NR_enh_Part2

[2] R2-2501484 Reply LS on SSB adaptation

[3] R4-2415663 Topic summary for [112bis] [216] Netw_Energy_NR_enh_Part2

[4] Reply LS on SSB adaptation