3GPP TSG-RAN WG4 Meeting # 115	R4-2506710

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.27.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for Rel-19 NES

Document for:	Discussion

# Introduction

For Rel-19 NES, the performance requirements discussion shall start from RAN4#115. In this contribution, we provide our views on the performance impacts on relevant objectives.

# Discussion

For the three relevant objectives, we provide our views in a case by case manner. Considering that this is the first meeting for performance part discussion, the contribution is mainly about the new introduced test cases.

## OD-SSB

For OD-SSB, per progress in core requirement discussion, the proposed test case lists are as follows:

Table I. Test cases list for OD-SSB

| TC | Case#1/Case#2 | FR1/FR2 | Remarks |
| --- | --- | --- | --- |
| Deactivated SCell measurement | Case#1 | Both FR1 and FR2 |  |
|  | Case#2 | Both FR1 and FR2 |  |
| SCell activation based on OD-SSB | Case#1 | Both FR1 and FR2 | FFS whether to consider both known and unknown case |
|  | Case#2 | Both FR1 and FR2 |  |
| Direct SCell activation based on OD-SSB | Case#1 | Both FR1 and FR2 | FFS whether to consider both known and unknown case |
|  | Case#2 | Both FR1 and FR2 |  |
| PUCCH SCell activation based on OD-SSB | Case#1 | Both FR1 and FR2 | FFS whether to consider both known and unknown case |
|  | Case#2 | Both FR1 and FR2 |  |
| Note: For Case#2， FFS whether to consider the case when AO-SSB and OD-SSB are in different frequency carreirs. |  |  |  |

For OD-SSB, define TC as Table I and FFS whether to define TC for following requirements pending on core requirements discussion:

- FFS Multiple OD-SSB carriers for deactivated SCell measurement

- FFS OD-SSB parameter updating after SCell activation (i.e. scenario 3B)

- FFS OD-SSB based SCell activation, e.g. Rel-18 SCell activation with L3 reporting, multiple SCell activation.

Proposal 1: For OD-SSB, define TC as Table I and FFS whether to define TC for following requirements pending on core requirements discussion:

- FFS Multiple OD-SSB carriers for deactivated SCell measurement

- FFS OD-SSB parameter updating after SCell activation (i.e. scenario 3B)

- FFS OD-SSB based SCell activation, e.g. Rel-18 SCell activation with L3 reporting, multiple SCell activation.

Regarding the details of SSB/SMTC configurations, to verify the new behaviour/requirements introduced in Rel-19 NES, it is proposed to defined periodicity of OD-SSB, SMTC and measCycleSCell as 20ms/40ms/160ms respectively.

Proposal 2: Configure the periodicity of OD-SSB, SMTC and measCycleSCell as 20ms/40ms/160ms respectively.

## SSB adaption

For SSB adaption, per progress in core requirement discussion, the proposed test case lists are as follows. Based on core requirements discussion, there is also impact on L1/L3 measurement. For instance, RAN4 define requirements at transition for L1-RSRP/L1-SINR/CBD. However, if taking legacy TC as reference, there is no such TC for requirements at transition for L1/L3 measurement. Thus, we propose no TC for requirements at transition for L1/L3 for SSB adaptation.

Proposal 3: No TC for requirements at transition for L1/L3 for SSB adaptation as legacy.

Proposal 4: For SSB adaptation, define TC as Table II.

Table II: TC for SSB adaptation

| TC | FR1/FR2 | Remarks |
| --- | --- | --- |
| SCell activation with SSB adaptation | Both FR1/FR2 | Unknown case |
| Direct SCell activation with SSB adaptation | Both FR1/FR2 | Unknown case |
| PUCCH SCell activation with SSB adaptation | Both FR1/FR2 | Unknown case |

Proposal 5: Similar as OD-SSB, the SSB/SMTC periodicity configuration are as follows for SSB adaption:

- SSB periodicity: 40 ms to 20 ms

- SMTC periodicity: 40ms

## OD-SIB1

For OD-SIB1, two major impacts are identified: UL-WUS timing and paging interruption. In legacy cases, paging interruption requirements are not verified in a TC. However, TSI-NR may need to be updated for a TC with OD-SIB1.

Table III: TC for OD-SIB1

| TC | Remarks |
| --- | --- |
| Cell reselection to FR1 target Cell | UL-WUS timing are verified in the same TC and FFS TSI-NR |
| Cell reselection to FR2 target Cell | UL-WUS timing are verified in the same TC and FFS TSI-NR |

Proposal 6: For OD-SIB1, define TC as Table III.

# Conclusions

Proposal 1: For OD-SSB, define TC as Table I and FFS whether to define TC for following requirements pending on core requirements discussion:

- FFS Multiple OD-SSB carriers for deactivated SCell measurement

- FFS OD-SSB parameter updating after SCell activation (i.e. scenario 3B)

- FFS OD-SSB based SCell activation, e.g. Rel-18 SCell activation with L3 reporting, multiple SCell activation.

Proposal 2: Configure the periodicity of OD-SSB, SMTC and measCycleSCell as 20ms/40ms/160ms respectively.

Proposal 3: No TC for requirements at transition for L1/L3 for SSB adaptation as legacy.

Proposal 4: For SSB adaptation, define TC as Table II.

Proposal 5: Similar as OD-SSB, the SSB/SMTC periodicity configuration are as follows for SSB adaption:

- SSB periodicity: 40 ms to 20 ms

- SMTC periodicity: 40ms

Proposal 6: For OD-SIB1, define TC as Table III.

# References

[1] R4-2504907 WF on RRM requirements for Netw_Energy_NR_enh_Part2