3GPP TSG-RAN WG4 Meeting # 115	R4-2506736

Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on impacts for RRM measurement prediction

Source: 	Huawei, HiSilicon

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

The revised SID on AI/ML for mobility in NR agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

This contribution focuses on RRM measurement prediction in AI/ML mobility.

Discussion

In R18 study item phase on “AI/ML for NR air interface”, the general functional framework for AI/ML for NR air interface is illustrated in below figure (duplicated from TR38.843[4]). The functional architecture and high-level procedure can be regarded as a baseline for AI/ML aided mobility.

Figure 4.4-1: Functional framework for AI/ML for NR Air Interface [TR 38.843]

There is a parallel R19 WI on “AI/ML for NR air interface” which focus on use cases of CSI compression, beam management and positioning. As per the revised SID on AI/ML mobility [1], the work from “AI/ML for NR air interface” shall be leveraged as much as possible.  In RAN4 progress on R19 AI/ML for NR air interface, the following agreements are achieved in [3]. These agreements provide a good guidance on RRM core requirements definition. In the following, the RRM requirements for R19 AI/ML mobility are provided.

Issue 1-4: Inference latency handling Agreement: - Use the following as the starting point - Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements. - Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency. - Check the above bullets based on per use case, and if other WGs define the inference latency, RAN4 can revisit the above bullets. Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded Issue 1-6: Legacy RRM requirements handling Agreement: - UE shall meet all legacy RRM requirements (non-AI/ML) even during the AI/ML operation mode.

General aspects

In RAN2, the evaluation scenarios for measurement prediction with prioritization are listed in [TR38.744].

Table 5.2.1-1 prioritization of evaluation scenarios [TR38.744]

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

There are two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption) [5].

During RAN2#126 meeting, companies had sufficient discussion on the scenario priority with the target study goal. The final conclusion is to set scenario 6 (FR2 to FR2 intra-frequency spatial domain with the first goal) as the middle priority. We suggest RAN4 to follow RAN2 priority at the current stage, and then further analyse middle priority scenarios after sufficient progress on high priority scenarios. In addition, in R19 AI/ML interface, spatial domain predication is being carried out for beam management. Some kinds of methodology can be leveraged or inherited to AI/ML mobility scenario 6.

Proposal 1: RAN4 to focus on the discussion on scenario 2, 3 and 4 with high priority in this release.

Data collection requirements

Data collection function provides input data to model training, management and inference function. If new measurement quantity is introduced in AI/ML mobility, new requirements would be specified accordingly, otherwise no RRM requirements on data collection is observed.

Proposal 2: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed.

Inference requirements

Measurement requirements for RRM measurement prediction

For the below evaluation scenarios for measurement prediction in [TR38.744], Case A and case B in intra-frequency temporal domain are also defined in RAN2.

Table 5.2.1-1 prioritization of evaluation scenarios [TR38.744]

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Agreements in RAN2#126 Chairman notes 7 To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window 8 To agree on methodology of Intra_F_C_T_Case B: Intra-frequency intra-cell temporal domain prediction is done by predicting sub set measurement (case B) instances in temporal domain of the same cell for both FR1_to_FR1 and FR2_to_FR2. Several measurement reduction rates should be aligned among companies. The detail values for measurement reduction rate are FFS.

We know temporal domain case A and case B have different working mechanism. For both case A and case B, OW (observation window) and PW (prediction window) are introduced [5].

Figure 5.2.1-1 Example of intra-frequency temporal domain case A [TR38.744]

In intra-cell temporal domain prediction case A, observation window is a duration UE performs actual measurement. Within an observation window, the measurement period shall follow the legacy measurement period. The prediction window is a duration UE predicts measurement. Observation window and prediction window will slide. It means that the instances in previous prediction window would become part of the observation window when UE perform measurement in predicted instances. It shall be noted that the observation window length depends on model performance, channel environment and observation length, etc.

The sliding window in case A is similar as the evaluation period in legacy RLM requirement. From defining requirement perspective, a observation window length for predication (or called as prediction evaluation period) and prediction window length may need to be specified.

Proposal 3: For intra-frequency temporal domain prediction case A, observation window length (or called as prediction evaluation period) and prediction window length may need to be specified.

For scenario 4- FR2 intra-frequency temporal domain case A, RAN2 did some evaluation, and some observations are made [TR38.744]:

- Higher UE speed correlates with decreased prediction accuracy

- Longer PW length correlates with decreased prediction accuracy

- Increasing the OW length can improve the prediction accuracy, especially when the OW length is relatively short. However, once the OW length exceeds a certain value, further increase of the OW length does not yield significant benefit.

To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, we think limited doppler shift (corresponds to limited UE speed), limited PW length and proper OW length shall be set.

Proposal 4: To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing.

In FR1 intra-frequency temporal domain prediction case B, the measurement results in PW are predicted and these time instances are completely skipped by UE. RAN2 provide two examples duplicated as below, and example 2 is recommended as baseline for evaluation.

Figure 5.2.1-2 Example 1 of intra-frequency temporal domain case B [TR38.744]

Figure 5.2.1-3 Example 2 of intra-frequency temporal domain case B [TR38.744]

Measurement Reduction Rate in temporal domain (MRRT) is used for case B in RAN2 evaluation work [TR38.744].

MRRT = skipped measurement time instances / total measurement time instances

It is observed that,

- Under different MRRT (e.g., 1:1, 2:3), the prediction accuracy would be different. Increasing MRRT would result in decreased prediction accuracy.

- Regarding example1 in figure 5.2.1-2 and example 2 in Figure 5.2.1-3, MRRT for both examples are the same, that is 0.5. However the prediction error may be not the same, even with the same MRRT value. Taking an extreme example, following the predication strategy like example 1, using100 measurements in OW predict 100 measurements in PW, MRRT is still 0.5. However the prediction accuracy of the sample in the farthest instance in PW would be degraded. In other words, even under the same MRRT setting, different measurement skipping patterns can result in different prediction accuracy.

Therefore the accuracy requirements for intra-frequency temporal domain prediction case B shall be specified for certain MRRT with some limitations, e.g., PW length.

Proposal 5: For intra-frequency temporal domain prediction case B, RSRP prediction accuracy is to be defined under the side condition of MRRT with limited PW length.

For inter-frequency inter-cell frequency domain measurement, the idea is to leverage the RSRP result on one cell to infer the RSRP on another cell in another frequency. The prediction accuracy in frequency domain would be evaluated and defined in RAN4 RRM. With different channel correlation coefficient between two inter-frequency carriers, the measurement prediction accuracy is different. In RAN2, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy.

Cluster approach is under discussion in RAN2 and this approach mainly focus on frequency domain prediction. We think cluster approach or single cell approach would definitely impact on prediction accuracy. In RAN2, it is observed that cluster cells as input can outperform the prediction accuracy than single cell as input. From accuracy definition perspective, either the AI prediction model is based on cluster approach or single cell approach is up to implementation, unless RAN2 specifies certain explicit indication and corresponding UE capability. Therefore it is FFS on whether cluster/single cell approach is one side condition for prediction accuracy

RAN2#128 Agreements

For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach.

In cluster approach the model takes measurements from more than one cell as inputs.

Proposal 6: For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy is to be specified under certain conditions,

- channel correlation coefficient

- FFS on cluster/single cell approach, depending on whether explicit indication and corresponding UE capability are specified in RAN2.

Proposal 7: cluster approach or single cell approach would impact on the value of prediction accuracy.

The legacy measurement reporting delay is defined as a delay for UE transmitting the measurement report. In AI/ML mobility, the inference function used for prediction may need some times. Therefore the processing time of inference function may need to be considered in measurement reporting delay.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

Proposal 8: The processing time of inference function may need to be considered in measurement reporting delay.

Measurement prediction accuracy

Principle

In TR38.744, 3 sub-use cases for cell-level measurement prediction are considered.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

To facilitate discussion, the high-level measurement model in TS38.300 without AI is duplicated as below. In RAN4 RRM, the legacy measurement accuracy is defined at point A1, where A1 is measurements (i.e. beam specific measurements) reported by UE layer 1 to UE layer 3 after layer 1 filtering.

Figure 9.2.4-1: Measurement Model [in TS38.300]

For AI mobility, in RAN2, measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and ground truth L3 filtered cell-level measurement result for sub-use case 1, 2 and 3. From RAN4 perspective, as the final measurement results of the 3 sub-use cases for cell-level RRM measurement prediction are L3 cell-level, the measurement prediction accuracy shall be defined at point C.

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Proposal 9: For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C.

In sub-use case 1, the prediction error is generated from actual L1 beam level predicting L1 beam level measurement (prediction is executed at point A1).

In sub-use case 2, the prediction error is generated from actual L3 beam level predicting L3 beam level measurement (prediction is executed at point C).

In sub-use case 3, the prediction error is generated from actual L1 beam level predicting L3 beam level measurement (measurement result at point A1 predicting measurement result at Point C).

In the three cases, the inference model is used at different position in the measurement process. The AI prediction model algorithm would be various due to different input and output. In theory the prediction accuracy evaluated at point C may be different for 3 sub-use cases. Then the question is whether RAN4 needs to define prediction accuracy for each sub-use case or RAN4 only define one cell level prediction accuracy requirement to cover 3 sub-use cases.

If use-case specific prediction accuracy is defined, UE needs to know which prediction accuracy is to be satisfied. Then network shall indicate which the sub-use case is. RAN2 will discuss the related issues in LCM, RAN4 can wait for RAN2 progress.

If it is expected to define one unified prediction accuracy, based on simulation results RAN4 can select one sub-use case which has proper or acceptable prediction accuracy, or RAN4 can determine one prediction accuracy to cover all sub-use cases. The prerequisite of both ways shall be based on simulation results.

Proposal 10: RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

Metrics for Measurement prediction accuracy

In RAN4#113 meeting, for AI mobility, a high-level definition of L3 RSRP predication accuracy is agreed in [6].

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

It is agreed in [7] that for the ground truth of L3 RSRP in FR2, reported L3 RSRP measurement under sufficient high SNR is taken as a baseline. With the sufficient high SNR, the propagation environment can be regarded as “no noise”. Moreover the conclusion is aligned with the ground truth definition in R19 AL/ML interface WI for beam management.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

The ground truth definition for L3 RSRP accuracy in FR1 is still under discussion [7]. There are two alternatives.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

- Alternative 1: Ground truth for FR1 is based on the TE transmitted or reception power.

Channel conditions

In legacy FR1 conducted measurement accuracy test, UE antenna is directly connected to TE. The channel environment is AWGN.  As there is no channel fluctuation in time domain, TE can determine the ideal L3-RSRP result. For AI/ML based measurement prediction, to verify AI prediction capability, AWGN channel is not proper, and fading channel is to be employed.

Observation 1: Fading channel is to be employed for AI mobility measurement prediction.

The measurement time instances alignment between UE and TE

The time instance of ground truth of L3 RSRP shall be aligned with that of the predicted L3 RSRP. Then TE shall have the information on the specific time instance when UE performs prediction. As discussed above, for either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a predicted result is unknown to TE.

Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

Number of samples of L1 measurement

In legacy L3 measurement, the measurement accuracy is specified at point A and no L1 filter is applied. For AI/ML measurement prediction, the L3 RSRP measurement is specified at point C. Then the ground truth of L3 RSRP result shall be determined by the linear average with multiple L1 measurement samples. Although the current L3 RSRP measurement period is defined based on 5 L1 samples, the actual number of L1 samples is up to UE implementation.

Observation 3: The number of L1 measurement samples is unknown to TE.

Based on the above analysis, the factors of measurement time instances alignment between UE and TE and number of samples of L1 measurement are obstacles to use ground truth definition Alt1.

Proposal 11: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles:

- The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

- The number of L1 measurement samples is unknown to TE.

- Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

In general, no big obstacles are observed if the ground truth for FR1 is defined based on UE reported value (Alt 2). One tricky point is that UE may cheat, as UE also know its reported value. The cheating issue is in parallel under discussion in R19 AI interface beam management test. We can wait more progress on R19 AI interface beam management

Proposal 12: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

- how to avoid UE cheating can wait the progress in R19 AI interface beam management test.

Relative RSRP accuracy

In RAN4#114 bis meeting, the definition of relative RSRP accuracy was discussed and achieved below agreement [8]. However there are some FFS left. In the following, we provide analysis on these.

Issue 2-1-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

The first FFS is whether the relative accuracy of beam-level L3 RSRP is needed or not. The accuracy focusses on point E, which is defined in  [TS38.300]

E: a measurement (i.e. beam-specific measurement) after processing in the beam filter. The reporting rate is identical to the reporting rate at point A1. This measurement is used as input for selecting the X measurements to be reported.

On the one hand, as we know, the beam-level measurement report includes up to maxNrofRS-IndexesToReport SSB indexes in order of decreasing beam sorting quantity whose sorting quantity is above absThreshSS-BlocksConsolidation. Therefore the relative comparison is indeed performed among the beams from the same cell. Moreover the existing definition of relative RSRP accuracy has already includes the beam level measurements.

##### 10.1.2.1.2 Relative SS-RSRP Accuracy The relative SS-RSRP accuracy is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured.

On the other hand, in TR38.744, 3 sub-use cases for beam level measurement prediction are introduced, however L3 filtered beam level prediction sub-use cases are lower priority in RAN2.

3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and simulation should focus on FR2 intra-frequency temporal domain case A

So we think the definition of relative accuracy of beam-level L3 RSRP is needed if sub-use cases of L3 filtered beam-level measurement predication (at Point E) are considered in RAN4 in R19, otherwise the definition can be deprioritized.

Proposal 13: The below definition of beam-level relative predicted accuracy of beam-level L3 RSRP is needed if sub-use cases of L3 filtered beam-level measurement predication are considered in RAN4 in R19, otherwise the definition can be deprioritized.

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

Regarding the inter-frequency L3 RSRP, the first definition of predicted relative accuracy is for the cells which are both the target cells for AI/ML-based L3-RSRP inference.

- Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

Regarding the second definition of relative accuracy in WF [8], we think two cases shall be considered:

Case 1: cell 1 and cell 2 are on different frequency, and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2. This case is scenario 3 (cell1 is predicated based on cell2 in frequency domain).

Case 2: Real measurement is performed on serving cell @ f2 where AI/ML inference is not applied. Cell 1 is the target cell on a different frequency @f1 than serving cell. AI/ML measurement prediction with temporal domain case B is applied for the target cell, in other words, cell 1 is predicated based on AI/ML model in temporal domain. In this case, it is likely, the predicted RSRP of target cell @f1 is to be compared with the measured RSRP of serving cell @ f2. The advantage of the case is that the measurement gap can be reduced for measuring on frequency f1.

Therefore, the inter-frequency RSRP relative accuracy definition shall include:

- Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

- Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2,

- if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, or

- if cell 1 is the target cell based on intra-cell prediction in temporal domain

Proposal 14: For inter-frequency L3-RSRP,

- Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

- Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2,

- if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, or

- if cell 1 is the target cell based on intra-cell prediction in temporal domain

LCM requirements

In management function (LCM procedure), the functionality/model activation/deactivation/fallback to non-AI function are to be considered. If the functionality/model activation/deactivation/fallback to non-AI function are indicated by network, e.g., MAC CE/DCI triggered, the delay of activating/deactivating one function would be defined. The side conditions of activation/deactivation/fallback delay may also be discussed. Otherwise if these functionality/model changes are autonomously performed by UE, UE monitors the function performance, then the requirements would be related with monitoring requirements.

Proposal 15: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

Proposal 16: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

Performance Monitoring requirements

If performance monitoring is mainly executed at UE side, some metrics are to be defined so that UE itself can make decision of activation, deactivation, or fallback to non-AI. Then from RRM requirement perspective, the requirements of monitoring periodicity or reporting delay when certain metric is met may need to be specified.

If performance monitoring is executed at network, the monitoring function is up to network implementation. If new measurement quantity is introduced, new requirements would be specified, otherwise no RRM requirements on monitoring is observed.

As performance monitoring are under discussion in RAN1 R19 AI/ML air interface, RAN4 can wait for more progress.

Proposal 17: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface.

Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: RAN4 to start the discussion on scenario 2, 3 and 4 with high priority. Scenario 6 (FR2 to FR2 intra-frequency spatial domain for the 1st goal) can be carried out after sufficient progress are achieved on high priority scenarios.

Proposal 2: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed.

Proposal 3: For intra-frequency temporal domain prediction case A, observation window length (or called as prediction evaluation period) and prediction window length may need to be specified.

Proposal 4: To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing.

Proposal 5: For intra-frequency temporal domain prediction case B, RSRP prediction accuracy is to be defined under the side condition of MRRT with limited PW length.

Proposal 6: For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy is to be specified under certain conditions,

- channel correlation coefficient

- FFS on cluster/single cell approach, depending on whether explicit indication and corresponding UE capability are specified in RAN2.

Proposal 7: cluster approach or single cell approach would impact on the value of prediction accuracy.

Proposal 8: The processing time of inference function may need to be considered in measurement reporting delay.

Proposal 9: For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C.

Proposal 10: RAN4 to discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

Observation 1: Fading channel is to be employed for AI mobility measurement prediction.

Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

Observation 3: The number of L1 measurement samples is unknown to TE.

Proposal 11: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles:

- The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

- The number of L1 measurement samples is unknown to TE.

Proposal 12: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

- how to avoid UE cheating can wait the progress in R19 AI interface beam management test.

Proposal 13: The below definition of beam-level relative predicted accuracy of beam-level L3 RSRP is needed if sub-use cases of L3 filtered beam-level measurement predication are considered in RAN4 in R19, otherwise the definition can be deprioritized.

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

Proposal 14: For inter-frequency L3-RSRP,

- Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

- Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2,

- if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, or

- if cell 1 is the target cell based on intra-cell prediction in temporal domain

Proposal 15: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

Proposal 16: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

Proposal 17: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface.

Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2416855, WF on AI/ML Mobility, Nokia

- R4-2401570, WF on AI/ML, Qualcomm Incorporated

- TR38.843

- TR38.744

- R4-2420095, WF on AI/ML Mobility, Nokia

- R4-2502594, WF on AI/ML Mobility, Nokia

- R4-2504901, WF on AI/ML Mobility, Nokia