3GPP TSG-RAN WG4 Meeting #115	R4-2506783

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on RRM requirements for NTN in Ku band

Source: 	Huawei, HiSilicon

Agenda item:	7.11.7

Document for:	Discussion

Introduction

RRM requirements for NTN in Ku band are discussed in RAN4#114-bis, and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Requirement applicability

- Timing requirements

- Measurement and mobility requirements

In this paper, we will provide our views on RRM requirements for NTN in Ku band.

Discussion

Requirement applicability

Issue 1-2: applicability update (section 3.6.12). Proposals: - Option 1: The requirements for FR2-NTN/NTN band above 10GHz except for timing requirements in Clause 7 are applicable to Ku-band NTN with FR1-NTN and FR2-NTN numerologies. (LGE) Recommended WF - Continue discussion

Issue 1-13: terminology update Proposals: - Option 1: change ‘FR2-NTN’ to ‘[single receive beam] VSAT UE’ (Eutelsat Group) - Option 2: others Recommended WF - Continue discussion on how to align terminology.

In our view, the 2 issues 1-2 and 1-13 are same, and they both address the ‘mix’ of FR1-NTN and FR2-NTN. In RAN4#114 it is agreed in RF session [2] to extend FR1-NTN and FR2-NTN.

Sub-topic 1-2: Definition of extended FR1-NTN and FR2-NTN frequency ranges Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN (Note 1); 410 MHz – 14500 MHz. FR2-NTN (Note 2); 10700 MHz – 30000 MHz. NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications..

On the other hand, RRM requirements are defined assuming omni-directional Rx for UEs in FR1-NTN and directional Rx for UEs in FR2-NTN. This means a VSAT UE, which always uses directional Rx, working in FR1-NTN may not be able to meet requirements defined for FR1-NTN. To address this issue, we prefer to use the approach given by proposal in Issue 1-2. The problem with option 1 in Issue 1-13 is that it requires updates of terms across the whole spec which requires large efforts and is error prone.

Proposal 1: Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

Timing requirements

Issue 1-9: timing accuracy requirements for case 4 (section 7.1C) Agreement: - The same Te_NTN requirement applies to case 4. On top of the side conditions agreed for Rel-18 Ka band, RAN4 to add the following side condition to Te_NTN requirements for Case 4 - Option 1: Capture it as a NOTE on the test case in the core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec in the core corresponding test case. - Option 2: Capture it as a NOTE in core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec.

In RAN4#114-bis timing requirements for Case 1-3 are settled down. For Case 4, we support to adopt option 2 with 5.12s.

First, between option 1 and 2, we do not see a need to capture in core requirements how to set a parameter in test cases. Instead, the parameter can be defined directly in test cases. For example, the interval between adjacent epoch times is defined as 10.24s for GSO and 2.56s for NGSO in R17. We can re-use these values for R19 test cases for Ku bands. If there is a real problem on the ephemeris update rate for Case 4, it should be captured as side condition for core requirements.

Second, on the exact values, based on our simulation, we do not see clear performance difference between 2.56s and 5.12s update interval at 95%-tile. It is clear that the smaller update interval is, the better accuracy UE can achieve. On the other hand, it will cause additional overhead and restrictions from NW side to schedule SIB19 very frequently. As a compromise, we see 5.12s is a reasonable value.

Proposal 2: For timing requirement in Case 4, capture it as a NOTE in core requirement: The interval between adjacent epoch times should not exceed [5.12] sec.

Measurement and mobility requirements

Issue 1-3: Measurements of inter-frequency NR cells with NTN carrier (section 4.2.2.12). Proposals: - Option 1: The differentiation of requirements in clause 4.2.2.12 should not be based on the Rel-18 frequency range definition but on UE/Antenna type and capabilities. (Eutelsat Group) Recommended WF - Continue discussion

We do not support option 1 in Issue 1-3.

- In R18 RAN4 did not define TN-NTN cell reselection requirements for VSAT working in Ka band. It is not clear why requirements are needed for VSAT working in Ku bands. In our view, VSAT working in TN is not a very strong use case no matter the VSAT is supporting Ka or Ku band.

- The existing requirements in clause 4.2.2.12 cannot be re-used. The requirements in clause 4.2.2.12 are defined by referring to cell reselection requirements for FR1-NTN in clause 4.2C.2.4. As discussed in section 2.1, the existing FR1-NTN requirements do not apply at least for baseline VSAT with single Rx beam, and we do not prefer to open the discussion for the more capable multi-Rx beam VSAT in this WI when there is no requirement for the baseline VSAT.

Proposal 3: Do not extend the applicability of clause 4.2.2.12 to VSAT.

Issue 1-4: inter-satellite cell reselection requirements (section 4.2C). Proposals: - Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs in Rel-19. (Nokia) Recommended WF - Continue discussion

We do not support option 1 in Issue 1-4.

- In R18 RAN4 did not define inter-sat cell reselection requirements for VSAT working in Ka band. It is not clear why requirements are needed for VSAT working in Ku bands. Technically, we do see some significance of the requirements for VSAT, but it is not proper to define them in this WI as it is not a specific issue for Ku bands.

- Defining the requirements requires non-trivial efforts. RAN4 needs to discuss the UE behavior for cell search and measurement of a different satellite, e.g. whether it is beam sweeping or beam steering, and the related delay requirements. Considering there are only 2 meetings left for the core part, we suggest not to extend the WI scope too much.

Proposal 4: Do not define inter-satellite cell reselection requirements in this WI.

Issue 1-5: UE measurement capability (section 4.2C.2.1). Proposals: - Option 1: For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least: - Intra-frequency carrier [in the case of VSAT UE with single receive beam, non-simultaneous monitoring intra-frequency carriers from more than one SAN], and - Depending on UE capability, FR1-NTN UE 7 NR inter-frequency carriers and for all VSAT UE up to [8] inter frequency carriers. (Eutelsat Group) Recommended WF - Continue discussion

We do not support option 1 in Issue 1-5.

- On the first bullet, it is not clear why measurement capability of intra-frequency only applies to VSAT with single Rx beam. In our understanding, it should also apply for VSAT with multiple Rx beams.

- On the second bullet, we can understand the motivation, but it seems current capability of 7 carriers is enough for totally 8 carriers (1 serving carrier + 7 inter-frequency carriers).

Proposal 5: Re-use existing measurement capability requirements for VSAT in Ku bands.

Issue 1-6: NR SAN FR2-NTN – NR SAN FR2-NTN Handover (section 6.1C.1.3) Agreement: - Reuse the same value of T_sat_beam as in Ka band for Ku band - For VSAT UEs operating in FR1-NTN numerology, not specify the RRM requirements for TN-NTN mobility, inter-RAT mobility [or satellite switching with re-sync].

Based on discussion in RAN4#114-bis, the remaining open issue is whether to define requirements for satellite switching with re-sync for VSAT. In our view, this is an acceptable enhancement in this WI.

Satellite switching is similar to HO where UE knowns a specific target satellite, and UE can directly search and get sync to the target satellite like in blind HO at least for the hard switch. For soft switch, whether it is feasible to keep the connection to source satellite while obtaining sync to the target satellite depends on the Rx beam switching delay, and it may feasible to do so for UE with electronical beam.

For any case, the delay should include an additional component for fine beam steering, and it can be defined by re-using the beam steering time Tsat_beam from HO requirements.

Proposal 6: Define requirements for satellite switching with re-sync for VSAT at least for hard switch, and FFS for soft switch. The additional beam steering time can be defined by re-using Tsat_beam from inter-satellite HO requirements.

Issue 1-12: RLM (section 8.1C) Proposals: - Option 1: Confirm the assumption that LMax is equal to 8 for FR1-NTN (Nokia) Recommended WF - Continue discussion

We support option 1 in Issue 1-12.

As discussed in section 2.1, for VSAT working in FR1-NTN, the requirements defined for FR2-NTN should apply as default. For LMax, RAN1 did not increase the value for the extension of FR1-NTN to 14.5GHz, so the it should be same as existing FR1-NTN rather than 64 defined for FR2-NTN. This is an exception where FR1-NTN requirements would apply for VSAT working in FR1-NTN.

Proposal 7: RAN4 to clarify that LMax defined for FR1-NTN applies for VSAT working in FR1-NTN.

Conclusions

In this paper we provided our views on RRM requirements for NTN in Ku band.

Proposal 1: Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

Proposal 2: For timing requirement in Case 4, capture it as a NOTE in core requirement: The interval between adjacent epoch times should not exceed [5.12] sec.

Proposal 3: Do not extend the applicability of clause 4.2.2.12 to VSAT.

Proposal 4: Do not define inter-satellite cell reselection requirements in this WI.

Proposal 5: Re-use existing measurement capability requirements for VSAT in Ku bands.

Proposal 6: Define requirements for satellite switching with re-sync for VSAT at least for hard switch, and FFS for soft switch. The additional beam steering time can be defined by re-using Tsat_beam from inter-satellite HO requirements.

Proposal 7: RAN4 to clarify that LMax defined for FR1-NTN applies for VSAT working in FR1-NTN.

Reference

- R4-2504897, WF on RRM requirements for NR_NTN_Ku_bands, Apple

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)