3GPP TSG-RAN WG4 Meeting #115	R4-25xxxx

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on UE CLI measurement for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	7.25.4.1

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#114-bis, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for UE CLI measurement.

- General issues

- Specific issues for L1-SRS-RSRP

- Specific issues for L1-CLI-RSSI

In this paper, we will provide our views on RRM requirements for UE CLI measurement in SBFD.

Discussion

General issues

Requirement applicability

In R16, following applicability is defined for SRS-RSRP and CLI-RSSI.

The UE capable of performing CLI measurements shall be able to measure SRS-RSRP and CLI-RSSI which are defined in TS38.215 [4] within active DL BWP. The measurements requirements in this clause apply for TDD mode only. CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resouce is configured within active BWP

For L1-SRS-RSRP and L1-CLI-RSSI, RAN1#118 agreed following.

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband FFS: Method#3: UE measures RSSI within UL subband

In RAN1#119, Method#3 is also supported. In addition, in RAN1#120-bis following is agreed.

Agreement Configuration of Method 1 or Method 3 are not explicitly provided for L1 CLI-RSSI measurement and reporting. Configuration of Measurement resource type #1 or Measurement resource type #2 are not explicitly provided for L1 CLI-RSSI measurement. - CLI-RSSI measurement resource is configured in one UL subband only, in one DL subband only, or across two DL subbands only. - If CLI-RSSI measurement resource is configured across two DL subbands, the frequency resources of CLI-RSSI measurement resource are derived by excluding the frequency resources outside DL usable PRBs.

In RAN4#114-bis, most companies including us suggested to re-use the R16 applicability.

Some companies suggested that measurement resource for method #2 and method #3 should be within UL active BWP, and there is no restriction on resource location for method #1. We have a different view because UL active BWP is for UE transmission and it’s clear from RAN1#118-bis agreement that CLI measurement resources should be within DL active BWP.

Some companies suggested to define the detailed applicability that SRS is confined within UL subband, and RSSI resources for method #1 and #3 are confined within DL and UL subband, respectively. One issue with this proposal is that part of RSSI resources for method #1 can be within UL subband (with resource type #2), so it is not fully correct to state that RSSI resources for method #1 are confined within DL subband. Another issue with this proposal is that the restriction on DL active BWP is missing.

Based on analysis of different proposals and RAN1 agreements, we suggest to re-use the R16 applicability

Proposal 1: L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency

- when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

- when L1-CLI-RSSI measurement resource is configured within BW of DL active BWP

Specific issues for L1-SRS-RSRP

Scheduling restriction

In RAN4#114-bis, the scheduling requirements for L1-SRS-RSRP are mostly settled down, and the only open issue is whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

- For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

We do not support to define restriction after the SRS resource for switching between UL subband and DL subband. This restriction is needed if UE is using different Rx BW in SBFD symbols than in DL symbols. In DL symbols, the Rx BW should at least cover the DL active BWP. In SBFD symbols, UE may use a special filter that covers only DL subband(s), so that the UE-to-UE CLI from the UL subband is suppressed.

Although there is some benefit, it will increase UE’s cost due to implementing a special RF filter, and it will cause performance loss with interruptions e.g. at transition between SBFD and non-SBFD symbols, or before and after CLI measurement in UL subband, or even at transition between UE’s Tx and Rx in consecutive SBFD symbols.

Considering the pros and cons as well as the standardization efforts in RAN1 and RAN4, we suggest to not consider RF retuning before and after the measurement, i.e. UE is assumed to be able to receive in the UL subband on SBFD symbols without RF retuning.

Proposal 2: Do not introduce scheduling restriction for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

Measurement restriction

The remaining open issue in measurement restriction for L1-SRS-RSRP is the collision between L1-SRS-RSRP and L1-CLI-RSSI.

- When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

We do not have strong view which one to prioritize. In our view, it is similar as existing measurement restriction for SSB and CSI-RS based L1 measurement where UE is required to perform one but not both of the two measurements.

Proposal 3: If the SRS resources configured for L1-SRS-RSRP are partially or fully overlapping with measurement resources for L1-CLI-RSSI, it is up to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

Another issue is the collision with L3 CLI measurement. Our understanding is that L1-SRS-RSRP cannot be performed at same time as L3 SRS-RSRP or L3 CLI-RSSI. Even L1-SRS-RSRP measurement is based on UL timing, it does not necessarily mean the measurement timing is same as L3 SRS-RSRP or L3 CLI-RSSI. The exact measurement timing is up to UE implementation, and for example, UE may account for possible cell phase error for L3 SRS-RSRP or L3 CLI-RSSI measurement and may not do so for L1-SRSR-RSRP.

The next question is which one between L1-SRS-RSRP and L3 SRS-RSRP/L3 CLI-RSSI to prioritize when their measurement resources are overlapping in time. Similar to above, we do not have strong view and we suggest to leave it to UE implementation as such collision is not likely to happen in real deployment.

Proposal 4: If the SRS resources configured for L1-SRS-RSRP are partially or fully overlapping with measurement resources for L3 SRS-RSRP/L3 CLI-RSSI, it is up to UE implementation whether to measure L1-SRS-RSRP or L3 SRS-RSRP/L3 CLI-RSSI.

Specific issues for L1-CLI-RSSI

Measurement period

In RAN4#114-bis following is agreed for DUD case with method #1 and resource type#2.

- For DUD case with method #1 and resource type#2 - In RAN4 understanding, if UE can support simultaneous PDSCH reception on the two DL subbnads for DUD, it can support simultaneous RSSI measurement on the two DL subbands based on 1 sample. - RAN4 to decide the number of samples for measurement period based on RAN1 progress on UE capability.

In RAN1#120-bis, the following description is agreed for GF 60-1 Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures. It can be seen that it has not been decided whether it is mandator or optional for an SBFD capable UE to support DL reception in two DL subbands.

1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols [FFS: 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols [FFS: 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG)

Based on the RAN4 agreements and status in RAN1 feature list discussion, we suggest RAN4 to confirm that the number of samples for DUD case with method #1 and resource type#2 is 1, at least for the case when UE supports DL reception in two DL subbands. RAN4 can wait for RAN1 conclusion on whether the case when UE does not support DL reception in two DL subbands exists, and further discuss the requirements.

Proposal 5: RAN4 to confirm that the number of samples for DUD case with method #1 and resource type#2 is 1, at least for the case when UE supports DL reception in two DL subbands.

- RAN4 to wait for RAN1 conclusion on whether the case when UE does not support DL reception in two DL subbands exists or not, and if so, discuss whether and how to define requirements.

Scheduling restriction

In RAN4#114-bis, the scheduling requirements for L1-CLI-RSSI are mostly settled down, and the only open issue is whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

- Method #3: - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

The issue is same as for CLI-SRS-RSRP, and for the same consideration as in section 2.2.1, we suggest RAN4 not to introduce scheduling restriction for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

Proposal 6: Do not introduce scheduling restriction for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

Measurement restriction

In RAN4#114-bis following is agreed for measurement restriction for L1-CLI-RSSI. The remaining issue is whether measurement requirements apply for L1-CLI-RSSI when RSSI resources and resources for L1/L3 RRM measurements overlap in time in FR1.

- Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

In our view, measurement requirements apply for L1-CLI-RSSI could apply in most cases as in most cases UE would be able to measure RSSI and L1/L3 at same time, so it should be taken as default, and RAN4 can further define the cases where requirements cannot apply.

Based on RAN4#114-bis discussion, we identified the following two cases:

- MG: if RSSI resource collides with MG, it is clear that UE cannot measure RSSI and L1/L3 at same time

- Different SCS: if RSSI resource is configured with a different SCS from the SSB used for L1/L3 measurement, UE may not be able to measure RSSI and L1/L3 at same time

Proposal 7: Measurement requirements apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR1, except for the following cases

- If RSSI resource collides with MG

- If RSSI resource is configured with a different SCS from the SSB used for L1/L3 measurement

Another issue is the collision with L3 CLI measurement. Our understanding is that L1-CLI-RSSI cannot be performed at same time as L3 SRS-RSRP or L3 CLI-RSSI as L1-CLI-RSSI is based on DL timing, while L3 SRS-RSRP or L3 CLI-RSSI is based on UL timing.

The next question is which one between L1-CLI-RSSI and L3 SRS-RSRP/L3 CLI-RSSI to prioritize when their measurement resources are overlapping in time. Similar to section 2.2.2, we do not have strong view and we suggest to leave it to UE implementation as such collision is not likely to happen in real deployment.

Proposal 8: If the SRS resources configured for L1-CLI-RSSI are partially or fully overlapping with measurement resources for L3 SRS-RSRP/L3 CLI-RSSI, it is up to UE implementation whether to measure L1-CLI-RSSI or L3 SRS-RSRP/L3 CLI-RSSI.

SCS for performing measurement

In RAN#120 following is agreed on how UE determines SCS of L1-CLI-RSSI measurement resource.

Agreement - UE determines the subcarrier spacing of the CLI-RSSI measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - UE determines the subcarrier spacing of the SRS-RSRP measurement resources configured in a CSI-ResourceConfig based on the subcarrier spacing of the DL BWP identified by the higher-layer parameter bwp-Id in the CSI-ResourceConfig. - Note: No RAN1 spec impact is expected.

In TS 38.133, clause 9.7.3.1, for L3 CLI-RSSI measurement we have

The subcarrier spacing for CLI-RSSI measurement resource configuration can be same or different from the subcarrier spacing of active BWP. UE shall perform CLI-RSSI measurement with the SCS of the active BWP.

Given the RAN1 agreement, SCS for L1-CLI-RSSI measurement resource configuration can only be same as the SCS of DL active BWP. Combining the R16 requirements for L3 CLI-RSSI, it is straightforward that UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP, for both method #1 and method #3.

Proposal 9: UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP, for both method #1 and method #3.

Conclusions

In this paper we provided our views on RRM requirements for UE CLI measurement in SBFD.

Proposal 1: L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency

- when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

- when L1-CLI-RSSI measurement resource is configured within BW of DL active BWP

Proposal 2: Do not introduce scheduling restriction for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband.

Proposal 3: If the SRS resources configured for L1-SRS-RSRP are partially or fully overlapping with measurement resources for L1-CLI-RSSI, it is up to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

Proposal 4: If the SRS resources configured for L1-SRS-RSRP are partially or fully overlapping with measurement resources for L3 SRS-RSRP/L3 CLI-RSSI, it is up to UE implementation whether to measure L1-SRS-RSRP or L3 SRS-RSRP/L3 CLI-RSSI.

Proposal 5: RAN4 to confirm that the number of samples for DUD case with method #1 and resource type#2 is 1, at least for the case when UE supports DL reception in two DL subbands.

- RAN4 to wait for RAN1 conclusion on whether the case when UE does not support DL reception in two DL subbands exists or not, and if so, discuss whether and how to define requirements.

Proposal 6: Do not introduce scheduling restriction for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

Proposal 7: Measurement requirements apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR1, except for the following cases

- If RSSI resource collides with MG

- If RSSI resource is configured with a different SCS from the SSB used for L1/L3 measurement

Proposal 8: If the SRS resources configured for L1-CLI-RSSI are partially or fully overlapping with measurement resources for L3 SRS-RSRP/L3 CLI-RSSI, it is up to UE implementation whether to measure L1-CLI-RSSI or L3 SRS-RSRP/L3 CLI-RSSI.

Proposal 9: UE shall perform L1-CLI-RSSI measurement with the SCS of the DL active BWP, for both method #1 and method #3.

Reference

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon