3GPP TSG-RAN WG4 Meeting #115	R4-2506786

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on RRM impacts for SBFD operation

Source: 	Huawei, HiSilicon

Agenda item:	7.25.4.2

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#114-bis, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for RRM impacts for SBFD operation.

- CSI-RS based L1 measurement

- CSI-RS based L3 measurement

- Scheduling restriction

In this paper, we will provide our views on RRM impacts for SBFD operation.

Discussion

CSI-RS based L1 measurement

puncturing in frequency domain

in RAN4#114-bis following is agreed for CSI-RS L1 measurement – puncturing in frequency domain. The core requirements for all Cases 1, 2 and 3 are mostly settled down, while the relaxed accuracy for Case 3 is in scope of performance part. The remaining open issue for core part is highlighted in yellow which is common for all Cases in our view.

- Agreements (adhoc session on Thursday with additions) - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE.

The issue is raised up in [2] based on the following RAN1#118-bis agreement.

Agreement [RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

In our view, option 1 is reasonable. For a single CSI-RS resource, UE is not supposed to take measurement on an occasion if it has not finished processing of a previous occasion of the same resource, so measurement interval should not be smaller than the processing time. Based on analysis in [2], for UE indicating that CSI processing time is scaled by 2, the processing time would be around 8ms.

On the other hand, in existing delay requirements for L1-RSRP and L1-SINR, UE may be required to measure every 4 slots, since both TReport and TCSI-RS can be configured as 4 slot which is smaller than 8ms.

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |

It is noted that the CSI processing time is only applicable for L1-RSRP/L1-SINR which follow CSI framework. RLM/BFD/CBD is not impacted by the RAN1 agreement.

Proposal 1: If UE indicates that CSI processing time is scaled by 2, update the delay requirements for L1-RSRP and L1-SINR by adding a lower bound of 8ms on top of existing delay requirements.

different symbol types

in RAN4#114-bis, following is agreed for CSI-RS L1 measurement – different symbol types.

- Agreements - For L1-RSRP and L1-SINR - For CSI-RS based L1-RSRP/SINR measurements, UE measures the CSI-RS occasions on the valid symbol type indicated by network - For L1-RSRP and L1-SINR measurement period, further discuss the following options. - Option 1: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach. - Option 2: other - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

For L1-RSRP and L1-SINR, we support to adopt option 1. Based on following RAN1 agreement, the case where a single CSI-RS resource has occasions on both SBFD and non-SBFD symbols types is considered by RAN1, and UE should only measure the occasions on the valid symbols type as indicated by the NW in CSI-ReportConfig.

Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig.

Existing measurement period for L1-RSRP/L1-SINR is defined based on CSI-RS resource periodicity, and this means all resource occasions are counted for the measurement. Under SBFD operation, the measurement period should only count resource occasions on the valid symbol type, or in other words, the measurement period should be extended by excluding the resource occasions on the invalid symbol type. We suggest to follow the R16 NR-U approach to define the measurement period considering that 1) it is difficult to define the ratio of valid resource occasions over the total resource occasions since it depends on time domain SBFD configuration and periodicity and offset of the CSI-RS resource, and 2) we already use the NR-U approach to account for the impact of collision with dynamic UL.

Proposal 2: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

CSI-RS based L3 measurement

In RAN4#114, following is agreed for CSI-RS based L3 measurement.

- Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

We suggest to apply the RAN4#114 agreement also for DUD case. One clarification for DUD case is that there are two DL subbands, and the condition for the legacy requirements to apply should be that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols. If the CSI-RS resources in two DL subbands are considered, it means some RBs in between the two DL subbands are punctured. As UE is not aware of SBFD operation in neighbour cell, UE will be measuring only noise and interference on those punctured RBs.

During the discussion in RAN4#114, the concern for DUD case was the required cell BW may be large for the existing requirements to apply, and in real world the condition may not be met for many cases or for typical cases. We understand this is not big issue.

In the following tables, we marked the cases where the transmission bandwidth configuration is enough for both CSI-RS and L1-SRS-RSRP requirements to apply, based on the assumption that 1) at least 48 RBs are required for one DL subband, and the BWs for the two DL subbands are almost same, 2) at least 24 RBs are required for the UL subband (for the L1-SRS-RSRP requirements to apply), and the BW can only be selected from the values in Table 5.3.2-1 and Table 5.3.2-2, and 3) some guard band are needed.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

Based on the analysis, both CSI-RS and L1-SRS-RSRP requirements can apply in many cases, and some are typical cases for SBFD operation, e.g. 30kHz SCS with 60/80/100MHz CBW. Of course, the conditions for both CSI-RS and L1-SRS-RSRP requirements to apply are not met for all applicable CBWs for SBFD, but this is normal in that most RAN4 requirements are defined based on certain side conditions, and they do not apply for all cases. We do not see strong reason to preclude L3 CSI-RS measurement requirements for DUD.

Another clarification on the RAN4#114 agreement is that the condition for the legacy requirements to apply should be that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols of the cell transmitting the CSI-RS. This is because whether a CSI-RS resource will be punctured in frequency domain depends on the SBFD configuration in the cell transmitting the CSI-RS, but not on the SBFD configuration in UE’s serving cell. We assume this was the common understanding when RAN4 made the agreement in RAN4#114, but it is better to make it clear to avoid possible confusion.

Technically, it may happen that SBFD configuration of neighbour cell is different from UE’s serving cell. For intra-frequency measurement, this would be a rare case, but anyway the existing scheduling restriction would apply because the measurement is same as in legacy from UE perspective. For inter-frequency measurement, measurement gap will be used, so symbols type of serving cell does not matter anyway.

Proposal 3: For L3 CSI-RS measurement, apply the RAN4#114 agreement also for DUD case and update the RAN4#114 agreement as follows.

- UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS.

Scheduling restriction

One issue in scheduling restriction is whether to update the existing requirements to take DL/UL transition time into account. For example, RAN4 has defined scheduling restriction for TDD for L3 measurement but not for L1 measurement. The reason is that there is always guard period for the DL-to-UL transition, and UL transmission on UL symbols would never overlap with SSB reception for the serving cell in a reasonable configuration. However, there is no guard period between SBFD symbol and SSB symbol. UL transmission on SBFD symbol immediately after SSB symbol may collide with SSB reception due to TA.

Based on discussions in last meeting, we understand such a collision is already addressed by the following RAN1#116 agreement. Therefore, we suggest RAN4 not to update the existing scheduling restriction requirements to account for DL/UL transition time.

Agreement For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Taking L1 SSB measurement on SBFD symbols as example, not only the UL transmission on SSB symbols, but also the UL transmission on the symbol immediately after the SSB symbols would be dropped. Based on the note in the RAN1 agreement, we understand that this will be captured in RAN1 spec. In fact, dropping of additional UL transmission due to lack of sufficient transition time is not a specific issue for measurement, but it may also happen for other DL reception, e.g. UL transmission on the symbol immediately after an SBFD symbol where UE receives PDSCH will be dropped.

Considering that the impact of DL/UL transition will be addressed by RAN1 spec and it’s not limited to measurement, we suggest RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time.

This is also aligned with the RAN4#114-bis agreement for the scheduling restriction for L1 CLI measurement, where RAN4 agreed not define scheduling restriction on symbols after symbols for CLI resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211.

Proposal 4: RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time.

Another issue raised up by some companies last meeting is the case when SSB for neighbor cell is not within DL subband of UE’s serving cell. In our understanding, such measurement must be inter-frequency because serving cell SSB is always within DL subband. Inter-frequency SSB may be measured outside gaps if UE supports R16 inter-frequency measurement without gap and the SSB is within UE active BWP. In this case, UE would measure SSB outside DL subband, and the question is whether UE can receive data within the DL subband at the same time.

In our view, this depends on whether UE Rx BW is assumed to only cover DL subband or the whole DL BWP on SBFD symbols. Technically, we understand UE may use a special RF filter on SBFD symbols whose Rx BW will only cover DL subband, so that the UE-to-UE CLI from the UL subband is suppressed. Although the benefit is there, we do not suggest to take this assumption for defining RAN4 requirements.

- It will increase the cost of the UE due to special RF filter.

- There will be interruptions due to RF retuning e.g. at transition between SBFD and non-SBFD symbols, or before and after CLI measurement in UL subband, or even at transition between UE’s Tx and Rx in consecutive SBFD symbols.

- It will incur large standardization efforts in RAN1 and RAN4 RF, as such RF retuning has not been considered in the transient period discussion in RAN1 and RAN4 RF.

Proposal 5: RAN4 not to define new scheduling restriction for the case when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots.

Conclusions

In this paper we provided our views on RRM impacts for SBFD operation.

Proposal 1: If UE indicates that CSI processing time is scaled by 2, update the delay requirements for L1-RSRP and L1-SINR by adding a lower bound of 8ms on top of existing delay requirements.

Proposal 2: Measurement period for CSI-RS based L1-RSRP/L1-SINR is extended by excluding resource occasions on invalid symbol type following the R16 NR-U approach.

Proposal 3: For L3 CSI-RS measurement, apply the RAN4#114 agreement also for DUD case and update the RAN4#114 agreement as follows.

- UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS.

Proposal 4: RAN4 not to update existing scheduling restriction or define new scheduling restriction to account for DL/UL transition time.

Proposal 5: RAN4 not to define new scheduling restriction for the case when SSB for L3 measurement is within DL active BWP but not completely contained in DL subbands on SBFD symbols/slots.

Reference

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504480, Discussion on SBFD Operation, MediaTek inc.