3GPP TSG-RAN WG4 Meeting #115	R4-2506789

Malta, MT, 19 – 23 May, 2025

Title: 	Simulation assumption and additional simulation results for LP-WUR measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.28.5.1

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#114-bis, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to evaluate the link level performance of LP-WUR measurement. In RAN4#114-bis, we provided our simulation results [2] for LP-SS based on the selected binary sequences from RAN4#114.

In this paper, we will provide our views on the remaining issues for simulation assumption and also provide updated simulation results.

Discussion

  - Simulation assumption

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

Issue 2-1-5: On ideal LP-RSRP/RSRQ in simulation Agreement: Use on method 1 in P2 as a base further check the difference between method 1 and method 2 in P2 and check the channel estimation assumption in ideal RSRP for OOK based LR

In our view, the ideal RSRP should be based on the serving cell signal after fading channel. By definition, it is the received power on certain time or time/frequency resources, and the received power is impacted by the channel fading, so the ideal RSRP should be based on actual channel fading instead of statistic attenuation. Besides, we understand it has been the principle in R15 simulation to derive the normal RRM accuracy.

Similarly, for ideal RSRQ, the RSSI part should be based on the actual total received power at the specific time or time/frequency resources instead an assumed total received power.

Proposal 1: RAN4 to confirm that ideal RSRP is derived from the serving cell signal after the fading channel; ideal RSRQ is derived based on ideal RSRP divided by the actual RSSI.

Issue 2-1-7: On OOK interference

Our simulation results are shown in Table 1 in section 2.2 for all cases supported by RAN1, we find that the performance for M=1, L=6 is particularly poor, especially in TDL-C channel. The specific of this case is that the binary sequence for the serving cell and interfering cell are opposite to each other. It can be seen that the target accuracy of 3.5dB cannot be reached even with 4 samples in TDL-C.

For M=1, L=6 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0 1 0] Interference cell: [0 1 0 1 0 1]

We further checked the reason for the poor performance. We used the basic RSRP calculation method in the simulation where RSRP is calculated by total power on the ON symbols minus that on the OFF symbols.

- For AWGN channel, the interference is always 6dB lower than the signal, so ideally the RSRP error would be 10*log(0.75)=-1.25dB, and we also find this is achievable when noise is small enough.

- For fading channel, when we are looking at 5% or 95%-tile of the error CDF, the interference is quite close to the signal due to the instantaneous channel fading of the service link and interference link. In this case, the RSRP error would be close to 10*log(0)=-inf.

In our view, there are 2 approaches for to address such cases with opposite sequence.

- Alt 1: define requirements for all combinations, and the requirements are based those with opposite sequence (worst case)

- Alt 2: define the requirements without considering combinations with opposite sequence, and the requirements are not applicable for those with opposite sequence

We suggest to adopt Alt 2 because the combinations with opposite sequence are in minority, but they will drag down the requirements significantly (longer delay plus degraded accuracy).

Proposal 2: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences.

Issue 2-1-11: Others on simulation assumptions and campaign

In our view, there is no need to align the simulation assumption on whether OOK based LR is using I-branch only or using both I- and Q-branch. This is an UE implementation issue, and it should be up to each vendor to decide the proper assumption for defining the requirements. Our results in section 2.2 are based on using both I- and Q-branch.

Proposal 3: RAN4 not to align simulation assumption on whether OOK based LR is using I-branch only or using both I- and Q-branch.

  - Simulation results for LP-SS

The parameters selected for simulations are as follows.

- Serving cell SINR: -3dB

  - Serving cell SNR = -2.4dB, interfering cell SNR = -8.4dB, i.e. 6dB power difference

- Sample number: 1, 2, 3, 4

- Sampling rate: 7.68MHz

- ADC bit width: 8bit

- Filter: 3th Order Butterworth with 3.96MHz bandwidth

- Interference type: LP-SS, interfering cell is fully synchronous to serving cell, binary sequence for LP-SS is independent for serving cell and interfering cell

The results in this section are for OOK based LR, i.e. without utilizing the overlaid sequence on LP-SS. Our updated simulation results for LP-SS are given in Table 1.

Table 1: Simulation results for LP-SS

| {M,L} | LP-SS Binary Sequence | Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | channel | Interference Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RSRP error |  |  |  | RSRQ error |  |  |  |  |  |  |
|  |  | X=1 | X=2 | X=3 | X=4 | X=1 | X=2 | X=3 | X=4 |  |  |  |
| {1,6} | Opposite sequence | 3.42 | 2.63 | 2.49 | 2.42 | 3.47 | 2.73 | 2.58 | 2.47 | -3 serving cell SNR = -2.4dB neighbor cell SNR = -8.4dB | AWGN | LP-SS |
| {1,8} |  | 2.93 | 2.34 | 2.29 | 2.28 | 2.66 | 2.06 | 1.98 | 1.98 |  |  |  |
| {2,12} |  | 2.76 | 2.09 | 2.03 | 2.00 | 2.43 | 1.78 | 1.72 | 1.67 |  |  |  |
| {2,16} |  | 2.35 | 1.68 | 1.63 | 1.60 | 2.19 | 1.46 | 1.42 | 1.35 |  |  |  |
| {4,16} |  | 3.10 | 2.34 | 2.26 | 2.24 | 2.64 | 1.98 | 1.88 | 1.84 |  |  |  |
| {4,32} |  | 2.33 | 1.69 | 1.66 | 1.63 | 2.24 | 1.50 | 1.45 | 1.41 |  |  |  |
| {1,6} | Opposite sequence | 30.39 | 6.31 | 4.68 | 4.07 | 30.65 | 6.46 | 5.25 | 4.22 |  | TDL-C |  |
| {1,8} |  | 4.21 | 2.57 | 2.35 | 2.17 | 3.93 | 2.33 | 2.22 | 1.99 |  |  |  |
| {2,12} |  | 3.94 | 2.23 | 1.95 | 1.94 | 3.89 | 2.10 | 1.88 | 1.74 |  |  |  |
| {2,16} |  | 3.56 | 1.95 | 1.91 | 1.72 | 3.52 | 1.91 | 1.83 | 1.69 |  |  |  |
| {4,16} |  | 4.06 | 2.53 | 2.23 | 2.10 | 3.85 | 2.28 | 2.05 | 1.87 |  |  |  |
| {4,32} |  | 3.63 | 2.11 | 1.95 | 1.73 | 3.63 | 2.09 | 1.96 | 1.81 |  |  |  |

It can be seen that without considering the cases with opposite sequence for serving and interfering cell as discussed in section 2.1, the target accuracy of 3.5dB can be met for all {M,L} combinations with 2 samples, for both AWGN and TDL-C channels. We suggest to take number of samples for measurement X as [2].

Proposal 3: RAN4 to take samples for LP-SS measurement X as [2].

  - Simulation results for SSB

Our simulation results for SSB measurement are given in Table 2.

Table 2: Simulation results for SSB

| Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | channel | Interference Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSRP error |  |  |  | RSRQ error |  |  |  |  |  |  |
| X=1 | X=2 | X=3 | X=4 | X=1 | X=2 | X=3 | X=4 |  |  |  |
| 3.28 | 2.29 | 2.04 | 1.95 | 2.88 | 2.40 | 2.16 | 2.05 | -3 serving cell SNR = -2.4dB neighbor cell SNR = -8.4dB | AWGN | SSB |
| 4.18 | 2.75 | 2.55 | 2.55 | 3.31 | 2.84 | 2.70 | 2.67 |  | TDL-C |  |

It can be seen that the target accuracy of 3.5dB can be met with 2 samples, for both AWGN and TDL-C channels. We suggest to take number of samples for measurement Y as [2].

Proposal 4: RAN4 to take samples for LP-SS measurement Y as [2].

Conclusions

In this paper we provided our views on simulation assumption and also provided updated simulation results.

Proposal 1: RAN4 to confirm that ideal RSRP is derived from the serving cell signal after the fading channel; ideal RSRQ is derived based on ideal RSRP divided by the actual RSSI.

Proposal 2: LP-SS measurement requirements are not applicable to cases where serving cell and neighbour cell are using opposite binary sequences.

Proposal 3: RAN4 not to align simulation assumption on whether OOK based LR is using I-branch only or using both I- and Q-branch.

Proposal 3: RAN4 to take samples for LP-SS measurement X as [2].

Proposal 4: RAN4 to take samples for LP-SS measurement Y as [2].

Reference

- R4-2504908, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

- R4-2503899, Simulation assumption and additional simulation results for LP-WUR measurement, Huawei, HiSilicon