3GPP TSG-RAN4 Meeting #115	R4-2506791

# Malta, MT, 19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on MR measurement requirements for serving cell in IDLE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to email discussion in RAN4#114-bis, requirements for MR serving cell measurement and evaluation requirements in IDLE are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce requirements for MR serving cell measurement and evaluation requirements in IDLE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Essential requirements for LP-WUR operation are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 4.X.2.3	Measurement and evaluation of serving cell by MR

The requirements in this claue apply for UE measurement and evaluation of serving cell using MR, when LP-WUS UE is not in [RRM offloading mode] as defined in [1].

##### 4.X.2.3.1	Requirements for evaluation of cell selection criterion

When LP-WUS UE is not in [RRM relaxation mode] as defined in [1], the requirements in claue 4.2.2.2 shall apply.

When LP-WUS UE is in [RRM relaxation mode] as defined in [1], the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every 16 DRX cycle.

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by 8 DRX cycles.

If the LP-WUS UE has evaluated according to table 4.X.2.3.1-1 in 16*Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the LP-WUS UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting LP-WUS UE measurement activities.

If the LP-WUS UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the LP-WUS UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where T= 10s.

Table 4.X.2.3.1-1: Nserv

| DRX cycle length [s] | Nserv [number of 16*DRX cycles] |
| --- | --- |
| 0.32 | 4 |
| 0.64 | 4 |
| 1.28 | 2 |
| 2.56 | 2 |

##### 4.X.2.3.2	Requirements for evaluation of LP-WUS UE related conditions

When LP-WUS UE is not in [RRM relaxation mode] as defined in [1], the LP-WUS UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the following LP-WUR related conditions defined in TS 38.304 [1], if configured,

-	[entry condition for LP-WUS monitoring]

-	[entry condition for RRM relaxation]

-	[entry condition for RRM offloading]

-	FFS: exit condition for RRM relaxation

for the serving cell at least once every M1 DRX cycle; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second,

-	otherwise M1=1.

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by DRX cycle/2.

If the LP-WUS UE has evaluated according to table 4.X.2.3.2-1 in Nserv consecutive DRX cycles that the serving cell fulfils the entry condition for [LP-WUR monitoring, RRM relaxation or RRM offloading], the LP-WUS UE shall perform corresponding actions as defined in clause XX in [1].

FFS: If the UE has evaluated according to table 4.X.2.3.2-1 in Nserv consecutive DRX cycles that the serving cell fulfils the exit condition for [RRM relaxation], the UE shall perform corresponding actions as defined in clause YY in [1].

Table 4.X.2.3.2-1: Nserv When UE is not in [RRM relaxation mode]

| DRX cycle length [s] | Nserv [number of DRX cycles] |
| --- | --- |
| 0.32 | M1*4 |
| 0.64 | M1*4 |
| 1.28 | 2 |
| 2.56 | 2 |

When LP-WUS UE is in [RRM relaxation mode] as defined in [1], the LP-WUS UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the following LP-WUR related conditions defined in TS 38.304 [1], if configured,

-	[entry condition for LP-WUS monitoring]

-	[entry condition for RRM offloading]

-	FFS: exit condition for RRM relaxation

for the serving cell at least once every 16 DRX cycle.

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by 8 DRX cycles.

If the LP-WUS UE has evaluated according to table 4.X.2.3.2-2 in 16*Nserv consecutive DRX cycles that the serving cell fulfils the entry condition for [LP-WUR monitoring or RRM offloading], the LP-WUS UE shall perform corresponding actions as defined in clause XX in [1].

FFS: If the LP-WUS UE has evaluated according to table 4.X.2.3.2-2 in 16*Nserv consecutive DRX cycles that the serving cell fulfils the exit condition for [RRM relaxation], the LP-WUS UE shall perform corresponding actions as defined in clause YY in [1].

Table 4.X.2.3.2-2: Nserv when UE is in [RRM relaxation mode]

| DRX cycle length [s] | Nserv [number of 16*DRX cycles] |
| --- | --- |
| 0.32 | 4 |
| 0.64 | 4 |
| 1.28 | 2 |
| 2.56 | 2 |

<End of Change 1>