3GPP TSG-RAN WG4 Meeting #115	R4-2506795

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on other RRM requirements for Rel-19 NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.31.5.2

Document for:	Approval

Introduction

RRM requirements for other requirements in NTN are discussed in RAN4#114-bis, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues for DL coverage enhancement.

- SSB periodicity extension

- SMTC enhancement

- RACH scheduling restriction

In this paper, we will provide our views on RRM requirements for DL coverage enhancement in NTN.

Discussion

SSB periodicity extension

In RAN4#114-bis following is agreed on RRC Re-establishment requirements.

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) Agreement (online): - In RRC re-establishment requirements, for case where the SMTC for the inter-frequency carrier is not provided, the value of TSMTC,i is assumed to be 160ms: - Discuss the applicability of the requirement for with the updated value of TSMTC,i to 160ms after the related RAN1/2 agreement. ## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Agreement (online): - In RRC re-establishment requirements, for the case SSB Ês/Iot < -8dB and unknown NR cell, - Make the following updates - Remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2 for the applicable scenarios, and the value of TSMTC and TSMTC,i is FFS - Tidentify_intra_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier - Tidentify_inter_NR,i: MAX (800 ms, [22] x TSMTC,i) for inter-frequency carrier

The remaining issue is the definition of TSMTC and TSMTC,i for the case when serving cell SSB Ês/Iot is < -8dB. In our view, with Note 1 removed, the value of TSMTC and TSMTC,i should be

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier, and

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier

In current requirements, the assumption is that when serving cell SSB Ês/Iot is < -8dB, serving cell timing may be unreliable, so the SMTC offset based on serving cell timing is not used for searching target cell. On the other hand, the periodicity of the SMTC provided by the NW is reliable and can still be used by UE to perform blind search. When SMTC is not provided for the carrier, based on agreement for Issue 1-2-A, the STMC periodicity is assumed to be 160ms.

Proposal 1: For RRC Re-establishment requirements, for the case when serving cell SSB Ês/Iot is < -8dB, TSMTC and TSMTC,i are

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier, and

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier

On the applicability of 160ms as default SSB periodicity for initial cell selection, RAN1 agreed to introduce FG 65-1, so the updated requirements for RRC Re-establishment should apply only for UE supporting FG 65-1. On the other hand, RAN1/2 did not make any agreement on whether UE should assume 160ms SSB periodicity for initial cell reselection on all carriers, so RAN4 needs to further wait for their conclusions.

Another open issue is related to the paging interruption requirements.

## Issue 1-5: Paging Further discussion: - For the paging interruption requirements for NGSO scenarios when the SSB periods is equal to 160 ms: - No update is needed, and K is 5.

In RAN4#114-bis, some companies also suggested to update the paging interruption requirements. In our understanding, the existing interruption requirements, which are based on configured SMTC periodicity, should apply for all configurable SMTC periodicities including 160ms. We can see the point that the paging interruption time becomes longer with larger SMTC periodicity, but this is the consequence from the trade-off for better coverage, and we do not think RAN4 should tighten the requirements for this trade-off.

Proposal 2: Do not update the requirements for paging interruption for default SSB periodicity extension.

SMTC enhancement

In RAN#129-bis, an LS [2] is sent to RAN4 with following action.

For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

Our view is that it is feasible to support RAN2 agreements in Rel-19 timeframe.

In current NTN requirements, either smtc2 (2 SMTCs with same offset and different periodicities) or smtc4 (up to 4 SMTCs with different offsets and same periodicity) can be supported, but not at the same time based on following agreement from RAN4#101.

Issue 3-1-5: Legacy smtc1 and smtc2 configuration Agreements: RAN4 to not consider legacy smtc2 configuration when multiple SMTCs with different offsets configured for NTN MO.

In our view, the motivation to support 2 SMTC periodicities is for the scenario where different neighbor cells are transmitting SSB with different periodicities, as RAN2 mentioned in the LS that it will be possible to have different SSB periodicities among neighbor cells in the same frequency layer. For this scenario, it is straightforward to re-use the principle of requirements for smtc2 in R15, i.e. different measurement periods apply to different neighbor cells, depending on which SMTC periodicity a neighbor cell is associated to.

Of course, RAN4 can further discuss if any adaptation is needed in the requirements, but we do not see any technical obstacle to support RAN2 agreements in R19.

We also do not see any issue in supporting 6 SMTCs in IDLE/INACTIVE. In R17, the number of SMTCs configured in IDLE/INACTIVE can already exceed the UE capability, and the following requirements apply.

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected.

RAN2 agreed that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases, so the only difference compared to R17 is that UE will select SMTC within its capability out of 6 rather than 4 configured SMTCs. As the SMTC selection is up to UE implementation, we do not see any issue to support 6 configured SMTCs in IDLE/INACTIVE.

Proposal 3: Confirm to RAN2 that it is feasible to support their agreements in Rel-19 timeframe from RAN4 perspective.

RACH scheduling restriction

In RAN4#114-bis, the issue of RACH scheduling restriction was shortly discussed based on [3] but without agreement. In our view, the issue is valid and needs to be addressed by both RAN1 and RAN4.

- Although the objective is for DL coverage enhancement, the issue of insufficient satellite beams that can be simultaneously active is valid for both Tx and Rx. Therefore, beam hopping may also apply for UL.

- Offset of RACH occasion (RO) cannot be configured, but it is always in radio frame 0 or 1 according to Table 6.3.3.2-2 of 38211.

  - If 160ms RO periodicity is used, then only PRACH from UEs in a limited subset of satellite footprints can be received by the NW because in radio frame 0 or 1 the Rx beam is active only for a subset of satellite footprints.

  - If <160ms RO periodicity is used, then for a specific satellite footprint, not all ROs will be valid because the Rx beam is active only in a subset of ROs for this satellite footprin, and it is possible that UE selects an RO that will not be received by the NW as raised up in [3].

For above reasons, we see it meaningful to apply a time offset and duration for valid RO so that UE in each satellite footprint will transmit PRACH on ROs that corresponds to active Rx beam for its satellite footprint. The detailed solution, including how the offset and duration are determined and whether the offset and duration also apply for Tx and Rx of other RACH related signals and channels, can be up to RAN1. Once RAN1 decides on the solution, RAN4 can define scheduling restriction requirements accordingly.

Proposal 4: Send LS to inform RAN1 about the need for time offset and duration for PRACH transmission considering UL beam hopping.

A draft LS is provided in the Annex.

Conclusions

In this paper we provided our views on RRM requirements for DL coverage enhancement in NTN.

Proposal 1: For RRC Re-establishment requirements, for the case when serving cell SSB Ês/Iot is < -8dB, TSMTC and TSMTC,i are

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier, and

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier

Proposal 2: Do not update the requirements for paging interruption for default SSB periodicity extension.

Proposal 3: Confirm to RAN2 that it is feasible to support their agreements in Rel-19 timeframe from RAN4 perspective.

Proposal 4: Send LS to inform RAN1 about the need for time offset and duration for PRACH transmission considering UL beam hopping.

Reference

- R4-2504913, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R2-2503068, LS on SMTC enhancements, RAN2

- R4-2503536, On R19 other NTN RRM requirements, Apple

Annex: draft LS on scheduling restriction for RACH related signals and channels

3GPP TSG-RAN WG4 Meeting #115 R4-250xxxx Malta, MT, 19 – 23 May, 2025 Title: LS on scheduling restriction for RACH related signals and channels Response to: Release: Release 19 Work Item: NR_NTN_Ph3-Core Source: RAN4 To: RAN1 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 is defining requirements for DL coverage enhancement, and it is RAN4 understanding that the satellite beam hopping may also apply in UL. RAN4 is discussing scheduling restriction requirements for PRACH and other Tx and Rx of other RACH related signals and channels, and it is identified that a time offset and duration for PRACH transmission is needed considering UL beam hopping. RAN4 kindly asks RAN1 to decide the details of the time offset and duration for PRACH transmission considering UL beam hopping, and whether the time offset and duration apply to other Tx and Rx of other RACH related signals and channels. 2. Actions: To RAN1: RAN4 kindly asks RAN1 to decide the details of the time offset and duration for PRACH transmission considering UL beam hopping, and whether the time offset and duration apply to other Tx and Rx of other RACH related signals and channels. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #116 August 25 – August 29, 2025 India (TBC), IN RAN WG4 Meeting #116-bis October 13 – October 17, 2025 Prague, CZ