3GPP TSG-RAN WG4 Meeting # 115		                                         R4-2506797

Malta, MT, 19th - 23rd, May, 2025

Title: 					Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	7.18.3.2

Document for:	Discussion

Introduction

The RRM Phase 5 FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC discussion, as outlined in WF[1], was one of the topics during the last meeting. Key contributions from companies are detailed in WF[1]. In this paper, we present our perspectives and contributions regarding FR2-1 L3 measurement delay, focusing on optimising the CSSF outside gap in CA/DC scenarios.

Discussion

In the last meeting, the companies agreed to introduce Solution 1 and Solution 3. For these two solutions, there are still some parts that need to be discussed. There are two points we would like to provide our views which are whether solution 1 is possible to extend to the FR1 and FFS of solution 3.

Solution 1:

At the last meeting, the companies discussed several issues related to Solution 1. The companies had different views on the details of Solution 1 and how to expand the solution to FR1. We will provide our views in the following parts.

Revision on Solution 1

The discussion of the last meeting has been captured in the below from the last meeting’s WF[1].

Issue 1-2-1: revision on solution 1 - Proposal: For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

In our view, we are open to continuing the discussion on the description of Solution 1. This may relate to the network indication format. For example, if the network indication can reconfigure the MOs, the network can configure the SSB-based report on the intended MO. Even when the network configures CSI-RS on multiple MOs without any SSB-based reports, the network will still select one MO for the UE to measure and report on. Therefore, this revised Solution 1 should be updated after the capability design is finalised.

Proposal 1: Discuss this revised Solution 1 update after the indication design is finalised.

Whether this solution can be extended to the FR1?

The following content has been extracted from the last meeting’s WF[1].

Issue 1-2-2: Expand solution 1 to FR1 case [Way forward]: - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbour cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

Solution 1 enables the network to allow UE to measure a single serving CC per band, even when multiple serving CCs’ MOs are configured. This solution is based on the assumption that FR2 CCs experience minimal interference due to the use of directional beams for signal transmission and reception. However, in FR1, signal transmission and reception typically rely on omnidirectional antennas, which are more susceptible to interference. This susceptibility, combined with the fact that intra-band cells are not completely co-located, means that cells within the same band may experience varying levels of interference. Consequently, a single CC cannot accurately represent the measurement results for the entire band in FR1.

Observation 1: Expanding solution 1 to FR1 can only work on some corner cases, such as the co-located case.

In summary, expanding solution 1 to FR1 requires further analysis and more evidence of gains. Our understanding is that solution 1 is not recommended for expansion to the FR1 band.

Proposal 2: Solution 1 is not recommended for expanding to the FR1 band.

Solution 3:

In the last meeting, the following ad hoc agreement about 3-searcher scenarios is captured in WF[1]:

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC) [Way forward]: - FFS: - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF. - Note: 3-searchers will be used by UE if UE report this capability. - FFS: Other restriction (triggered by Huawei): - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple) - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple) - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT) - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

Issue of NW indication for 3-searcher solution

Regarding the introduction of a network indication for a specific SCC to use the third searcher, allocating the third searcher to a single SCC may lead to two issues. On one hand, one SCC would be measured faster than another SCC that requires neighbor cell measurement, which is not logical. Additionally, there is no significant benefit in accelerating the measurement of a single SCC to such an extent.

On the other hand, if the network indicated SCC is the one requiring neighbour cell measurement, it would occupy a total of 1.5 searchers—0.5 searcher shared between FR1 and FR2 SCCs, and the third searcher dedicated to the network indicated SCC.

Overall, this network-indicated approach could introduce implementation challenges for the 3-searcher solution.

Observation 2:  This network-indicated approach could introduce implementation challenges for the 3-searcher solution.

Issue of capabilities

Over the last several meetings, we have provided our analysis on the relationship between the 3-searcher solution and per-FR GAP. In our view, if the UE is configured with per-FR GAP, it may trigger at least three searchers for measurement in certain situations. For example, one searcher would be used for FR1 PCC measurement, a second searcher for FR1 SCC measurement, and an additional searcher for FR2 GAP measurement.

In the last meeting, companies thought linking the 3-searcher solution to the capability of per-FR GAP was confusing and unsuitable. As a result, a new capability for the 3-searcher solution has been approved. However, configuring this new capability together with the per-FR GAP capability may exceed the UE's measurement processing capacity. For instance, if three measurements outside the GAP in FR1 and one within the GAP measurement in FR2 occur simultaneously, the UE would effectively trigger four searchers for measurement. This could impose higher performance requirements on the UE's measurement processing capacity than originally intended. In our view, these two capabilities should be restricted so that only one can be configured at once.

On the one hand, the new 3-searcher requirements are not applicable when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. During this moment, UE will follow legacy 2-searcehr requirements. After this collision, UE will back to the 3-searcher requirement.

Proposal 3: Other restriction for the 3-searcher new capability

The new 3-searcher requirements are not applicable when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. During this moment, UE will follow legacy 2-searcehr requirements. After this collision, UE will back to the 3-searcher requirement.

Conclusions

Proposal 1: Discuss this revised Solution 1 update after the indication design is finalised.

Observation 1: Expanding solution 1 to FR1 can only work on some corner cases, such as the co-located case.

Proposal 2: Solution 1 is not recommended for expanding to the FR1 band.

Observation 2:  This network-indicated approach could introduce implementation challenges for the 3-searcher solution.

Proposal 3: Other restriction for the 3-searcher new capability

The new 3-searcher requirements are not applicable when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. During this moment, UE will follow legacy 2-searcehr requirements. After this collision, UE will back to the 3-searcher requirement.

References

- R4-2504899 WF on RRM requirements for NR_RRM_Ph5_Part1