3GPP TSG-RAN WG4 Meeting #115		R4-2506827

St. Julian’s, Malta, 19th – 23rd May 2025

Agenda item:	7.3.2

Source:	Intel Corporation

Title:	Views on less than 5MHz CA/DC UE features in Rel-19

Document for:	Discussion

# Introduction

The Rel-19 RAN4 UE feature list discussions start in this meeting with the guidance for drafting RAN4 UE feature list as in the below box.

RAN4 Chair: * Single contribution per company is allowed in this agenda with the summary of proposed RAN4 related feature list. After the decision is made under the agenda of the individual WI, the corresponding UE capability will be captured in the Rel-19 feature list under this agenda. * The proponents also need submit the contribution with details to the agenda of the corresponding WI under "7.xx.xx General aspects" and the decision should be made under the agenda of the individual WI. * If there is no corresponding agenda, the proponents can directly submit the contributions with technique details in the dedicated agenda for Rel-19 feature list, and the decision will be made under this agenda. Moderator: It is highly appreciated that rapporteur companies can submit the FGs for your own WI under agenda AI.8. And in your contribution, please clarify the status of each FG, e.g. whether it had already been agreed in previous RAN4 meetings.

In this paper we provide information on the less than 5MHz CA/DC UE features in Rel-19. There is no new UE capabilities signalling needed in TS 38.306 and we propose to record the UE features in the feature list and TR 38.822 in Rel-19.

# Discussion

## Background on Rel-19 less than 5MHz CA/DC UE features

Previous agreements and LS

RAN4 made the agreement shown in the below box and sent an LS on NR channel BW less than 5MHz to RAN2 [1].

RAN4 has had extensive discussions on the UE RF requirements for Rel-19 less than 5MHz UE configured with CA and DC band combinations. RAN4 reached consensus on the associated UE capability signalling aspects as provided below: - UE needs to be able to indicate for each carrier in the CA/DC band combination whether it supports less than 5 MHz channel bandwidth operation for DL/UL - To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination UE needs to indicate UE capabilities defined for NR < 5MHz channel bandwidth in Rel-18 and the existing UE capability applicability restriction to single carrier needs to be removed. - UE also indicates BCS including support of less than 5MHz channel bandwidth - For BCS signalling the following framework is used: - Traditional BCSs (i.e., not BCS4 or BCS5): UE is expected to support 3MHz channel bandwidth in the band in the band combination if it indicates a corresponding BCS, which includes 3MHz channel bandwidth in RAN4 specifications. No signalling changes are expected by RAN4. - BCS4: if UE indicates support for BCS4 including a band that it also supports 3 MHz channel bandwidth for, then it shall support 3 MHz for CA for that band in the band combination. No signalling changes are expected by RAN4. - BCS5: if UE indicates support for BCS5 and minimum channel bandwidth of 3MHz as a part of BCS signalling, then it shall support 3 MHz for CA for that band in the combination. The existing BCS5 signalling framework needs to be modified to allow indication of 3 MHz as minimum channel bandwidth. - It should be possible for UE to indicate support of CA/DC with less than 5MHz channel bandwidth starting from Rel-18 (i.e., allow early implementation from Rel-18) - RAN4 has not identified a need for new signaling element. The signalling details are up to RAN2.. The specific UE feature groups for NR less than 5 MHz channel bandwidth, which have single carrier restriction are listed below: - support12PRB-CORESET0-r18 - support3MHz-ChannelBW-Asymmetric-r18 - support3MHz-ChannelBW-Symmetric-r18 - support5MHz-ChannelBW-20PRB-CORESET0-r18 - support12PRB-CORESET0-GSCN-41637-r18

In response to the LS RAN2 had successfully made corrections in the specifications TS 38.331 and TS 38.306 to accommodate UE capabilities reporting according to the RAN4 agreements. In particular, the agreements below were made in RAN2 #128 and CRs [2-3] were agreed.

- Extend the supportedBandwidthDL/UL to include 3MHz. - On the per band capabilities, do not indicate the 3M in the channelBWs-DL/UL but keep the support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18; - Extend supportedMinBandwidthDL/UL-r17 to include 3MHz - Remove the single carrier restriction in the field description of support5MHz-ChannelBW-20PRB-CORESET0-r18 and support12PRB-CORESET0-GSCN-41637-r18.

There is no further need to have any change in the RAN2 specifications TS 38.331 and TS 38.306 on Rel-19 signalling. The existing capabilities signalling specified in Rel-18 were updated to remove single carrier constraints and can be used for Rel-19 UE features.

Observation: No further change is needed in TS 38.331 or in TS 38.306 for Rel-19 less than 5MHz CA/DC UE features.

## UE feature list implementation

RAN4 UE feature list and TR 38.822

TR 38.822 captures the RAN4 UE feature list in every release of specifications since Rel-15. In Rel-19 though there is no need to specify any new signalling further in the RAN2 specifications, we can still make record in the RAN4 UE feature list and further implement into TR 38.822 the description of UE less than 5MHz CA/DC features; and it is necessary to make clear that no new signalling is required together with the Rel-19 UE feature groups. See the below proposal for RAN4 UE feature list in Rel-19.

Table: Rel-19 RAN4 UE Feature List for NR_FR1_lessthan_5MHz_BW_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. NR_FR1_lessthan_5MHz_BW_Ph2 | X-1 | Support of less than 5MHz CBW operation in CA/DC | - Support for 3 MHz symmetric channel bandwidth in DL and UL in CA/DC - Support for 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL in CA/DC - Support 12 PRB CORESET0 in CA/DC - Support 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637 in CA/DC - Support 5 MHz channel bandwidth with 20 PRB CORESET0 in CA/DC | Layer-1 UE feature group 51-1, 51-1a, 51-2a, 51-2b, 51-3 | Yes |  | The UE does not support corresponding UE features in CA/DC operations | NA (see Note column) | FDD only | FR1 only | NA | All components in the feature group do not require new capabilities signalling definition in Rel-19 and respective Rel-18 capability signalling is reused based on RAN2 agreements. | Optional |

Proposal #1:	Implement the Rel-19 RAN4 UE feature groups for less than 5MHz CA/DC and note that there is no new signalling specified.

One alternative is to leave it blank in the Rel-19 UE feature list and in TR 38.822 for Rel-19 less than 5MHz CA/DC UE features; this leads to no spec record on the Rel-19 UE feature discussions for the work item either internally in 3GPP RAN or externally, which may lead to certain ambiguity and is undesirable.

# Conclusion

In this paper we provided information on the background of prior discussions on the less than 5MHz CA/DC UE features in Rel-19. There is no new UE capabilities signalling needed in TS 38.306 and TS 38.331 and we propose to record the UE features in the feature list and TR 38.822 in Rel-19.

Proposal #1:	Implement the Rel-19 RAN4 UE feature groups for less than 5MHz CA/DC and note that there is no new signalling specified.

# References

- R4-2417119 “LS on Rel-19 NR channel BW less than 5MHz for FR1”, Intel Corporation, RAN4 #112bis, October 2024

- R2-2411163 CR to TS 38.331 “Correction on the Less than 5M Bandwidth”, ZTE Corporation, vivo, RAN2 #128, November 2024

- R2-2411164 CR to TS 38.306 “Correction on the Less than 5M Bandwidth”, ZTE Corporation, vivo, RAN2 #128, November 2024