3GPP TSG-RAN WG4 Meeting # 114bis											R4-2506844

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	draft TP to TS 38.194 on CW frequency error and unwanted emssion

Agenda Item:	7.26.3.3

Document for:	Approval

- Introduction

In this contribution, we update the TR based on the agreement in the last meeting.

- Text proposal

## <<Start of Change>>

## 7.3	Frequency error

#### 7.3.1	General

The requirements in clause 7.3 apply to the transmitter ON period.

Frequency error is the measure of the difference between the actual carrier wave transmit frequency and the assigned frequency. This requirement shall be applied at the antenna connector supporting transmission in the operating band.

#### 7.3.2	Minimum requirement

The frequency of carrier wave measurements shall be accurate to within ±0.1 ppm observed over a period of 1 ms compared to the carrier frequency declared by the manufacturer.

## 7.5	Unwanted emission

#### 7.5.1	Phase noise

The phase noise is the unwanted emissions outside the centre frequency of carrier wave resulting from random fluctuations in the phase of signal in the transmitter but excluding spurious emissions.

The phase noise of the CW applies to frequencies (ΔfOOB) starting from the ±[1]kHz to ±[2016] kHz offset from centre frequency of carrier wave declared by the manufacturer. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 7.5.2 are applicable.

The power of CW transmitter shall not exceed the levels specified in Table 7.5.1-1

Table 7.5.1-1: CW phase noise emission limit

| ΔfOOB (kHz) | Phase noise emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 1-8.5 | [-30] | [7.5 kHz] |
| ± 8.5-2016 | [-40] |  |

#### 7.5.2	Spurious emission

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfOOB below the centre frequency of carrier wave declared by the manufacturer, up to ΔfOOB above centre frequency of carrier wave declared by the manufacturer, where the ΔfOOB is defined in table 7.5.1-1. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [x].

The basic limits of either table 7.5.2-1 (Category A limits) or table 7.5.2-2 (Category B limits) shall apply. Unless otherwise stated, all requirements are measured as mean power (RMS).

Table 7.5.2-1: General CW transmitter spurious emission limits in FR1, Category A

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -13 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | -13 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz.. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies for Band for which the upper frequency edge of the DL operating band is greater than 5.2 GHz. |  |  |  |

Table 7.5.2-2: General CW transmitter spurious emission limits in FR1, Category B

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz | -36 dBm | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz |  | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | - 30 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies for Band for which the upper frequency edge of the DL operating band is greater than 5.2 GHz. |  |  |  |

## <<End of Change>>

- Reference