3GPP TSG-RAN WG4 Meeting # 115	R4-2506856

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	NR NTN HPUE Tx Requirements

Agenda Item:	7.10.3.2.1

Document for:	Approval

Introduction

In RAN4#114, the summary and WF in NR NTN HPUE requirements is in [1][2]. A comprehensive discussion paper for Tx is in [3].

In this paper, further discussion was made for NR-NTN. It is noted that the duty cycle related discussion is not included here but put to another paper for Iot-NTN Tx.

Requirements Applicability

Current condition:

In RAN4#114, considerable progress has been made on the requirements that would be defined, and almost all the cases have been set except a few cases. an updated requirements applicability was provided in [3] which is also copied below:

Table 1: Current requirements applicability in R19 (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [4Tx] | Non-handheld |
| PC1.5 | FFS if time allows |  |
|  | 2Tx | Non-handheld |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

In RAN4#114bis, no new agreements on these issues were made, and those requirements applicabilities were not changed for PC1/2/1.5. The following observation from [3] can be reused:

Observation 1 (Re-submit): There are two remaining unconfirmed scenarios for requirements definition:

- cases other than 2Tx non-handheld in PC1.5

- 4Tx for PC1 non-handheld

For PC2, considering typical implementation that single Tx is usually more preferred, especially for NTN, some down selections to 1Tx were discussed. However, no new agreements were made. Currently, still both 1Tx/2Tx can be considered. Since this is aligned with what FDD HPUE had defined in TN, and indeed the components industry may still not that ready, it is ok to keep the current agreements.

For PC1.5, there was some discussion on introducing 1Tx for non-handheld. However, this was strongly objected by some other parties, since no counter parts in TN and relatively untypical implementation. Considering this is already a compromise after long discussion, and potentially more work will be introduced, it is proposed to restrict the PC1.5 requirements to what currently already defined. The following proposal is re-submitted:

Proposal 1a (Re-submit): Do not consider defining requirements other than 2Tx non-handheld in Rel-19 for PC1.5.

For PC1 non-handheld, the 4Tx was raised in architecture. The main reason for this architecture is this can reuse the current PC2 components which is for handheld ecosystem, which might be easier/economical in implementation even it is 4Tx.

However, no further agreements can be reached yet. Since 4Tx requirements already defined in TN for PC1.5, it looks also possible to quickly setting up a set of requirements without further delay. So adding one more implementation may also be possible.

Proposal 1b (Re-submit): 4Tx PC1 non-handheld requirements can be further discussed.

HPUE Impact for Tx Requirements

PC2 Power boosting

In last meeting, power boosting were studied in [4], in which many waveforms were studied.

It should be noted the current discussion is quite different from what was studied and defined for power boosting in Rel-18. In Rel-18, the defined power boosting was finally restricted to inner region, and only a modest 1dB for PC3 and 0.5dB for PC2 were defined. By comparison, some of the boosting here discussed were much larger, and close to PC1.5.

Observation 2: Current TN power boosting defined for Rel-18 coverage enhancement is quite conservative and may not help much for NTN.

Considering there are only two meetings left, and it may be challenging to do extensive alignment work, we think it might be difficult to go beyond what was defined for Rel-18. However, considering the conservative 0.5dB boosting value, reusing Rel-18 boosting framework may also not that useful.

Based on this situation, maybe postpone the power boosting to next release is more appropriate action. If some boosting to be introduced into this release, reusing Rel-18 framework might be another compromise.

Proposal 2: Prefer to postpone the power boosting in NTN to next release. If something is needed in this release, reusing Rel-18 power boosting framework can be a compromise.

PC1 ACLR

In last meeting, the ACLR discussion for PC1 has been discussed for a long time. And the following tentative agreement is as following:

Tentative agreement:

- PC1 ACLR is [34] dBc

  - Companies to further evaluate this, and to be confirmed in next meeting.

This is a compromise value that have considered the co-existence study, legacy TN PC1 ACLR (37dBc), TN/NTN PC2 ACLR (31dBc). The target is try to cover the implementation of 4Tx using legacy TN components, and also satisfy more strict co-existence for NTN. There were also proposals to use different values for different implementations, but this may create new problems, thus were not be agreed by the group. We think this is by far the best compromise that can be done, and can be used as the agreement.

Proposal 3: Define PC1 ACLR as 34dBc.

PC1 MPR

Currently one of the main difficulties in defining MPR requirements for PC1 is that there are two different possible reference architectures: 1Tx and [4Tx].

For 1Tx, it is based on a high power PA and a duplexer can be operated in the power level of PC1 + front end IL, which might up to 35dBm or even higher. This is considerably higher than the current duplexer in smartphone can do, and PAs and duplexer which is different from the current smartphones may need to be considered.

Observation 3: Different architectures need to be considered for the derivation of PC1 MPR.

In this case, traditionally the PC3 MPR is used as starting point, since these components are deemed to have more power capabilities and possible larger form factors.

Proposal 4a. For 1Tx architecture, PC1 MPR may use PC3 MPR as starting point.

For proposals using 4Tx, the current smartphone components were considered. The MPR would have to consider more requirements such as those defined for lower power classes as starting point. However, considering the possible [34]dBc ACLR, more MPR is also expected.

Proposal 4b. For possible 4Tx architecture, PC1 MPR may consider PC2 and/or 4Tx PC1.5 requirements as starting point.

Power reduction beyond certain duty cycle

In last meeting, there is following issue:

Issue X: Proposal to allow power reduction beyond certain duty cycle

Way forward:

- Encourage companies to further discuss the following proposal:

  - Option 1: For NR NTN PC2 and PC1.5, and IoT NTN PC2, allow but not mandate UE to reduce it’s transmit power when 50% Tx duty cycle in a radio frame is exceeded. (Qualcomm)

It should be noted, the power back off currently is only defined in TN, and not NTN. So there is not a mandate power back off in case of duty cycle larger than 50%, and no ground on “allow but not mandate”.

Current in SAR related discussion, there is still duty cycle related discussion. But there is not too much progress and agreements so far.

Observation 4a. There is currently no mandate or optional power backoff related to duty cycle defined in NTN.

For TN operation, the current power backoff related to duty cycle was discussed in TDD context, and there was multiple agreements before that for FDD HPUE, the duty cycle would not considered [5].

To accommodate the SAR limits of the NR PC2 FDD High Power UE, both UE-based and network-based solutions are considered in the study phase. For UE-based solution, the UE implementation based mechanism is used to ensure SAR compliance. In addition to UE-based solution, an optional method of reporting duty-cycle capability was also intensively discussed, but there is no conclusion reached.

In addition, although there was also draft CR try clarify this but not agreed such as [6], still there was basic understanding this is actually for TDD. If more concerns were raised, this should belong to a maintenance issue for TN, and should not be discussed in NTN scope.

Observation 4b: Duty cycle related backoff currently defined in TN spec belongs to maintenance issue and should be discussed in NTN.

Proposal 5: Clarify that no duty cycle related backoff has been defined for NTN yet.

Conclusion

In this paper, the following observations and proposals are provided:

Requirement applicability

Table 1: Current requirements applicability in R19 (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [4Tx] | Non-handheld |
| PC1.5 | FFS if time allows |  |
|  | 2Tx | Non-handheld |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Observation 1(Re-submit): There are two remaining unconfirmed scenarios for requirements definition:

- cases other than 2Tx non-handheld in PC1.5

- 4Tx for PC1 non-handheld

Proposal 1a(Re-submit): Do not consider defining requirements other than 2Tx non-handheld in Rel-19 for PC1.5.

Proposal 1b(Re-submit): 4Tx PC1 non-handheld requirements can be further discussed.

PC2 Power boosting

Observation 2: Current TN power boosting defined for Rel-18 coverage enhancement is quite conservative and may not help much for NTN.

Proposal 2: Prefer to postpone the power boosting in NTN to next release. If something is needed in this release, reusing Rel-18 power boosting framework can be a compromise.

PC1 ACLR

Proposal 3: Define PC1 ACLR as 34dBc.

PC1 MPR

Observation 3: Different architectures need to be considered for the derivation of PC1 MPR.

Proposal 4a. For 1Tx architecture, PC1 MPR may use PC3 MPR as starting point.

Proposal 4b. For possible 4Tx architecture, PC1 MPR may consider PC2 and/or 4Tx PC1.5 requirements as starting point.

Power reduction beyond certain duty cycle

Observation 4a. There is currently no mandate or optional power backoff related to duty cycle defined in NTN.

Observation 4b: Duty cycle related backoff currently defined in TN spec belongs to maintenance issue and should be discussed in NTN.

Proposal 5: Clarify that no duty cycle related backoff has been defined for NTN yet.

- References

[1] R4-2504681, Topic summary for [114bis][123] NR_IoT_NTN_HPUE_part1, Moderator (Samsung), RAN4#114bis

[2] R4-2505113, WF on HPUE RF requirements for NR-NTN, Samsung, RAN4#114bis

[3] R4-2503840, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114bis

[4] R4-2504610, Initial power boosting result for NR-NTN for a PC2 NR PA, vivo, RAN4#114bis

[5] TR 38.861, Study on high power UE (power class 2) for one NR FDD band

[6] R4-2213365, Clarification of duty cycle not applying to FDD bands, Huawei, HiSilicon, RAN4#104-e