3GPP TSG-RAN WG4 Meeting # 115	R4-2506857

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	NR NTN HPUE Rx Requirements

Agenda Item:	7.10.3.2.2

Document for:	Approval

Introduction

In the previous meeting, progress was made on PC2 NR-NTN RSD as following in the WF in [2]:

Issue 1-10: General discussion of RSD analysis

Ad-hoc agreements:

- In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation.

Issue 1-11: General discussion of RSD of PC1.5 and PC1

Ad-hoc agreements:

- Companies agree to bring more analysis for the RSD values for PC1.5/PC1 non-handheld UE.

  - Further check these in next meeting.

Issue 1-13: RSD requirements for PC2 on n255

Ad-hoc agreements:

- For n255 PC2:

|  | 5MHz | 10MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
| RSD@1Tx | [0.8] | [0.8] | [0.9] | [0.9] |
| RSD@2Tx | [1.6] | [1.6] | [1.7] | [1.7] |

Issue 1-14: RSD requirements for PC1.5 and PC1.

Way forward:

- Encourage companies to further discuss the following proposals:

  - Option 1: For PC1.5 2Tx (MTK)

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 2.2 | 2.2 | 2.2 | 2.2 |
| n255 | 3.2 | 3.2 | 3.3 | 3.3 |
| n254 | 2.2 | 2.2 | 2.2 |  |

  - Option 2: For PC1 1Tx (MTK)

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3] | [3] | [3] | [3] |
| n254 | [3] | [3] | [3] |  |

There are still open issues for all the power classes. In this paper, some of the proposals and analysis are provided.

HPUE Impact for NR-NTN RSD

General

Similar to [3] which submitted in last meeting, it should be noted that the scenarios needed to be defined for RSD is related to what Tx requirements would be defined, and here is a table from [4] in Tx part:

Table 1: Current requirements applicability in R19 (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [4Tx] | Non-handheld |
| PC1.5 | FFS if time allows |  |
|  | 2Tx | Non-handheld |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Observation 1 (Re-submission): RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

For the differentiation of handheld and non-handheld, the main parameter in SD derivation are antenna isolation. However, as discussed before, more simplified requirements would bring a lot of merits, and unified requirements for handheld and non-handheld is preferred.

Observation 2(Re-submission): Unified requirements for handheld and non-handheld are preferred as much as possible.

PC1.5

Currently, 2Tx for non-handheld is confirmed for PC1.5, and the requirements need to be defined for all the bands.

Most of the assumptions If PC1.5 for 2Tx could be agreed to be defined, then it should not be too difficult for PC1.5 2Tx RSD derivation, since its similarity to PC2 2Tx in the derivation process.

Observation 2a (re-submission): Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx, if defined.

Furthermore, although only non-handheld for 2Tx PC1.5 considered currently, it should be noted that it may still be possible to extend the use case of 2Tx PC1.5 to handheld UE type in some later stage as long as implementation is possible. In this case, keep more aligned assumptions may bring some simplicity in the future and thus preferred.

Observation 2b (re-submission): For PC1.5, although only non-handheld and 2Tx is considered currently, try to use assumptions also applicable for handheld may also meaningful.

PC1

According to early discussion in [3], following observations are submitted:

Observation 3a (re-submission): Currently PC1 is only for 1Tx and non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 3b (re-submission): For PC1 1Tx, if following de-sense analysis in 37.880, the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands, and 0dB RSD can be deduced.

However, still non-zero value is proposed. We are not sure what kind of assumption is made for the duplexer and other components, and detailed analysis seems would be different compared to PC2/PC1.5. So if non-zero value proposed, then more analysis are needed.

In addition, since PC1 for NR-NTN is not likely to be implemented in handheld type UEs even in the near future, future commonality with handheld may not necessary.

Observation 3c (re-submission): PC1 for NR-NTN is not likely to be implemented in handheld type UEs even in the near future, future commonality with handheld may not necessary.

However, it is admitted that the example components such as cavity filter used in the 37.880 is very large form factor. Since it is in the order of centimetres and the thermal effects is also not that good, it is quite challenging even for some FWA devices. In other hand, the thermal effects is also challenging and for cavity filter.

In another direction, the current technology for mobile phone duplexers is also evolving. E.g. BAW and FBAR is improving the power level, and the margin shortage compared to what PC1 needs is shrinking. With the price of cost, IL, and area, there are still room for future improvement based on current technology. So it might be also reasonable to keep some more possibilities to use the technology originated from smart phone duplexers, by introducing some relaxations for PC1.

Observation 4 (New): Introduce some relaxation for future RSD for non-handheld may also be reasonable.

Proposal 1 (New): More possibilities of reference components and related relaxations might be considered for PC1. However, if no more and agreements on this can be reached, 0dB can still be used since this is in line with legacy cases.

For 4Tx cases, since the target is reusing current smartphone components, there could be different components assumptions and thus the RSD analysis is also different

Proposal 2 (Re-submission): FFS for PC1 with 4Tx cases.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Observation 1(Re-submission): RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

Observation 2(Re-submission): Unified requirements for handheld and non-handheld are preferred as much as possible.

PC1.5

Observation 2a(Re-submission): Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx, if defined.

Observation 2b(Re-submission): For PC1.5, although only non-handheld and 2Tx is considered currently, try to use assumptions also applicable for handheld may also meaningful.

PC1

Observation 3a(Re-submission): Currently PC1 is only for 1Tx and non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 3b(Re-submission): For PC1 1Tx, if following de-sense analysis in 37.880, the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands, and 0dB RSD can be deduced.

Observation 3c(Re-submission): PC1 for NR-NTN is not likely to be implemented in handheld type UEs even in the near future, future commonality with handheld may not necessary.

Observation 4 (New): Introduce some relaxation for future RSD for non-handheld may also be reasonable.

Proposal 1 (New): More possibilities of reference components and related relaxations might be considered for PC1. However, if no more and agreements on this can be reached, 0dB can still be used since this is in line with legacy cases.

Proposal 2(Re-submission): FFS for PC1 with 4Tx cases.

- References

[1] R4-2504681, Topic summary for [114bis][123] NR_IoT_NTN_HPUE_part1, Moderator (Samsung), RAN4#114bis

[2] R4-2505113, WF on HPUE RF requirements for NR-NTN, Samsung, RAN4#114bis

[3] R4-2503841, NR NTN HPUE Rx Requirements, vivo, RAN4#114bis

[4] R4-2503840, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114bis