3GPP TSG-RAN WG4 Meeting # 115	R4-2506864

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion of UE RF impact for LB-LB switching

Agenda Item:	7.7.3

Document for:	Approval

Introduction

In RAN#106, a new WID [1] was approved on Low band carrier aggregation via switching. A motivation paper including some pictures was provided in [2].

In RAN4#114, most discussions were made focus on switching time for low NR band carrier aggregation via switching. Some other issues were listed but not discussed. The main results are WF [4] and LS [5].

In RAN4#114bis, more agreements were reached, and the WF and summary are in [7][8].

In this paper, further discussions were made.

Discussion

Basic scenario

For the basic switching behavior studied, the following figure from [2] was also partly copied here:

It should be noted the two cases for switching can be interpreted as two states:

- State1: The UE operates in both Tx and Rx on the FDD band.

- State2: The UE operates only in Rx mode on the SDL band.

DL interruption & Timing analysis

In last meeting, several contributions provide detailed analysis on the interruption, and possible arrangement of the switching gap which is likely to be considered for RAN1, such as in [10][11].

Generally, those analysis are most similar, though some difference exist in details. There are still different understandings on:

- Where the interruptions should be located, whether in Pcell FDD, Scell SDL, or configurable;

- Whether requirements should reflect all those parameters. E.g. whether include TA, MRTD in RAN4 requirements illustrations.

In legacy similar cases of Tx Switching, not all those parameters and conditions been defined as ON/OFF mask. Of course, legacy cases are also simpler compared to current condition.

This part is actually quite RAN1 dependent, and it may not be necessary to specify every details of as part of time mask requirements.

Proposal 1: It may not be necessary to provide detailed interruption pictures as part of ON/OFF mask including TA and MRTD etc.

Time Mask requirements

A lot of proposals for time mask requirements have been provided in last meeting. Some of them are quite simple such as in [12], in which only switching periods were reflected. Some have considered a lot more such as in [10], in which TA and MRTD were also considered. This issue has been discussed in previous issue.

It should be noted that a lot issues are still RAN1 dependent, and previous analysis are preliminary.

Observation 1: Many scenarios are still depending on RAN1 discussion, such as the location of switching period.

Furthermore, it was found the 10us “transient period’ in SDL were also considered in some contributions. However, the 10us “transient period” was defined before as a Tx requirement in general ON/OFF mask, and there were no analysis on whether there would be such requirement for Rx before.

Observation 2: There was no “transient period” studied/defined for Rx currently which corresponds to 10us general ON/OFF mask for the transmitter requirements.

Even if such a “transient period” is needed for receiver to turn on, it is likely to be shorter than the Tx counterpart. Considering this is rather short, and the fairly difficulty on verification of short disruption of Rx, it may not be that useful or reasonable to define this for Rx, unless new study is conducted.

Proposal 2: Do not consider additional “transient period” for receiver which corresponds to 10us transient period defined currently for Tx ON/OFF mask.

Based on the previous proposals, one tentative baseline for time mask might be as following ,which is based on [10][13] with minor adjustment for the case that switching period located in FDD PCell.

Proposal 3: The following two figures can be starting point for time mask requirements (latter referenced from [10]):

Figure 1:	Time mask with the switching period located on the FDD carrier

Figure 2:	Time mask with the switching period located on the SDL carrier (Copied from [10])

Rx testing

In last meeting, the testability issue was mentioned in [7][9] for Rx, but no solution provided. It is realized there are some difficulty on demodulation tests for such short time periods in the order of tens of microseconds.

Basically there were three options.

Option 1: By means of RRM tests.

This may involve some specific behaviour or signal transmission defined and measured. However, this would be in the scope of RRM discussion and difficult to be discussed here in RF agenda.

Option 2: Using demodulation tests.

In order for demodulation to be effective, thousands or tens of thousands blocks are needed. This many need to introduce some specific test pattern, and duplicate the switching period for multiple (e.g. may at least thousands of) times for a specific resource, to measure the throughput or BLER. However, this procedure would need to be carefully designed, since the demodulation tests may not be that easy to differentiate the performance difference for the degradation in one or two symbols.

Option 3: Do not test Rx switching period.

If option 1 and 2 do not work well, we may have to go this direction that do not test Rx switching period. This may also reasonable to a certain degree, in that if this is difficult to be verify, the actual impact to the real deployment may also not considerable.

It is proposed to further study these options:

Proposal 4: Further study the following options for Rx testability issue.

- Option 1: By means of RRM tests.

- Option 2: Using demodulation tests.

- Option 3: Do not test Rx switching period.

Conclusion

In this paper, a preliminary analysis has been done for Low band carrier aggregation via switching. The following observations and proposals have been provided:

DL interruption & Timing analysis

Proposal 1: It may not be necessary to provide detailed interruption pictures as part of ON/OFF mask including TA and MRTD etc.

Time Mask requirements

Observation 1: Many scenarios are still depending on RAN1 discussion, such as the location of switching period.

Observation 2: There was no “transient period” studied/defined for Rx currently which corresponds to 10us general ON/OFF mask for the transmitter requirements.

Proposal 2: Do not consider additional “transient period” for receiver which corresponds to 10us transient period defined currently for Tx ON/OFF mask.

Proposal 3: The following two figures can be starting point for time mask requirements (latter referenced from [10]):

Figure 1:	Time mask with the switching period located on the FDD carrier

Figure 2:	Time mask with the switching period located on the SDL carrier (Referenced from [10])

Rx testing

Proposal 4: Further study the following options for Rx testability issue.

- Option 1: By means of RRM tests.

- Option 2: Using demodulation tests.

- Option 3: Do not test Rx switching period.

- References

[1] RP-243317, New WID on low band carrier aggregation via switching, Apple, RAN#106

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, RAN#106

[3] R4-2502853, Topic summary for [114][126] NR_LBCA_Sw, Moderator(Apple), RAN4#114

[4] R4-2503016, WF on low-low band switching, Apple, RAN4#114

[5] R4-2502877, LS on switching periods for low-low band switching, AT&T, RAN4#114

[6] R4-2500768, Discussion of UE RF impact for LB-LB switching, vivo, RAN4#114

[7] R4-2505223, WF on UE RF requirements for LB CA via switching, Apple, RAN4#114bis

[8] R4-2504679, Topic summary for [114bis][121] NR_LBCA_Sw, Moderator(Apple), RAN4#114bis

[9] R4-2503845, Discussion of UE RF impact for LB-LB switching, vivo, RAN4#114

[10] R4-2504190, RF requirements for low-band CA via switching, Ericsson, RAN4#114bis

[11] R4-2503336, LB (FDD) – LB (SDL) CA with switching, Xiaomi, RAN4#114bis

[12] R4-2503194, UE RF requirements for LB CA via switching, Apple, RAN4#114bis

[13] R4-2503731, Discussion on UE RF requirements for LB-LB CA switching, Spreadtrum, RAN4#114bis