3GPP TSG-RAN WG4 Meeting #115	R4-2506890

St. Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:	8

Source:	Intel Corporation

Title:	Views on RAN4 Rel-19 UE feature list

Document for:	Discussion

# Introduction

The Rel-19 RAN4 UE feature list discussions start in this meeting with the guidance for drafting RAN4 UE feature list as in the below box.

RAN4 Chair: * Single contribution per company is allowed in this agenda with the summary of proposed RAN4 related feature list. After the decision is made under the agenda of the individual WI, the corresponding UE capability will be captured in the Rel-19 feature list under this agenda. * The proponents also need submit the contribution with details to the agenda of the corresponding WI under "7.xx.xx General aspects" and the decision should be made under the agenda of the individual WI. * If there is no corresponding agenda, the proponents can directly submit the contributions with technique details in the dedicated agenda for Rel-19 feature list, and the decision will be made under this agenda. Moderator: It is highly appreciated that rapporteur companies can submit the FGs for your own WI under agenda AI.8. And in your contribution, please clarify the status of each FG, e.g. whether it had already been agreed in previous RAN4 meetings.

This paper provides rapporteur’s input on NR less than 5MHz CA/DC UE features in Rel-19 and also the summary of company proposals on the feature lists for Rel-19 RRM enhancements and Rel-19 NES enhancements WIs

# NR_FR1_lessthan_5MHz_BW_Ph2 (rapporteur input)

RAN4 already discussed the UE capabilities signalling for NR_FR1_lessthan_5MHz_BW_Ph2 in the WI Core part and the corresponding signalling was introduced by RAN2 as explained in more details in [1]. Meantime, RAN4 has not yet concluded on the associated UE feature list and the rapporteur proposal on the associated feature list is provided in the table below (same as proposed in [1] submitted for the WI agenda).

NOTE to moderator: The respective feature list has not been formally agreed yet.

Table 1: Rel-19 RAN4 UE Feature List for NR_FR1_lessthan_5MHz_BW_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. NR_FR1_lessthan_5MHz_BW_Ph2 | X-1 | Support of less than 5MHz CBW operation in CA/DC | - Support for 3 MHz symmetric channel bandwidth in DL and UL in CA/DC - Support for 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL in CA/DC - Support 12 PRB CORESET0 in CA/DC - Support 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637 in CA/DC - Support 5 MHz channel bandwidth with 20 PRB CORESET0 in CA/DC | Layer-1 UE feature group 51-1, 51-1a, 51-2a, 51-2b, 51-3 | Yes |  | The UE does not support corresponding UE features in CA/DC operations | NA (see Note column) | FDD only | FR1 only | NA | All components in the feature group do not require new capabilities signalling definition in Rel-19 and respective Rel-18 capability signalling is reused based on RAN2 agreements. | Optional |

# Netw_Energy_NR_enh

The detailed proposals on the UE feature list for Netw_Energy_NR_enh WI are provided in [2] and are summarized in the table below:

Table 2: Rel-19 RAN4 UE feature list for Netw_Energy_NR_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. Netw_Energy_NR_enh | 54-1 | UE support on no lower bound for measurement periodicity during measurements on OD-SSB in the FMW | - 1) UE capability to whether no lower bound of measurement periodicity when measuring on OD-SSB within Fast Measurement windows is supported | TBA | Yes | NA | The UE applies lower bound of [10] ms measurement periodicity for OD-SSB measurements within FMW | Per UE | No | No | NA |  | Optional with capability signalling |
|  | 54-2 | UE capabilities on the supported additional processing time during deactivated SCell measurements based on OD-SSB | - 1) UE supported value of additional processing time during deactivated SCell measurements based on OD-SSB | TBA | Yes | NA | The UE applies 5ms additional processing time during deactivated SCell measurements based on OD-SSB | Per UE | No | No | NA |  | Optional with capability signalling |

# NR_RRM_Ph5

The detailed proposals on the UE feature list for NR_RRM_Ph5WI are provided in [3] and are summarized in the table below:

Table 3: Rel-19 RAN4 UE feature list for NR_RRM_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_RRM_Ph5 | 51-1 | UE support on solution 1 for CSSF outside gap enhancements | - 1) UE support for reduced CSSF outside gap in CA, NR-DC, EN-DC and NE-DC | NA | Yes | NA | The UE does not support reduced CSSF outside gap value in CA or NR-DC or EN-DC or NE-DC | Per-UE | NA | FR2 only | NA |  | Optional with capability signalling |
|  | 51-2 | UE support on solution 3 for CSSF outside gap enhancements | - 1) UE support for 3-searcher assumptions in FR1 only CA and/or NR-DC - 2) UE support for 3-searcher assumptions in FR1 only EN-DC - 3) UE support for 3-searcher assumptions in FR1+R2 CA - 4) UE support for 3-searcher assumptions in FR1+FR2 NR-DC | NA | Yes | NA | The UE does not support reduced CSSF outside gap solution 3 | Per-UE | NA | NA | NA |  | Optional with capability signalling |
|  | 51-3 | UE support on the reduced beam sweeping factor N during L3 measurements and mobility procedures | - 1) UE supported value of reduced beam sweeping factor N during L3 measurements and mobility procedures | NA | Yes | NA | The UE does not support reduced beam sweeping factor | Per-UE | NA | NA | NA | Candidate values: {2, 4, 6} | Optional with capability signalling |

# Conclusion

This paper provides our updated views on details of UE feature list for selected work items in Section 2. It is proposed to endorse the proposed feature lists in

- Table 1. Rel-19 RAN4 UE feature list for NR_FR1_lessthan_5MHz_BW_Ph2 WI.

- Table 2. Rel-19 RAN4 UE feature list for Netw_Energy_NR_enh WI.

- Table 3. Rel-19 RAN4 UE feature list for NR_RRM_Ph5 WI.

# References

[1] R4-2506827 “Views on less than 5MHz CA/DC UE features in Rel-19”, Intel Corporation, RAN4 #115, May 2025

[2] R4-2506824 “Discussion on RRM requirements for On-demand SSB SCell operations”, Intel Corporation, RAN4 #115, May 2025

[3] R4-2506828 “Views on UE features for Rel-19 RRM enhancements WI”, Intel Corporation, RAN4 #115, May 2025