# 3GPP TSG-RAN WG4 Meeting #115	R4-2506899

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	9.3

Source: 	Ericsson

Title:	Discussion on RAN2 LS about MUSIM and MG collision

Document for:	Discussion

# Discussion

RAN2 sent an LS about MUSIM and MG collision when NW doesn’t configure the priority for MUSIM gap.

R2-2503163 On the collision between the MUSIM Gap and Measurement Gap (for the UE supporting musim-GapPriorityPreference capability) in R18, RAN2 identified that the network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa. In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together. RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases.

In RAN4 #112bis meeting, RAN4 had made the agreement about the collision between MUSIM gap and any measurement gap without priority based on MGRP as follow. From our understanding, the same principle can be applied if NW doesn’t configure the priority for MUSIM gap together with measurement gap.

RAN4 #112bis Issue 2-3-2: Solutions for collision between MUSIM gap and any measurement gap without assigned priority Agreement: - P2: Collision is handled based on the MGRP of the collided gaps - P2-1: In a collision, the gap occasion with longer MGRP will be kept when any measurement gaps in the collision gaps is not assigned a priority; and the gap occasion with shorter MGRP will be dropped. - P2-2: No requirements apply if any of the two gaps in a collision have the same MGRP.

Proposal 1: RAN4 to send LS to reply RAN2 as follow.

- When collisions between MUSIM gaps without priority and measurement gap(s) with/without priority, the UE requirement is defined as follow.

  - In a collision, the gap occasion with longer MGRP will be kept when any measurement gaps in the collision gaps is not assigned a priority; and the gap occasion with shorter MGRP will be dropped.

  - No requirements apply if any of the two gaps in a collision have the same MGRP.

- Summary

In this contribution, we discussed the RAN2 LS about collision between MUSIM gap and MG. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to send LS to reply RAN2 as follow.

- When collisions between MUSIM gaps without priority and measurement gap(s) with/without priority, the UE requirement is defined as follow.

  - In a collision, the gap occasion with longer MGRP will be kept when any measurement gaps in the collision gaps is not assigned a priority; and the gap occasion with shorter MGRP will be dropped.

  - No requirements apply if any of the two gaps in a collision have the same MGRP.