# 3GPP TSG-RAN WG4 Meeting #115	R4-2506904

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.27.3.1

Source: 	Ericsson

Title:	Discussion on OD-SSB in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, the following OD-SSB related issues were agreed.

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW Sub-issue 1: Lower bound for measurement in FMW Agreement Introduce UE capability on whether a lower bound is needed, i.e., - The lower bound is not needed (default value), or, - A lower bound is needed and based on OD-SSB periodicity of [10] ms - Further confirm whether [10] is feasible, and only one value Sub-issue 2: CSSF Agreement - When NW configures single OD-SSB carrier with multiple legacy SSB carriers, CSSF =1 for OD-SSB layer - Extend the FMW window time by one OD-SSB periodicity - Longer measurement delay of other frequency layers with AO-SSBs are expected Sub-issue 5: Searcher assumption Agreement RAN4 shall not change searcher assumption in this work item. Issue 1-2-1: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreement - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow. - max(5*measCycleSCell, 5*DRX cycles) for FR1, or - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2. - Otherwise, the same approach in Case 1 can be reused. - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation. - Note 2: T2 discussion is a separate issue. Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures. Issue 1-4-1: Additional Processing time for OD-SSB reception – scenario Issue 1-4-2: Additional Processing time for OD-SSB reception - value Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: Introduce different values with UE capability: [2 or 3]ms and 5ms. Issue 1-4-4: L1 measurement after SCell activation when DRX is configured Agreement For Scenario 2 and Scenario 2a, when DRX is configured, following the legacy requirements based on DRX with the consideration of OD-SSB is not deactivated. Issue 1-4-7: OD-SSB based Neighbor cell measurement in FR2 Agreement Neighbor cell measurements for FR2 should follow legacy requirements and no change in Rx beam sweeping is required. Issue 1-4-9: Interruption Agreement - New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. - New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. - The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation.

In this contribution, we continue to discuss on the topic on OD-SSB operation of Rel-19 NES WI from RAN4 perspective.

- OD-SSB based deactivated SCell measurement

## 2.1 Case 1: No always-on SSB on the cell

In last meeting, RAN4 made the agreements about OD-SSB based deactivated SCell measurement requirement in Case 1, but some details related to OD-SSB activation procedure are still FFS.

T2 start point and duration

In last meeting, RAN4 spent lots of time to discuss UE’s behaviour when NW deactivated the OD-SSB and activates the OD-SSB again. The wayforward is shown as follow.

Sub-issue 1: The start point of T2 Wayforward The start point of T2 is - Option 1: - The time point of OD-SSB deactivation - Option 2: - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify. - Option 2a: - The time point of OD-SSB activation Sub-issue 2: The minimum interval(T2) between multiple FMWs Wayforward - Option 1: 5* measCycleSCell - Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) - Option 1b: 5*max (measCycleSCell, DRX cycles) - Option 2: Minimum measurement period defined in existing spec. - Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Wayforward - Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: The UE is assumed to perform fast mode measurement without cell identification period. - Option 3: No requirement for UE within T2.

When UE finished the fast deactivated Scell measurement and reporting, UE is allowed to fallback to the legacy deactivated measurement considering the UE power consumption. The intention of T2 is to protect UE to further achieve power saving gain in deactivated SCell mode. In our understanding, both option 1 and option 2a can achieve the same results. Compared to defining the start point of T2 from the OD-SSB deactivation, the T2 duration should be longer if the start point of T2 defined from the OD-SSB activation. However, if the start point of T2 is defined from OD-SSB indication, there will be additional standard work to further discuss how to include the time duration from OD-SSB activation to OD-SSB deactivation shown in the figure below. To facilate the specification work, we still propose to define T2 from OD-SSB deactivation. The T2 duration can be [5]*max(measCycleSCell, DRX cycle).

Observation 1: The intention of defining T2 is to guarantee that UE can achieve power saving when NW enters power saving mode.

Observation 2: There is no technical difference to define the start point of T2 as OD-SSB indication or OD-SSB deactivation.

Proposal 1: The start point of T2 is defined as the point in time when UE receives the OD-SSB deactivation command.

Proposal 2: The duration of T2 equals to [5]*max(measCycleSCell, DRX cycle).

In our understanding, a reasonable NW behaviour is not to activate the OD-SSB again within T2 since it violates the NES princinple to introduce the OD-SSB. When NW re-activates the OD-SSB within T2, UE only needs to follow legacy deactivated SCell measurement requirement without cell identification.

Proposal 3: When NW activates the OD-SSB again within the T2, the UE follows legacy deactivated Scell meas. Requirement.

## 2.2 Case 2: With always-on SSB on the cell

In last meeting, RAN4 agreed to follow the OD-SSB based deactivated SCell measurement procedure in Case 1 with some explicit conditions. The remaining issues are whether to define the requirement for different AO-SSB and OD-SSB scenarios.

Issue 1-2-1: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreement - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow. - max(5*measCycleSCell, 5*DRX cycles) for FR1, or - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2. - Otherwise, the same approach in Case 1 can be reused. - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation. - Note 2: T2 discussion is a separate issue.

Alt Time-C1

In last meeting, RAN4 agreed to apply the Effective Measurement Periodicity(EMP) for deactivated SCell and SCell activation. There is a FFS for other procedure. From our understanding, the EMP can also be  applied to the L1 measurement for the active serving cell after SCell activation.

Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures.

Proposal 4: Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation.

Alt Time-C2

At the same time, RAN1 also introduced a new scenario Alt Time-C2 where the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern. In our understanding, some UEs may have issue to combine both OD-SSB and AO-SSB to perform measurement and SCell activation in this case. However, the UE at least can follow the basic princinple to use OD-SSB to perform measurement in FMW. During the meeting, one concern from UE vendors is about how to handle the scheduling restriction for AO-SSB. From our understanding, no scheduling restriction is expected for the AO-SSB occasions since the requirement is defined based on OD-SSBs.

Figure 1. OD-SSB and AO-SSB transmission in scenario Alt Time-C2

Proposal 5: In Case 2 Alt Time C2, RAN4 to follow the same FMW principle as in case 1 to define the requirement based on the OD-SSB periodicity to perform measurement in FMW if UE supports Case 2 Alt Time C2 scenario.

Scenario 3

In RAN1 #120 meeting, RAN1 also agreed to introduce a new scenario in which the frequency location of OD-SSB is different from the frequency location of AO-SSB. In last RAN2 meeting, a LS about serving cell MO design for this scenario was sent to RAN4. At least one additional servingCellMO is configured to indicate OD-SSB from RAN2’s design.

RAN2 #129b Working assumption - When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Figure 2. OD-SSB and AO-SSB transmission in scenario 3

In our understanding, it looks fine to indicate multiple serving cell MOs for the same serving cell. RAN4 needs to define the UE behaviour based on such RAN2 signalling design.

When NW indicates the OD-SSB transmission, UE shall at least monitor OD-SSB for serving cell measurement and/or SCell activation. In addition, UE needs to perform messurement without gap in both AO-SSB and OD-SSB frequencies since NW has configured both MOs for AO-SSB frequency layer and OD-SSB frequency layer which are in the same BWP. However, a potential open issue is how to define intra-frequency in RAN4 specification. Typically, the neighbour cells will be deployed with the same CD-SSB frequency. Thus, the CD-SSB frequency can be the intra-frequency. However, considering the design in RedCap for NCD-SSB, NW can also indicate the NCD-SSB frequency as the intra-frequency. Considering the tradeoff between UE complexity and NW’s flexibility, we propose to follow the similar mechanism that the intra-frequency is indicated by OD-SSB activation/deactivation.

Proposal 6: RAN4 to send the reply LS to confirm RAN2’s WS to introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Proposal 7: Intra-frequency is indicated by OD-SSB activation/deactivation if UE supports the scenario 3. Otherwise, the intra-frequency shall be always CD-SSB(AO-SSB) frequency.

Proposal 8: UE shall perform measurement on both frequencies for neighbour cells regardless of OD-SSB activation/deactivation.

Proposal 9: UE needs to perform measurement without gap in both AO-SSB and OD-SSB frequencies if UE supports the scenario 3. Otherwise, UE treats OD-SSB frequency measurement as inter-frequency with/without gap measurement.

Proposal 10: If UE supports the scenario 3,

- When OD-SSB is activated, UE shall perform intra-frequency measurement in OD-SSB frequency.

- Before OD-SSB activation or after OD-SSB deactivation, AO-SSB frequency measurement is the intra-freq measurement. UE needs to perform inter-frequency measurement on OD-SSB frequency.

- OD-SSB based SCell activation

In RAN4 #113 meeting, RAN4 agreed to follow the legacy known/unknown Scell activation princinples. Based on current known condition in SCell activation, the UE needs to send a valid L3 measurement report (MR) for the SCell being activated. Thus, during the time period where the Scell is deactivated, UE needs to perform deactivated SCell measurement. After NW receiving the L3 MR, the SCell can be believed as a known SCell. When NW indicates the SCell activation command, UE is assumed to perform SCell activation based on known SCell requirement. RAN4 needs to discuss the impacts due to OD-SSB for these Scell activation requriements.

Known cell condition

In RAN4 #113 meeting, there is an agreement about the known cell condition as follow.

Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

Currently, the known cell condition is as follow. The known cell condition is assumed the same SSB is used to perform measurement and Scell activation.

FR1 SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. FR2 For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

When OD-SSB is introduced, the known cell condition shall be updated based on different OD-SSB and AO-SSB relations.

- Case 1: Deactivated SCell Measurement – AO-SSB; SCell activation-OD-SSB

- Case 2: Deactivated SCell Measurement – AO-SSB+OD-SSB; SCell activation-OD-SSB

- Case 3: Deactivated SCell Measurement – OD-SSB; SCell activation-AO-SSB

Observation 3: The measured SSB and the SSB used for SCell activation can be different.

In RAN1 reply LS, the spatial relation is the same between AO-SSB and OD-SSB. Thus, after UE performs measurement based on any type of SSB, the cell shall be believed as a known cell even if the SSB used in SCell activation is different with the SSB in measurement. At the same time, the measurement period to evaluate the known cell condition shall be the same as legacy.

Proposal 11: RAN4 to reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Proposal 12: The legacy known cell condition shall be updated to clarify the measured SSB and the SSB used in SCell activation can be different.

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation

- The SSB used in SCell activation has the same SSB index as the SSB measured which can be detectable

SSB-less SCell scenario

RAN4 also introduced the SSB-less SCell activation requirement for intra-band and inter-band collocated scenario in release 18. UE follows the SCell activation command to activate the SCell without SSB. In our understanding, UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation phase. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 13: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

- Others

Scenario

In last meetings, RAN4 agreed to define requirements for different SCell activation procedures based on OD-SSB. However, RAN4 hasn’t made any agreement about OD-SSB based multiple SCells activation. From our understanding, OD-SSB based multiple SCells activation shall be the basic scenario when NW supports Rel-19 NES since it’s highly possible to support multiple cells with NES feature other than only single SCell.

Issue 2-1-1: OD-SSB based multiple SCells activation Wayforward - Option 1: - RAN4 to define OD-SSB based multiple SCells activation delay requirements. - The limitation is needed - It doesn’t impact the WI’s completion. - Option 2: - Not define multiple OD-SSB carriers requirement in this release. Issue 2-1-2: Fast SCell Activation Agreement - RAN4 to only update the known cell condition considering OD-SSB for Rel-17 fast SCell activation. - The requirement is based on A-TRS of Rel-17 fast SCell activation requirement. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion. Issue 2-1-3: Deactivated PUCCH SCell activation Agreement - RAN4 to define OD-SSB based deactivated PUCCH SCell activation requirement. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion. Issue 2-1-4: Rel-18 SCell Activation with the L3 reporting during activation Agreement - FFS RAN4 to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion.

At the same time, we understand the complexity to define multiple SCells measurement and activation requirements. Thus, it’s fine to limit the work scope. To simplify the discussion, in deactivated Scell mode, OD-SSBs transmission is non-overlapping among multiple OD-SSBs frequency layers. In other words, UE is always handling single Scell’s OD-SSB once a time. When considering multiple SCells activation, the scope can be limited as all SCells being activated shall be OD-SSBs’ SCells. The same multiple SCells activation requirement can be applied by updating the SMTC to OD-SSB periodicity.

Observation 4:When NW supports Rel-19 NES feature, OD-SSB based multiple SCells is assumed to be a typical scenario.

Proposal 14: RAN4 to define OD-SSB based multiple SCells activation delay requirements. The scope can be limited as follow.

- In deactivated SCell stage, OD-SSBs transmission are non-overlapping among multiple OD-SSBs’ frequency layers.

- For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells.

Additional processing time for OD-SSB deactivation

In last RAN1 meeting, RAN1 made the working assumption about deactivation of OD-SSB transmission and sent the LS to RAN4. We understand that the same deactivation processing time can be confirmed same as OD-SSB activation based on RAN1 LS.

Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Proposal 15: RAN4 to confirm RAN1’s WS about OD-SSB deactivation processing time. T_min=m+3Nslotsubframe,μ+1.

L1 measurement after Scell activation

In last meeting, RAN4 made the agreement on L1 measurement after SCell activation procedure for scenario 2 and 2a.

Issue 1-4-4: L1 measurement after SCell activation when DRX is configured Agreement - For Scenario 2 and Scenario 2a, when DRX is configured, following the legacy requirements based on DRX with the consideration of OD-SSB is not deactivated.

At the same time, RAN1 also made the agreement to support the scenario 3B.

RAN1 #120b Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

It is clearly mentioned in RAN1 agreement that OD-SSB indication is only used for updating the OD-SSB parameter other than deactivating the OD-SSB transmission in scenario 3B. When NW indicates the OD-SSB parameter updating, such as to change the OD-SSB from a dense periodicity for SCell activation to a sparse periodicity to maintain the timing. UE shall follow the updated OD-SSB periodicity in non-DRX mode. When DRX is configured, the similar agreement made in scenario 2 and 2a can be reused. UE just performs measurement following the DRX cycle.

Observation 5: In scenario 3B, OD-SSB indication is only used for updating the OD-SSB parameter other than deactivating the OD-SSB transmission.

Proposal 16: When MAC-CE indicates OD-SSB updating in scenario 3B,

- UE performs the L1-RSRP measurement based on updated OD-SSB periodicity if DRX is not configured,

- Otherwise, UE still follow legacy DRX based requirement.

Scheduling restriction for L1 measurement after SCell activation

As mentioned in section 2.2 case 2, UE may only perform measurement on OD-SSB in some scenarios, such as Alt Time C-2. In such scenario, no scheduling restriction and measurement restriction is expected for L1 measurement by AO-SSB. On the other hand, the scheduling restriction shall be applied to both AO-SSB and OD-SSB if the requirement is defined based on both of them.

Proposal 17: In Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement. The scheduling restriction shall be applied to both AO-SSB and OD-SSB if the requirement is defined based on both of them.

Neighbour cell measurement

In last meetings, RAN4 discussed the neighbour cell measurement requirement. The agreements are as follow.

RAN4 #114 Issue 7-2: OD-SSB based Neighbour cell measurement Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell Agreement: - For Scenario 1a: No RAN4 spec. change. Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. RAN4 #114b Issue 1-4-7: OD-SSB based Neighbor cell measurement in FR2 Agreement - Neighbor cell measurements for FR2 should follow legacy requirements and no change in Rx beam sweeping is required. Issue 1-4-8: OD-SSB based Neighbor cell measurement before OD-SSB activation Wayforward - RAN4 to discuss whether to perform measurement on neighbour cells’ in the same frequency of deactivated SCell before OD-SSB activation for Case#1.

The remaining part is UE’s behaviour for neighbour cell measurement before OD-SSB activation. From our understanding, once NW configures the measurement objects for the frequency, UE shall perform measurement on this frequency. Thus, before OD-SSB activation in Case #1, UE shall also perform the measurement in this frequency.

Proposal 18: In Case #1, UE to perform neighbour cell measurement based on legacy requirement with the configured measurement object.

Cell identification component

In last meetings, there is an open issue about whether UE needs to always perform cell identification to obtain coarse timing before deactivated Scell activation. The motivation to always require additional cell indentification is due to OD-SSB is not always transmitted.

Issue 2-1-6: OD-SSB based deactivated SCell measurement components - Proposals - Option 1: vivo - RAN4 to clarify the deactivated SCell measurement components includes both cell identification and measurement.

However, we think at least for the following cases UE doesn’t need to always perform cell identification. Firstly, for always-on SSB transmitted in case 2 means UE has performed the cell identification based on always-on SSB and doesn’t need to perform cell identification when OD-SSB is indicated. Secondly, in Case 1, if the OD-SSB SCell being measured is contiguous to an active serving cell in the same band, UE can refer on the active serving cell’s timing as legacy. No cell identification is needed. Thirdly, if UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is less than Xs and then the OD-SSB is transmitted again, cell identification is also unnecessary.

Proposal 19: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [X]s and then the OD-SSB is activated again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

Interruption

In last meeting, RAN4 agreed to define the OD-SSB based interruption requirements as follow. RAN4 needs to further discuss the interruption duration and position. In our understanding, the interruption duration shall be 0.5ms in FR1 and 0.25ms in FR2. The interruption window shall start from OD-SSB indication with HARQ processing to the end point of Tx. The end point Tx is Time instance A + T_processing when OD-SSB activation and Time instance B + T_processing for OD-SSB deactivation. The Tx for interruption due to FMW shall be the end point of FMW plus _processing.

Issue 1-4-9: Interruption Agreement - New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. - New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. - The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation.

Proposal 20: The interruption duration of OD-SSB activation/deactivation shall be 0.5ms.

Proposal 21: The interruption window end point Tx shall be as follow.

- OD-SSB activation end point: Time instance A + T_processing

- OD-SSB deactivation end point: Time instance B + T_processing

- FMW end point: The end point of FMW + T_processing

- Summary

In this contribution, we have discussed the OD-SSB requirements. Based on the discussions, we have made following proposals and observations:

Observation 1: The intention of defining T2 is to guarantee that UE can achieve power saving when NW enters power saving mode.

Observation 2: There is no technical difference to define the start point of T2 as OD-SSB indication or OD-SSB deactivation.

Observation 3: The measured SSB and the SSB used for SCell activation can be different.

Observation 4:When NW supports Rel-19 NES feature, OD-SSB based multiple SCells is assumed to be a typical scenario.

Observation 5: In scenario 3B, OD-SSB indication is only used for updating the OD-SSB parameter other than deactivating the OD-SSB transmission.

Proposal 1: The start point of T2 is defined as the point in time when UE receives the OD-SSB deactivation command.

Proposal 2: The duration of T2 equals to [5]*max(measCycleSCell, DRX cycle).

Proposal 3: When NW activates the OD-SSB again within the T2, the UE follows legacy deactivated Scell meas. Requirement.

Proposal 4: Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation.

Proposal 5: In Case 2 Alt Time C2, RAN4 to follow the same FMW principle as in case 1 to define the requirement based on the OD-SSB periodicity to perform measurement in FMW if UE supports Case 2 Alt Time C2 scenario.

Proposal 6: RAN4 to send the reply LS to confirm RAN2’s WS to introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Proposal 7: Intra-frequency is indicated by OD-SSB activation/deactivation if UE supports the scenario 3. Otherwise, the intra-frequency shall be always CD-SSB(AO-SSB) frequency.

Proposal 8: UE shall perform measurement on both frequencies for neighbour cells regardless of OD-SSB activation/deactivation.

Proposal 9: UE needs to perform measurement without gap in both AO-SSB and OD-SSB frequencies if UE supports the scenario 3. Otherwise, UE treats OD-SSB frequency measurement as inter-frequency with/without gap measurement.

Proposal 10: If UE supports the scenario 3,

- When OD-SSB is activated, UE shall perform intra-frequency measurement in OD-SSB frequency.

- Before OD-SSB activation or after OD-SSB deactivation, AO-SSB frequency measurement is the intra-freq measurement. UE needs to perform inter-frequency measurement on OD-SSB frequency.

Proposal 11: RAN4 to reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

Proposal 12: The legacy known cell condition shall be updated to clarify the measured SSB and the SSB used in SCell activation can be different.

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation

- The SSB used in SCell activation have the same SSB index as the SSB measured which can be detectable

Proposal 13: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 14: RAN4 to define OD-SSB based multiple SCells activation delay requirements. The scope can be limited as follow.

- In deactivated SCell, OD-SSBs transmission is none overlapping among multiple OD-SSBs frequency layers.

- For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells.

Proposal 15: RAN4 to confirm RAN1’s WS about OD-SSB deactivation processing time. T_min=m+3Nslotsubframe,μ+1.

Proposal 16: When MAC-CE indicates OD-SSB updating in scenario 3B,

- UE performs the L1-RSRP measurement based on updated OD-SSB periodicity if DRX is not configured,

- Otherwise, UE still follow legacy DRX based requirement.

Proposal 17: In Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement. The scheduling restriction shall be applied to both AO-SSB and OD-SSB if the requirement is defined based on both of them.

Proposal 18: In Case #1, UE to perform neighbour cell measurement based on legacy requirement with the configured measurement object.

Proposal 19: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [X]s and then the OD-SSB is activated again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

Proposal 20: The interruption duration of OD-SSB activation/deactivation shall be 0.5ms.

Proposal 21: The interruption window end point Tx shall be as follow.

- OD-SSB activation end point: Time instance A + T_processing

- OD-SSB deactivation end point: Time instance B + T_processing

- FMW end point: The end point of FMW + T_processing

- References

- R4-2504906, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

- R1-2503108, “LS to RAN4 on time instance B for on-demand SSB SCell operation”, LG

- R2-2503019, “LS on on-demand SSB”, Apple