# 3GPP TSG-RAN WG4 Meeting #115	R4-2506905

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.27.3.2

Source: 	Ericsson

Title:	Discussion on SSB adaptation in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, we achieved some agreements as follow[1].

### Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - SSB adaptation in SCell for NCD-SSB. - Time domain positions of SSBs within a burst is same before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. ### Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline. - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs. ### Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements Agreement: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - FFS Rel-18 SCell activation with L3 reporting - FFS Multiple SCells activation

In this contribution, we continue to discuss on the Rel-19 NES operation in Obj. 3 from RAN4 perspective.

# Adaptation of common signal/channel transmissions (CS/CC adaptation)

Tfirst_SSB

One of the remaining issues for SSB adaptation is to define the SSB adaptation processing time after SSB adaptation DCI indication. Currently, RAN1 is also discussing the timeline of DCI processing time for SSB adaptation with the following working assumption. Thus, we expect RAN1 will solve this SSB adaptation processing issue.

RAN1 #120bis Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst

Proposal 1: RAN4 to wait for RAN1’s progress on the DCI based processing time for SSB adaption.

L3 Requirement

In last RAN2 meeting, RAN2 made the following agreement about additional SMTC for SSB adaptation. Based on latest RAN2 agreement, multiple additional SMTC configuration are introduced. Upon reception of DCI indication for SSB periodicity switching, UE shall follow the updated SMTC based on the mapping between SSB periodicity and SMTC configuration. Therefore, the new SMTC is applied for L3 measurement.

RAN2 #128b agreement - For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. - Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Proposal 2: After SSB adaptation, UE follows the updated SMTC to perform L3 measurement based on the mapping between SSB periodicity and SMTC configuration.

Another important issue for SSB adaptation is how to define the requirement for transition period. Currently, in L1-RSRP measurement, UE follows the updated SSB periodicity after Tfirst_SSB which means UE drops the legacy measurement results. In L1-RSRP measurement, it looks fine since the measurement sample is less and no lower bound is defined. However, in L3-RSRP measurement, RAN4 already introduced a lower bound as 200ms, or 600ms which is too long. Therefore, it’s important to require UE to continue the measurement after SSB adaptation. The transition requirement can be defined based on the maximum period corresponding to the SMTC before and after adaptation.

Proposal 3: The transition requirement can be defined based on the maximum period corresponding the SMTC before and after SSB adaptation.

SCell activation

In last RAN2 meeting, RAN2 agreed no MAC CE signalling for SSB adaptation which means NW cannot trigger SCell activation and SSB adaptation based on the same MAC CE.

RAN2 #129 agreement - Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

In RAN4 #114 meeting, RAN4 made the agreement for SCell activation with SSB adaptation as follow which needs to be updated. From our understanding, if NW indicates SSB adaptation based on DCI before SCell activation command, UE shall activate the SCell based on the updated SSB if UE supports SSB adaptation capability. Therefore, we propose to update the agreement ‘together with SCell activation command’ to ‘no later than SCell activation command’.

Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG.

Proposal 4: RAN4 to update the agreement about SCell activation with SSB adaptation from ‘SSB adaptation command comes together with SCell activation command’ to ‘SSB adaptation command comes no later than SCell activation command’.

- Summary

In this contribution, we have discussed the RS adaptation requirements. Based on the discussions, we made the following proposals and observations:

Proposal 1: RAN4 to wait for RAN1’s progress on the DCI based processing time for SSB adaption.

Proposal 2: After SSB adaptation, UE follows the updated SMTC to perform L3 measurement based on the mapping between SSB periodicity and SMTC configuration.

Proposal 3: The transition requirement can be defined based on the maximum period corresponding the SMTC before and after SSB adaptation.

Proposal 4: RAN4 to update the agreement about SCell activation with SSB adaptation from ‘SSB adaptation command comes together with SCell activation command’ to ‘SSB adaptation command comes no later than SCell activation command’.

- References

- R4-2504907, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei