3GPP TSG-RAN4 Meeting #115	R4-2506931

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title: 	Discussion on Ku-band VSAT Variable Tx-Rx frequency

Agenda Item:	x.x.x.x.x

Document for:	Discussion and Approval

# Introduction

RAN4 has been discussing requirements for NTN Ku-band VSAT RE-requirements. Way forward [1] and [2] were agreed in RAN WG4 Meeting #114 and RAN WG4 Meeting #114bis and agreements in this documents are used as inputs in this analysis.

RAN WG4 Meeting #114

Sub-topic 3-10: Default Tx Rx frequency separation

Agreement: Variable TX RX spacing is assumed, how it is specified is FFS.

Ka band FR2 did not specify Tx Rx separation due to the large separation but may need to specify for Ku with close separation.

RAN WG4 Meeting #114 bis

Agreement:

Sub-topic 2-3: UE channel bandwidth per operating band

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n248 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

Sub-topic 2-6: FR1-NTN ARFCN definitions

Adopt the following for bands 248 and 247

| Band | SCS | Uplink |  |  | Downlink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 248 | 15 | 1333334 | <1> | 1366666 | 1113334 | <1> | 1250000 |
| 247 | 15 | 1316667 | <1> | 1333333 | 1113334 | <1> | 1250000 |

This document proposes the definition of the Variable Tx Rx frequency separation based on the agreements on CBW and ARFCN as listed above.

# Discussion

## Variable Tx - Rx frequency separation

Figure 1 shows the principle for defining the minimum and maximum Tx - Rx frequency separation. As the UL frequencies are higher than the DL frequencies the Tx – Rx separations will be negative values.

Figure 1. Principle for defining the minimum and maximum Tx - Rx frequency separation

Observation 1: RAN WG4 Meeting #114 and #114bis meeting agreed the ARFCN ranges for bands n248 and n247 and 50MHz for bands n509 and n508.

Observation 2: RAN WG4 Meeting #114bis meeting agreed minimum channel bandwidth of 5MHz for bands n248 and n247 and 50MHz for bands n509 and n508.

Observation 3: These minimum channel bandwidths can be used to define the Variable Tx-RX frequency separation in a way that it leaves full flexibility to support different Tx-Rx frequency also combined with asymmetric UL and DL channel bandwidth combinations.

For band n248 and n247 the frequencies corresponding the lowest and highest ARFCN are shown in Table 1.

Table 1. Frequencies corresponding the lowest and highest ARFCN for bands n248 and n247

| Band | SCS | Uplink |  |  | Downlink |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 248 | 15 | 1 333 334 | <1> | 1 366 666 | 1 113 334 | <1> | 1 250 000 |  |
| 247 | 15 | 1 316 667 | <1> | 1 333 333 | 1 113 334 | <1> | 1 250 000 |  |
| Freqs |  | Uplink |  |  | Downlink |  |  | Unit |
| 248 | Freqs | 14 000.010 | - | 14 499.990 | 10 700.010 | - | 12 750.000 | MHz |
| 247 | Freqs | 13 750.005 | - | 13 999.995 | 10 700.010 | - | 12 750.000 | MHz |

For channel bandwidth of 5MHz the ARFCN for the lowest and highest channel that still fits inside the band needs to be 167 steps above or below the minimum or maximum ARFCN of the corresponding band. 167 steps equals to 2.505MHz, which is slightly higher than half of the 5MHz minimum channel bandwidth and also ensures that minimum guard band requirements are met.

The corresponding ARFCN and channel center frequencies for such channels and the resulting minimum and maximum variable Tx-Rx frequency separation is shown in Table 2.

Table 2. ARFCN and channel center frequencies for such channels and the resulting minimum and maximum variable Tx-Rx frequency separation for bands n248 and n247

| Band | SCS | Uplink |  |  | Downlink |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 248 | 15 | 1 333 501 | <1> | 1 366 499 | 1 113 501 | <1> | 1 249 833 |  |
| 247 | 15 | 1 316 834 | <1> | 1 333 166 | 1 113 501 | <1> | 1 249 833 |  |
| Band |  | Uplink |  |  | Downlink |  |  | Unit |
| 248 | Freqs | 14 002.515 | - | 14 497.485 | 10 702.515 | - | 12 747.495 | MHz |
| 247 | Freqs | 13 752.510 | - | 13 997.490 | 10 702.515 | - | 12 747.495 | MHz |
| Band |  | Minimum Tx - Rx gap |  |  | Maximum Tx - Rx gap |  |  |  |
| 248 |  | 1 255.020 |  |  | 3 794.970 |  |  |  |
| 247 |  | 1 005.015 |  |  | 3 294.975 |  |  |  |
| Band | TX – RX carrier centre frequency separation |  |  | Unit |  |  |  |  |
| 248 | -3 794.970 to -1 255.020 |  |  | MHz |  |  |  |  |
| 247 | -3 294.975 to -1 005.015 |  |  | MHz |  |  |  |  |

For band n509 and n508 the frequencies corresponding the lowest and highest ARFCN are shown in Table 3.

Table 3. Frequencies corresponding the lowest and highest ARFCN for bands n509 and n508

| Band | SCS | Uplink |  |  | Downlink |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 509 | 120 | 1 333 336 | <8> | 1 366 664 | 1 113 334 | <8> | 1 250 000 |  |
| 508 | 120 | 1 316 672 | <8> | 1 333 328 | 1 113 334 | <8> | 1 250 000 |  |
| Freqs |  | Uplink |  |  | Downlink |  |  | Unit |
| 509 | Freqs | 14 000.040 | - | 14 499.960 | 10 700.010 | - | 12 750.000 | MHz |
| 508 | Freqs | 13 750.080 | - | 13 999.920 | 10 700.010 | - | 12 750.000 | MHz |

For channel bandwidth of 5MHz the ARFCN for the lowest and highest channel that still fits inside the band needs to be 167 steps above or below the minimum or maximum ARFCN of the corresponding band. 167 steps equals to 2.505MHz, which is slightly higher than half of the 5MHz minimum channel bandwidth and also ensures that minimum guard band requirements are met.

The corresponding ARFCN and channel center frequencies for such channels and the resulting minimum and maximum variable Tx-Rx frequency separation is shown in Table 4.

Table 4. ARFCN and channel center frequencies for such channels and the resulting minimum and maximum variable Tx-Rx frequency separation for bands n509 and n508

| Band | SCS | Uplink |  |  | Downlink |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 509 | 120 | 1 335 003 | <8> | 1 364 997 | 1 115 001 | <8> | 1 248 333 |  |
| 508 | 120 | 1 318 339 | <8> | 1 331 661 | 1 115 001 | <8> | 1 248 333 |  |
| Band |  | Uplink |  |  | Downlink |  |  | Unit |
| 509 | Freqs | 14 025.045 | - | 14 474.955 | 10 725.015 | - | 12 724.995 | MHz |
| 508 | Freqs | 13 775.085 | - | 13 974.915 | 10 725.015 | - | 12 724.995 | MHz |
| Band |  | Minimum Tx - Rx gap |  |  | Maximum Tx - Rx gap |  |  |  |
| 509 |  | 1 300.050 |  |  | 3 749.940 |  |  |  |
| 508 |  | 1 050.090 |  |  | 3 249.900 |  |  |  |
| Band | TX – RX carrier centre frequency separation |  |  | Unit |  |  |  |  |
| 509 | -3 749.940 to -1 300.050 |  |  | MHz |  |  |  |  |
| 508 | -3 249.900 to -1 050.090 |  |  | MHz |  |  |  |  |

Proposal 1: Define the UE (VSAT) UE TX-RX frequency separation for FR1-NTN bands n248 and n247 and FR2-NTN bands n509 and n508 as follows and draft CR has also been captured into Annex A: Draft CR:

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| n248 | -3 794.970 to -1 255.020 MHz |
| n247 | -3 294.975 to -1 005.015 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

Table 5.4.4-2: UE TX-RX frequency separation (FR2-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n509 | -3 749.940 to -1 300.050 MHz |
| n508 | -3 249.900 to -1 050.090 MHz |

# Conclusions

Observation 1: RAN WG4 Meeting #114 and #114bis meeting agreed the ARFCN ranges for bands n248 and n247 and 50MHz for bands n509 and n508.

Observation 2: RAN WG4 Meeting #114bis meeting agreed minimum channel bandwidth of 5MHz for bands n248 and n247 and 50MHz for bands n509 and n508.

Observation 3: These minimum channel bandwidths can be used to define the Variable Tx-RX frequency separation in a way that it leaves full flexibility to support different Tx-Rx frequency also combined with asymmetric UL and DL channel bandwidth combinations.

Proposal 1: Define the UE (VSAT) UE TX-RX frequency separation for FR1-NTN bands n248 and n247 and FR2-NTN bands n509 and n508 as follows and draft CR has also been captured into Annex A: Draft CR:

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| n248 | -3 794.970 to -1 255.020 MHz |
| n247 | -3 294.975 to -1 005.015 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

Table 5.4.4-2: UE TX-RX frequency separation (FR2-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n509 | -3 749.940 to -1 300.050 MHz |
| n508 | -3 249.900 to -1 050.090 MHz |

# References

[1] R4-2502290	Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat), TSG-RAN WG4 Meeting #114, Feb 2025

[2] R4-2504761	Way Forward for [114bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat), TSG-RAN WG4 Meeting #114bis, Apr 2025

[2] 3GPP TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 18)

# Annex A: Draft CR

< START OF CHANGE #1 >

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1 for FR1-NTN.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| n248 | -3 794.970 to -1 255.020 MHz |
| n247 | -3 294.975 to -1 005.015 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

Table 5.4.4-2: UE TX-RX frequency separation (FR2-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n509 | -3 749.940 to -1 300.050 MHz |
| n508 | -3 249.900 to -1 050.090 MHz |

< END OF CHANGE #1 >