# 3GPP TSG-RAN WG4 Meeting #115	R4-2506941

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.12.2.1

Source: 		Qualcomm Incorporated

Title: 	Views on Rel-19 ATG UE RF requirements

Document for:	Approval

# Introduction

In RAN4#114bis, UE RF requirements for Rel-19 ATG enhancement were discussed, and the agreements are captured in WF [1]. In this paper, we provide our further views on remaining issues.

# 2. Discussion

A good progress was made for the MSD for CA_n3A-n39A with n39 UL in [1]. The remaining issue for CA_n3A-n39A is the MSD for the case n3 is UL.

Issue 1-3: MSD for CA_n3A-n39A with n3 UL Agreement: - MSD requirements will be decided based on the average values among companies’ simulation results. - Encourage companies to provide results next meeting.

In RAN4#114, the following configurations were agreed for MSD for CA_n3A-n39A with n3 UL in [2].

Issue 2-4: MSD for CA_n3A-n39A with n3 UL WF: - Companies are encouraged to provide MSD values based on the following Tables. - Other MSD test configurations are not precluded. - All of these MSD values are only applicable to ATG UE. - Further study on how to capture these MSD values to in the specification. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n3; n39; [1722.5]; [25]; [15]; [160 (RBstart=0)]; [1900]; [30]; MSD Table Y; [>ACLR2].

Since UL BW of n3 is 25MHz, the maximum value of LCRB for UL RB allocation should be 133. In addition, usually, for the MSD analysis due to cross-band interference interference from a FDD band UL, FDD UL allocation is the same as UL RB allocation for the single carrier REFSENS definition.

For band n3, with 25MHz UL BW, the UL LCRB configuration defined in Table 7.3.2-3 of TS 38.101-1[3], i.e., 50, should be used here. And UL resource blocks shall be located as close as possible to the downlink operating band. So, we have the following proposal for the UL configuration for MSD analysis for CA_n3A-n39A with n3 UL.

Proposal 1: RAN4 to adopt the following UL configuration for MSD analysis for CA_n3A-n39A with n3 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1722.5 | 25 | 15 | 50 (RBstart=83) | 1900 | 30 | MSD Table Y | >ACLR2 |

With the above UL configuration, the following MSD values can be calculated assuming that 20dB antenna isolation considering a different form factor for CPE compared to handheld UE.

Proposal 2: RAN4 to take the following MSD values into account when defining MSD requirements for CA_n3A-n39A with n3 UL

Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers

| Output power | MSD (dB) |
| --- | --- |
| 23dBm | 1.0 |
| 24dBm | 1.3 |
| 25dBm | 1.6 |
| 26dBm | 1.9 |
| 27dBm | 2.3 |
| 28dBm | 2.7 |
| 29dBm | 3.2 |
| 30dBm | 3.6 |
| 31dBm | 4.1 |
| 32dBm | 4.5 |
| 33dBm | 5.0 |
| 34dBm | 5.4 |
| 35dBm | 6.0 |
| 36dBm | 6.6 |
| 37dBm | 7.3 |
| 38dBm | 8.1 |
| 39dBm | 9.0 |
| 40dBm | 10.2 |
| Note: MSD only applies for ATG UE targeted for CPE form factor. |  |

# 3.	Conclusion

In this paper, we provide our views on UE RF requirements in ATG enhancements. The following observations and proposals are made:

Proposal 1: RAN4 to adopt the following UL configuration for MSD analysis for CA_n3A-n39A with n3 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1722.5 | 25 | 15 | 50 (RBstart=83) | 1900 | 30 | MSD Table Y | >ACLR2 |

Proposal 2: RAN4 to take the following MSD values into account when defining MSD requirements for CA_n3A-n39A with n3 UL

Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers

| Output power | MSD (dB) |
| --- | --- |
| 23dBm | 1.0 |
| 24dBm | 1.3 |
| 25dBm | 1.6 |
| 26dBm | 1.9 |
| 27dBm | 2.3 |
| 28dBm | 2.7 |
| 29dBm | 3.2 |
| 30dBm | 3.6 |
| 31dBm | 4.1 |
| 32dBm | 4.5 |
| 33dBm | 5.0 |
| 34dBm | 5.4 |
| 35dBm | 6.0 |
| 36dBm | 6.6 |
| 37dBm | 7.3 |
| 38dBm | 8.1 |
| 39dBm | 9.0 |
| 40dBm | 10.2 |
| Note: MSD only applies for ATG UE targeted for CPE form factor. |  |

# 4.	References

- R4-2505112, WF on UE RF requirements for NR ATG, CMCC

- R4-2502952, WF on UE RF requirements for ATG, CMCC

- TS 38.101-1