# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2506947

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.17.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices, which was approved in [1], wherein the objectives related to 8Rx for CPE/FWA /vehicles are listed as below:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we give some initial discussions on the UE performance requirements for 8Rx CPE/FWA/vehicle/industrial device, which include inter-cell interference cancellation and intra-cell inter-user interference cancellation based on the lasting meeting conclusion [2].

# 2 Discussion

As we all know, co-channel inter-cell and intra-cell inter-user interference cancellation performance requirements were introduced in Rel-17. And in Rel-18, RAN4 introduced 8Rx demodulation requirements for PDSCH and CSI performance. With 8Rx, larger performance gain can be verified, due to larger spatial domain freedom compared to 2/4Rx. As a consequence, from demodulation perspective, there are more benefits to define UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices under interference scenarios.

  - Sub-topic 1-1 General

8Rx UE MMSE-IRC receiver coverage

Agreement: - Down select the [limit] to one of 0.5dB or 1.0dB. Remaining proposals: - Option 1: Introduce one more rule on the top of the agreed criteria: After reviewing the simulation results for all the test cases by the agreed criteria, if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases, which are applicable for both baseline and simplified receivers.

The controversial issue is that how to define a criteria to cover two sets of requirements for specific configuration in last meeting. Regarding baseline and simplified receivers, which is different implementations for baseband. From our point of view, we prefer to define separate requirements for both of them. Therefore, RAN4 needs to discuss the actual value of difference for baseline and simplified receivers, also needs to specific the value based on which calculate methods are used. In legacy requirements, RAN4 always used average SNR or SINR value among different companies. The average is one of the most commonly used indicators to describe the central tendency of a dataset, providing information on the center point or typical value of a set of data. Specifically, the average reveals the balancing point of all observation within the dataset. Generally, we also prefer to use average SNR as a threshold to measure the results of different processing. From the last meeting simulation results[3], we found that the two sets results for all baseline and simplified receivers are more than 1 dB. On the other side, since RAN4 did not reach a consensus on the Rank 4+4 scenario for MU-MIMO, there is no need to consider the case where the simplified receiver cannot work in the Rank 4+4 scenario. In other words, the test cases currently covered can both support baseline and simplified receivers.

Proposal 1. Propose to prefer to consider 1dB difference for defining two sets of requirements.

    - Sub-topic 1-5 Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

Proposals - Option 1: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 4: Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test: - UE support both 8Rx and 2/4Rx - The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement. - Option 5: RAN4 shall have the following applicability rules: - UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx. - UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

The motivation behind RAN4’s discussion of the test applicability rule that is to reduce the burden of UE if UE supports a high capability in testing. In Rel-17, RAN4 introduced inter interference scenario and intra-cell inter-iser interference scenario as mandatory feature for 2/4Rx UE. As illustrated in WID, 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario are also mandatory feature for UE. Hence, if UEs supporting both 8Rx and 2/4Rx, 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped. Otherwise, 2/4Rx tests could not be skipped.

Proposal 2. To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Release independent

Agreement: - The new Rel-19 requirements shall at least be release independent from Rel-18. Remaining proposals: - Option 1: The new Rel-19 requirements shall also be release independent from Rel-17. - Option 2: The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

Considering 8Rx requirements was introduced in Rel-18 and independent from Rel-17. Considering the inter interference cancellation is a mandatory feature. We prefer to consider the new requirements shall be release independent from Rel-17.

Proposal 3. Propose to consider the new Rel-19 shall also be release independent from Rel-17.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for UE demodulation performance requirements for 8Rx with MMSE-IRC , The conclusions are:

Proposal 1. Propose to prefer to consider 1dB difference for defining two sets of requirements.

Proposal 2. To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 3. Propose to consider the new Rel-19 shall also be release independent from Rel-17.

# 4 References

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements.

- R4-2504694, Way forward for [114bis][322] NR_demod_Ph5_Part2_UE. Nokia.

- R4-2503545, Summary of simulation results for ICI and MU-MIMO with 8RX.