# 3GPP TSG-RAN WG4 Meeting # 115                                 R4-2506952

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on UE and BS demodulation and CSI performance requirements for 7MHz channel bandwidth

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.8.6

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, a new WID on 7 MHz Channel Bandwidth for n26 and n5 was approved in [1], wherein the objectives related to 7MHz are listed as below:

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR)

In the following section, we will provide the detailed discussions for 7MHz UE and BS demodulation requirements.

# 2 Discussion

With the introduction of 5GNR, the channel bandwidths were limited to increments of 5 MHz. Now that the 3 MHz channel bandwidth has been added, there is the possibility of coverage of additional operator’s spectrum including 8 and 13 MHz via CA, but there is still not a good way to cover other channel bandwidths that have been requested by operators including 6, 7, 11, 12 and 13 MHz. Given the large investment in spectrum, especially low band spectrum, it is critical that operators have a way to efficiently utilize all their spectrum. While several approaches were initially proposed for this WID, the agreement was to include only native 7 MHz channel bandwidths for n5 and n26 in Rel-19.

UE demodulation requirements

The motivation for this work item is to introduce a new channel bandwidth to the existing n5 and n26 bands. In Rel-18, RAN4 introduced less than 5MHz channel bandwidth for PBCH, PDCCH and PUCCH requirements, as this requires puncturing to support transmission in small bandwidth, which is different from the legacy requirements. And now, the number of RBs for the 7MHz bandwidth is more than the number of RBs defined by PBCH and PDCCH. We do not see sufficient reason to define new requirements for PBCH and PDCCH.

On the other side, RAN4 achieved a common understanding that no UE only supports less than 5MHz channel bandwidth in Rel-18. Considering that we also believe that the same reason could also reused for 7MHz. Hence, we prefer not to define PDSCH requirements for 7MHz.

Proposal 1. Propose not to define PBCH requirements for 7MHz channel bandwidth.

Proposal 2. Propose not to define PDCCH requirements for 7MHz channel bandwidth.

Proposal 3. Propose not to define PDSCH requirements for 7MHz channel bandwidth.

Proposal 4. Propose not to define CSI requirements for 7MHz channel bandwidth.

BS demodulation requirements

Regarding BS side requirements, considering that the existing BS can support many bandwidth combinations, and there is no difference in baseband processing between 7M and 10M bandwidths. Therefore, we propose not to define PUSCH demodulation requirements for 7MHz.

Proposal 5. Propose not to define PUSCH requirements for 7MHz channel bandwidth.

5G NR defines multiple PUCCH formats, and each format supports a different number of RBs. In Rel-18, RAN4 only introduced demodulation requirement for PUCCH format 2 with 3MHz CBW. Considering that 7M bandwidth can support the number of RBs corresponding to all PUCCH formats. We propose not to define PUCCH requirements for 7MHz channel bandwidth.

Proposal 6. Propose not to define PUCCH requirements for 7MHz channel bandwidth.

For PRACH, there is no limit on the required bandwidth, it only needs to meet different configurations. In Rel-18, RAN4 agreed that no new PRACH requirements for less than 5MHz and added an applicable rule for BS to supports less than 5MHz, only test requirements relating to RACH preamble formats with 15kHz SCS with sequence length LRA=139, and PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply. Therefore, we propose not to define new PRACH demodulation requirements for 7MHz, and whether to define applicability rules need to be further discussed.

Proposal 7. Propose not to define new PRACH requirements for 7MHz channel bandwidth, and whether to define applicability rules need to be further discussed if necessary.

# 3 Conclusion

In this contribution, we give some discussions on the UE and BS demodulation requirements for 7MHz channel bandwidth, The conclusions are:

Proposal 1. Propose not to define PBCH requirements for 7MHz channel bandwidth.

Proposal 2. Propose not to define PDCCH requirements for 7MHz channel bandwidth.

Proposal 3. Propose not to define PDSCH requirements for 7MHz channel bandwidth.

Proposal 4. Propose not to define CSI requirements for 7MHz channel bandwidth.

Proposal 5. Propose not to define PUSCH requirements for 7MHz channel bandwidth.

Proposal 6. Propose not to define PUCCH requirements for 7MHz channel bandwidth.

Proposal 7. Propose not to define new PRACH requirements for 7MHz channel bandwidth, and whether to define applicability rules need to be further discussed id necessary.

# 4 References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA.