# 3GPP TSG-RAN WG4 Meeting # 115                                 R4-2506955

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on UE demodulation performance requirements for 6Rx

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.4

# Document for:	Approval

# 1 Introduction

In RAN #105 meeting, the revised WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx

In the following section, we will provide the detailed discussions for 6Rx UE demodulation requirements.

# 2 Discussion

In the last meeting, RAN4 discussed the technical issues in detail, but still have some open issues[2]. Next, we would like to address in the following for 6Rx UE demodulation requirements.

General

Test applicability rule for 6Rx test cases

- Way forward - Option 1: RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification. - Option 2: - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Regarding test applicability rule, this will affect further testing options for 6Rx UE, which has different devices to support different layers. According to previous meeting conclusion, only FWA devices can support 6 layers, and 4 layers can apply to FWA and handled UE devices. Starting from this conclusion, RAN4 may need to define different demodulation requirements for different devices. However, from the perspective of testing, if only FWA devices can support MIMO 6 layers, RAN4 shall define requirements for 6 MIMO layers FWA. And for 4 MIMO layers FWA and handled UE devices, if RAN4 would like to define one requirement for 4 MIMO layers, maybe RAN4 needs to consider handled UE’s scenario at first.

Proposal 1. Propose to postpone discussing test applicable rule until RAN4 decides on the 6Rx test cases.

PDSCH requirements for 6Rx

Test scenarios

- Way forward - Option 1: Test scenarios without interference: PDSCH Mapping Type A - Option 2: Test scenarios with interference - PDSCH demodulation requirements with inter-cell interference - PDSCH demodulation requirements with intra-cell inter-user interference

In the last meeting, some companies mentioned that interference scenario also shall be considered in this work item, since the interference cancellation scenarios are mandatory features for UE side. In legacy requirements, RAN4 introduced the corresponding features for 2Rx, 4Rx and 8Rx UEs. However, the relevant inference cancellation scenarios is not reflected in WID[1]. If RAN4 would like to introduce the corresponding requirements, RAN4 needs to update the WID in the RAN plenary at first.

Proposal 2. Propose to not consider test scenarios with interference in Rel-19.

Test cases for 4 MIMO layers for FWA and Handheld UE

- Way forward - Option 1: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE - Option 2: Two separate test cases for 4 MIMO layers for FWA and Handheld UE - Option 2a: One test case with ULA Low for FWA; Another test case with ULA Medium with β = 0.00718 for Handheld UE - Other options

In the last meeting, RAN4 agreed to introduce a single test case for 6 MIMO layers for FWA device. Regarding 4 MIMO layers, to reduce the testing burden, we also prefer to define a single test case, which means all potential assumptions shall be aligned with handled UE scenario.

Proposal 3. Propose to define a single test case for 4MIMO layers that is applicable for both FWA and handled UE.

Propagation channel and MIMO correlation for 6 MIMO layers

- Way forward - Option 1: TDLA 30-10 ULA Low - Option 2: TDLD 30-5 ULA Low

As we discussed in the meeting, low correlation must be considered for FWA devices. And for the propagation channel, some companies mentioned that 256 QAM may could be supported for 6MIMO layers. After we checked the current specification, the legacy 256QAM requirements also based on TDLA30-10 propagation condition. From the evaluation perspective, we prefer to consider TDLA 30-10 ULA Low as staring point.

Proposal 4. Propose to consider TDLA 30-10 ULA Low propagation condition for 6MIMO layers.

Propagation channel for 4 MIMO layers

- Way forward - Option 1: TDLA30-10 - Option 2: TDLC300-100 - Option 3: TDLA30-10 and TDLC300-100

Regarding 4 MIMO layers conditions, firstly, RAN4 should decide the test case for 4 MIMO layers, which needs to introduce separate test cases for FWA and handled UE devices or one single test case for both FWA and handle UE devices. If RAN4 agrees to introduce one single test case for 4 MIMO layers, we prefer to consider TDLC300-100 condition. Otherwise, we prefer to consider TDLA30-10 for FWA devices and TDLC300-100 for handled UE devices.

Proposal 5. Propose to consider TDLA 300-100 propagation condition if RAN4 agrees to introduce only one single test case for 4 MIMO layers. Otherwise, we prefer to consider TDLA30-10 for FWA devices and TDLC300-100 for handled UE devices.

MIMO correlation for 4 MIMO layers

- Way forward - Option 1: ULA Medium RUE6Rx=1β125β425β925β1625ββ125∗1β125β425β925β1625β425∗β125∗1β125β425β925β925∗β425∗β125∗1β125β425β1625∗β925∗β425∗β125∗1β125β∗β425∗β925∗β425∗β125∗1 - Option 1a: with β = 0.00718 - Option 1b: with β = 0.005154 - Option 2: ULA Low

As we mentioned above, 4 MIMO layers related assumptions depend on the specific test case definition. For FWA devices, ULA Low shall be considered. And for handled UE, we prefer to consider ULA Medium at first, because of the size of the handled UE.

Proposal 6. Propose to consider ULA low for FWA device and ULA Medium for handled UE if define two test cases for 4 MIMO layers. Otherwise, only consider ULA Medium to define one test case for 4 MIMO layers.

DMRS configuration for 6 MIMO layers case

- Way forward - Option 1: Rel-15 Type 1 DMRS 2+2 - Option 2: Rel-15 Type 1 DMRS 2+0 - Option 2: Rel-18 eType1 DMRS 1+1

In the previous feasibility phase, RAN4 evaluated different DMRS configurations, which can bring different system throughput gain. And now, from the perspective of the testing, Rel-18 DMRS configuration is a optional feature, not all UEs could support this default configuration. In RAN4 legacy requirements, DMRS 2+2 is also used in Rel-18 8Rx  requirements definition. Hence, we prefer to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

Proposal 7. Propose to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

Antenna configuration for 4 MIMO layers case

- Way forward - Option 1: 4Tx6Rx - Option 2: 8Tx6Rx

Regarding antenna configuration, we prefer to consider 8Tx6Rx for 4 MIMO layers, since more antennas can bring diversity gain.

Proposal 8. Propose to consider 8T6R for 4MIMO layers.

MCS for 6 MIMO layers case

- Way forward - Option 1: MCS13 (Table 2) - Option 2: MCS 13 as starting point (Table 1) - Option 3: 256QAM - Option 4: 64QAM (MCS Table 1) with the same MCS for both CW - Option 5: MCS17 as starting point (Table 1)

In feasibility phase, RAN4 evaluated the performance gain comparison between 64QAM and 256QAM. As for how RAN4 determines MCS, we think it is necessary to at least determine it by evaluating the corresponding MCS. Hence, we propose to evaluate the SNR operating points for 64QAM and 256QAM in the initial stage.

Proposal 9. Propose to consider MCS 13 for 6 MIMO layers, and for 64QAM and 256QAM need to evaluate at first.

SDR requirements for 6Rx

Static channel and precoding for 6Rx

- Proposals - Option 1: Introduce new 6Rx static channel. The effective channel matrix, which is a combination of the introduced 6Rx static channel and precoding matrix, should maintain good orthogonality across multi-layers. The effective channel matrices should be full rank and leading no zero input at the receiving antenna. - Option 2: - For 1T6R (Random precoding): H=111111 - For 2T6R (Random precoding): H=111j−jj111−jj−j - For 4T6R (Random precoding): - H=111111111−1−1−122j−2j22j3010j305j−33010j−22j2j−22j−3010j305j−3010j - For 8T6R (Fixed precoding with i1,1=i1,2=i2=0): H=34∗11111−j1111−1j1−j1−11−1−1j1−11−111−1−11−j11−1−1−1j−1j1−1−111−j1−1−11

Regarding the static channel for 4T6R, we just need to ensure that each column is orthogonal. Since the number of Tx is less than that of Rx, the H channel matrix can ensure that every two columns are orthogonal to each other, and there can be at most 6 orthogonal basis. Without no doubt, it is easy to choose four of the six orthogonal basis vectors. However, for 8T6R, since there are at most 6 orthogonal bases, only 6 orthogonal vectors can be selected, but the remaining 2 Tx antennas cannot add new orthogonal columns (because the rank has reached the upper limit of 6), resulting in these two columns being non-orthogonal to the existing columns, resulting in inter-layer interference.

Due to this reason, directly selecting orthogonal vectors while ignoring the precoding matrix is not feasible. Therefore, RAN4 needs more further discuss how to select the precoding matrix at first or directly select from the static channel of 8T8R even though all layers cannot be guaranteed to be orthogonal for 8T6R.

Observation 1. For 4T6R, the static channel matrix is not restricted to the orthogonal space basis vector, on the contrary, 8T6R is subject to constraints in this regard.

Proposal 10. RAN4 needs more discussion on how to find the static channel matrix for 8T6R, based on fixed precoding matrix or directly select from the static channel of 8T8R even though all layers cannot be guaranteed to be orthogonal for 8T6R.

CSI requirements for 6Rx

CQI test for 6Rx

- Way forward - Option 1: Not define CQI test - Option 2: Define CQI test - Option 2a: Only define CQI requirements with rank 4 under static channel - Option 2b: Define the following CQI test - Periodic CQI reporting requirements under AWGN - Wideband CQI reporting requirements with inter-cell interference under fading conditions

It is clearly mentioned in WID[1] that RAN4 needs to specify CSI requirements to support 6Rx, from our point of view, it is better to define CSI requirements in this item to follow the RAN plenary decision.

Proposal 11. Propose to define CQI requirements for 6Rx.

RI test

- Way forward - Option 1: Not define RI test for 6 layers - Option 2: Define RI test for 6 layers

Regarding RI test, which is used to verify that the reported rank indicator accurately represents the channel rank. For FWA and handled UE 6Rx, we did not find it necessary to define requirements, since we also did not define requirements for 8Rx.

Proposal 12. Propose to do not define RI requirements for 6Rx.

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx, The conclusions are:

Observation 1. For 4T6R, the static channel matrix is not restricted to the orthogonal space basis vector, on the contrary, 8T6R is subject to constraints in this regard.

Proposal 1. Propose to postpone discussing test applicable rule until RAN4 decides on the 6Rx test cases.

Proposal 2. Propose to not consider test scenarios with interference in Rel-19.

Proposal 3. Propose to define a single test case for 4MIMO layers that is applicable for both FWA and handled UE.

Proposal 4. Propose to consider TDLA 30-10 ULA Low propagation condition for 6MIMO layers.

Proposal 5. Propose to consider TDLA 300-100 propagation condition if RAN4 agrees to introduce only one single test case for 4 MIMO layers. Otherwise, we prefer to consider TDLA30-10 for FWA devices and TDLC300-100 for handled UE devices.

Proposal 6. Propose to consider ULA low for FWA device and ULA Medium for handled UE if define two test cases for 4 MIMO layers. Otherwise, only consider ULA Medium to define one test case for 4 MIMO layers.

Proposal 7. Propose to consider Rel-15 Type 1 DMRS 2+2 as default configuration.

Proposal 8. Propose to consider 8T6R for 4MIMO layers.

Proposal 9. Propose to consider MCS 13 for 6 MIMO layers, and for 64QAM and 256QAM need to evaluate at first.

Proposal 10. RAN4 needs more discussion on how to find the static channel matrix for 8T6R, based on fixed precoding matrix or directly select from the static channel of 8T8R even though all layers cannot be guaranteed to be orthogonal for 8T6R.

Proposal 11. Propose to define CQI requirements for 6Rx.

Proposal 12. Propose to do not define RI requirements for 6Rx.

# 4 References

- RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4. Huawei, HiSilicon.

- R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon.